| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C269_r4 WG11 Number: m18312 |
| --- | --- |

| Title: | A study on the impact of intra smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
|  | Ehsan Maani 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | ehsan.maani@am.sony.com |
|  | Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution presents the effect of disabling intra smoothing to the suggested HEVC Test Model in JCTVC-C122. Simulations showed that when the default intra smoothing was disabled, the average BD-Bitrate was reduced by about 1.4 % for Intra, and 1.9 % for Intra LoCo, with respect to the results in JCTVC-C122 with PIPE. Cross verification of the results were done with NEC and Mitsubishi.

# Introduction

A starting point for HEVC Test Model was presented in JCVC-C122. The purpose of this contribution is to study the effect of disabling the default intra smoothing in addition to the suggested Test Model in JCTVC-C122 for Intra and Low Complexity Intra.

The experiments were conducted with TMuC version 0.7.4. In addition to the configuration presented in JCTVC-C122, the DEFAULT_IS is disabled as in the yellow conditions in the following configuration:

Table 1: Configurations for High-Efficiency Condition from JCTVC-C122 with DEFAULT_IS disabled.

| Tool name | TMuC default setting (JCTVC-B300) | Proposed setting | Parameters for proposed setting |
| --- | --- | --- | --- |
| Coding unit | Min size: 8x8 Max size: 64x64 | Same as B300 | Same as B300 |
| Transform unit | Quadtree | Samsung based | QuadTreeTUFlag : 0 |
| Large transform | Max size: 64 | Max size: 32 | QuadTreeTULog2MaxSize : 5 MaxTrSize : 32 |
| Angular Intra Prediction | I64: 5 modes I32: 34 modes I16: 34 modes I8: 34 modes I4: 17 modes I2: 3 modes | I64: 5 modes I32: 17 modes I16: 17 modes I8: 17 modes I4: 17 modes I2: 3 modes | #if UNIFIED_DIRECTIONAL_INTRA const UChar g_aucIntraModeNumAng[7] = { 3, // 2x2 17, // 4x4 17, // 8x8 17, // 16x16 17, // 32x32 5, // 64x64 5 // 128x128 }; const UChar g_aucIntraModeBitsAng[7] = { 2, // 2x2 5, // 4x4 5, // 8x8 5, // 16x16 5, // 32x32 3, // 64x64 3 // 128x128 }; |
| Rotational transform (ROT) | Enabled | Disabled | ROT : 0 |
| Mode depend. directional transform (MDDT) | Enabled | Disabled | #define QC_MDDT 0 |
| Adaptive Intra Smoothing (AIS) on/off | Enabled | Disabled | AIS : 0 |
| Combined Intra Prediction (CIP) | Enabled | Disabled | CIP : 0 |
| Default Intra Smoothing on/off | Enabled | Disabled | #define DEFAULT_IS 0 |
| Planar prediction | Enabled | Disabled | #define PLANAR_INTRA 0 |
| Edge-based prediction | Enabled | Disabled | EdgePredictionEnable : 0 |
| Deblocking filter | Enabled | Same as B300 | Same as B300 |
| Adaptive loop filter (ALF) | Diamond | Same as B300 | Same as B300 |
| Entropy coder | PIPE | Same as B300 | Same as B300 |
| Transform coefficient coding | HHI | Same as B300 | Same as B300 |
| Internal Bit-Depth Increase (IBDI) | 4 bits | Same as B300 | Same as B300 |
| Rate Distortion Optimized Quantization (RDOQ) | On | Same as B300 | Same as B300 |
| CABAC-based RDO | On | Same as B300 | Same as B300 |
| Asymmetric Motion Prediction Unit (AMP) | Enabled | Disabled | AMP: 0 |
| Block Merging (MRG) | Enabled | Disabled | MRG: 0 |
| Motion vector prediction (AMVP/IMVP) | AMVP method | Same as B300 | Same as B300 |
| Interpolation filter | SIFO | DCT-IF 12 tap | InterpFilterType: 0 DIFTap: 12 |
| Adaptive motion vector resolution (AMVRES) | Enabled | Disabled | AMVRES : 0 |
| Transform Skip Flag | Disabled | Same as B300 | Same as B300 |
| PU-Based Merging | Disabled | Same as B300 | Same as B300 |
| Partition-based intensity compensation | Disabled | Same as B300 | Same as B300 |
| Transform precision extension | Enabled | Disabled | undefine TRANS_PRECISION_EXT |

Table 2. Configurations for Low-Complexity Condition from JCTVC-C122

(only the difference from High Efficiency Settings are listed)

| Tool name | TMuC default setting (JCTVC-B300) | Proposed setting | Parameters for proposed setting |
| --- | --- | --- | --- |
| Adaptive loop filter (ALF) | Disabled | Same as B300 | Same as B300 |
| Entropy coder | LCEC | Same as B300 | Same as B300 |
| Internal Bit-Depth Increase (IBDI) | Off | Same as B300 | Same as B300 |
| CABAC-based RDO | Off | Same as B300 | Same as B300 |
| Interpolation filter | DIF | DCT-IF 6 tap | InterpFilterType: 0 DIFTap: 6 |
| Adaptive motion vector resolution (AMVRES) | Disabled | Same as B300 | Same as B300 |
| LCEC Phase 2 | Disabled | Enabled | #define LCEC_PHASE2 (1) |
| Transform precision extension | Enabled | Same as B300 | Same as B300 |
| Shift bits for transform precision extension | 8x8: 7 bit 16x16: 6 bit 32x32: 5 bit 64x64: 4 bit | 8x8: 4 bit 16x16: 4 bit 32x32: 4 bit 64x64: 4 bit | const int g_iShift8x8 = 4; const int g_iShift16x16 = 4; const int g_iShift32x32 = 4; const int g_iShift64x64 = 4; |

The experiments were conducted with TMuC version 0.7.4 on a computer with Xeon X5680 @3.33Ghz, 2 processors, 12 cores, 48GB Memory. The Compiler was Microsoft VC1600 for x64 architecture.

# BD Summary

The anchor results were from JCTVC-C122. The results of disabling intra smoothing with the condition from JCTVC-C122 were compared with the anchor. Since timing comparison was not conducted, the timing information of the anchor is removed from the Excel spread sheet to avoid miss-interpretation.

Table 2: Experimental results with PIPE and DEFAULT_IS 0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 7.8 | 8.3 | 6.3 | 6.1 | 3.3 | 1.6 |
| Class B | 6.3 | 5.0 | 4.1 | 5.2 | 3.8 | 3.1 |
| Class C | 5.7 | 3.8 | 3.2 | 4.2 | 2.1 | 2.1 |
| Class D | 4.5 | 2.7 | 2.5 | 3.6 | 1.3 | 1.3 |
| Class E | 8.6 | 5.1 | 5.4 | 5.9 | -1.6 | -0.6 |
| All | 6.3 | 4.6 | 4.0 | 4.9 | 1.9 | 1.7 |

The corresponding results from JCTVC-C122 are repeated here for comparison:

Table 3: Experimental results from JCTVC-C122 with PIPE

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 7.4 | 7.6 | 5.2 | 4.7 | 3.0 | 1.2 |
| Class B | 6.7 | 5.2 | 4.2 | 6.3 | 5.6 | 4.7 |
| Class C | 8.0 | 5.9 | 5.1 | 7.5 | 5.3 | 5.1 |
| Class D | 6.1 | 4.1 | 3.9 | 5.6 | 3.5 | 3.5 |
| Class E | 11.3 | 6.6 | 7.1 | 9.6 | 2.3 | 3.6 |
| All | 7.7 | 5.6 | 5.0 | 6.8 | 4.2 | 3.9 |

# Visual results

Visual comparison was not performed.

# Conclusion

When DEFULT_IS is disabled, the average %BD-Rate were reduced by about 1.4 % for Intra, and 1.9 % for Intra LoCo, from the results in JCTVC-C122 with PIPE.