| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C274 WG11 Number: m18317 |
| --- | --- |

| Title: | TE10, Subset 1: Cross-Check Results of NEC’s Joint Deblocking-debanding Filter by Microsoft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhiwei Xiong Jizheng Xu | Email: {v-zxiong, jzxu}@microsoft.com |  |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This report presents the cross-check results of NEC’s joint deblocking-debanding filter by Microsoft.

- Software study

In NEC’s software, small pseudo noise is added to strong filtered images of intra-coded CU boundaries associated with transform sizes larger than or equal to 16x16. The small pseudo noise, whose element takes the value of -1, 0, or 1, is loaded from LUT according to its CU index, strong filter direction, and row and column indices.

There is no problem observed in compiling and running the software.

# Experimental results

The following platform and compiler has been used:

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.2 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.4 | 0.0 | 0.0 |
| All | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | -0.3 | 0.1 |
| Class E |  |  |  |
| All | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | -0.2 | 0.1 |
| Class E | 0.2 | 0.2 | 0.1 |
| All | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Detail results can be found in the accompanying file JCTVC-C274.xls.