| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C275 |
| --- | --- |
|  | WG11 Number: m18318 |

| Title: | Improved intra frame coding by PU/TU reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiulian Peng Jizheng Xu Feng Wu | Email: | {jzxu, fengwu}@microsoft.com |
| Source: | Microsoft University of Science and Technology of China |  |  |

_____________________________

# Abstract

This document presents an adaptive PU/TU reordering scheme for more accurate prediction in TMuC intra-frame coding. Within a PU, the coding order of TUs is rearranged to achieve better prediction for th inner TUs. Within a CU, a better coding order of PU can be selected also for a better prediction.

# Introduction

In TMuC, the intra prediction for a block still utilizes the traditional technique in H.264 by extrapolation of neighboring reconstructed block boundaries in the upper and left directions. However, for directional modes that lean to upper-right or bottom-left directions (10~17, 26~33 in Fig. 1), the predictions tend to be poorer than that for modes leaning to the upper-left direction (0~9, 18~25 in Fig. 1) for longer prediction distances. To better exploit spatial correlations, an adaptive PU/TU reordering scheme is proposed. It employs PU and TU level reordering to find adaptive reference pixels for different content.

# Algorithm description

## TU reordering

In TMuC0.7 intra-frame coding, for each PU there is a residue transform quad-tree (RQT) with z-order scanning, that is 1->2->3->4 in Fig.2 (a) for all directional modes. The upper and left reference pixels are used for prediction of each TU block (see Fig.3 (a)).

In this scheme, the scanning order of RQT is direction adaptive. For modes 10~17 with a positive angular value (see Fig. 1), we employ the right-first ordering as illustrated in Fig.2 (b). For modes 26~33 (see Fig. 1), we employ the bottom-first ordering as illustrated in Fig.2 (c). And for modes DC, 1~9 and 18~25, we still utilize the traditional z ordering. Along with different scanning orders, different reference pixels are used in block prediction, which are illustrated in Fig.3 (a) ~(c). It can be seen that through the direction-adaptive ordering, each directional mode can find its own reference pixels with the nearest prediction distance. The TU-level reordering is directly substituted for the original z-ordering in RQT without any overhead.

Figure 1. Directional modes in unified intra prediction.

(a)                                       (b)                                          (c)

Figure 2. Illustration of three scanning orders of transform QT: (a) z-order; (b) right-first-order; (c) bottom-first-order.

(a)                                     (b)                                             (c)

Figure 3. Illustration of reference pixels for three scanning orders in transform QT: (a) z-order; (b) right-first-order; (c) bottom-first-order.

## PU reordering

As we know, PU is the prediction unit in TMuC. For each CU, the partition mode can be SIZE_2Nx2N or SIZE_NxN in intra coding. When the partition is SIZE_NxN, the block ordering is still z-ordering. In spirit of TU reordering, we employ adaptive ordering in PU level too, since sometimes the directions of four PU blocks do not change much.

There are three scanning orders for each CU: z-order, right-first-order and bottom-first-order. Under z order, the previous direction-adaptive TU reordering is applied and the PU ordering does not change. While under the other two orderings, both the PU and TU orderings are changed, either right-first-order or bottom-first-order. The reference pixels are changed accordingly. Besides, the 33 directional modes are the same as before, except that those modes using the left reference pixels are reversed to use the right or bottom reference pixels instead, in the right-first and bottom-first orderings respectively. Since different orderings have different reference pixels, by PU reordering not only the prediction under directions 10~17and 26~33 but also that under modes 1~9 and 18~25 with a negative angular value can be improved.

Unlike the TU-level reordering, the three orders are chosen by rate-distortion optimization and need to be transmitted to the decoder.

# Compression performance discussion

Table I.  Comparison of coding efficiency for CfP sequences

| Seq | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- |
| Traffic | -0.8 | -0.6 | -0.6 |
| PeopleOnStreet | -0.6 | -1.8 | -1.9 |
| Kimono | -0.2 | -0.1 | 0.0 |
| Cactus | 0.0 | 0.1 | 0.1 |
| BasketballDrive | -0.1 | 0.1 | 0.1 |
| BQTerrace | -1.4 | -2.8 | -3.7 |
| BasketballDrill | -0.8 | -1.1 | -0.5 |
| BQMall | 0.0 | -0.2 | -0.4 |
| PartyScene | 0.1 | -0.1 | 0.0 |
| RaceHorses | -0.6 | -0.7 | -0.6 |
| BasketballPass | 0.0 | -0.2 | 0.0 |
| BQSquare | -0.4 | -0.7 | -0.6 |
| BlowingBubbles | -0.1 | 0.0 | 0.0 |
| RaceHorses | -0.4 | -0.3 | -0.4 |
| Vidyo1 | -0.6 | -1.7 | -1.5 |
| Vidyo3 | 0.0 | -0.7 | -0.1 |
| Vidyo4 | -0.3 | -1.2 | -1.4 |
| Average | -0.4 | -0.7 | -0.7 |

The proposed scheme has been integrated into the TMuC0.7 software. The TU-level reordering is applied to both the luminance and chrominance components and the PU-level reordering is only on the luminance component. The all intra high efficiency configuration in [1] is applied. Anchor is TMuC0.7.

The experimental results for all test sequences are shown in Table I.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

# Patent rights declaration(s)

Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).