| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C276 WG11 Number: m18319 |
| --- | --- |

| Title: | Screen content coding results using TMuC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cuiling Lan Jizheng Xu Feng Wu | Email: {v-culan, jzxu, fengwu@microsoft.com} |  |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

New coding tools have been presented for coding screen contents using AVC framework in JCTVC-B084. This document presents new coding results using TMuC, for intra coding and for random-access cases.

# Introduction

In JVTVC-B084[1], we have showed that by integrating two new modes into AVC, the coding performance for screen content can be much improved. But in that document, we only show results for images. In this document, we show that in TMuC framework, the two modes can also improve the coding performance for screen contents. Additionally, new coding results for video sequences are also shown.

# New coding tools for screen image coding

In this section, we revisit two new coding tools for screen image coding, one is Residual Scalar Quantization (RSQ) and the other is Base Colors and Index Map (BCIM). They are implemented based on the block (or coding unit) structure. Thus, it is easy to integrate the new tools into the current coding scheme by adding two additional intra coding modes. In the following, we will briefly introduce these two modes. More details can be found in [4].

## Residual scalar quantization (RSQ)

In the RSQ mode, directional prediction is also used for each block to generate the prediction residue. After the intra prediction, different from the current intra coding modes, no transform is applied on the residue. Instead, the quantization is directly applied on the residual signals, similar to APEC [5] used in KTA inter block coding. For text/graphics contents, usually the signal in spatial domain is sparser than that in transform domain. It will decrease the coding efficiency if transform is still performed on such contents. We also use the same direction setting when performing intra prediction. But different from the current intra prediction scheme, the residual quantization and sample reconstruction are done pixel by pixel within a block in our scheme. The reconstructed pixel from the above line or the left column can be used for the prediction of the next pixel along the same direction. The use of intra block reconstructed pixels for prediction is based on the assumption that the shorter the prediction distance is, the stronger the correlation will be.

## Base colors and index map (BCIM)

Having limited colors but complicated shapes is another property of the text and graphics parts on screen images. Such text/graphics blocks can be expressed concisely by several base colors together with an index map. It is somewhat like color quantization that is a process of choosing a representative set of colors to approximate all the colors of an image [6]. In the BCIM mode, we ﬁrst get the base colors of a block by using a clustering algorithm. All the base colors constitute a base color table. Then, each sample in the block will be quantized to its nearest base color. The index map indicates which base color is used by each sample. Different from color quantization, each text/graphics block, but not an entire image, has its own base colors and an index map for representation in our scheme. Thus, it is content adaptive for each block. In addition, since the base color number of a block is small, fewer bits are required to represent each mapped index.

In this mode, there are several key problems to solve. First, base colors should be chosen to well represent a block. Second, after base colors are selected, there should be an efficient entropy coding method to code the base colors and index map. If the coding method has been decided and is good enough, the problem is then simpliﬁed as the searching of the best base colors for that block. In this mode, we use dynamic programming to not only achieve the global optimal partition but also avoid intensive computation caused by clustering of multiple times and the iterative updating. The best base color number is decided in the rate-distortion sence.To represent a block, we need to encode the base colors and index map into the bitstream. Instead of using the binarization schemes in AVC, such as Exp-Golomb, the base color number, values and remapped indexes are all simply expressed by their binary format. The context setting for the index map coding is shown in Fig. 1. We take the number of base colors to be 4 as an example. In such a case, there will be 15 basic patterns of the neighboring indexes, as {AAAA, AAAB, AABA, ABAA, BAAA, AABB, ABAB, ABBA, AABC, BAAC, BCAA, ABAC, ABCA, BACA, ABCD}, where A, B, C and D stand for indexes that have been sorted by their occurrences. Thus, there will be 15 contexts for coding the current index. We also remap the current index to 0, 1, 2 or 3 when the current index equals to A, B, C or D before the entropy encoding.

Fig. 1. The neighboring indexes as contexts of the current index.

# Experimental results

We have integrated the RSQ and BCIM modes into TMuC7.0 software and use several screen images and videos to test its performance. The structure in TMuC which allowing flexible large block sizes enables an easy design to extend the proposed modes to flexible block sizes.

The RSQ modes are performed on the TU unit, with size ranging from 64x64 to 4x4. A flag is utilized to identify whether the mode is the original mode in TMuC or the RSQ mode. This mode is only checked on the prediction manners where the intra smoothing is not performed.

The BCIM modes works in two forms: one dimensional and three dimensional cases. For the one dimensional case, the BCIM mode is performed on the PU unit when the partition manner is SIZE_NxN, with block sizes ranging from 32x32 to 4x4. It reuses the DC prediction flag and a flag is utilized to distinguish them. For the three dimensional cases, the clustering is performed on the three dimensional block where only one index map is required for all the three color components. It works on the CU unit, with block sizes ranging from 64x64 to 8x8.

The images of size 1280x0124 and the coding results are shown in Fig. 3, Fig. 4 and Fig. 5. The curve marked by H.264 is the coding results of H.264. The curve marked by H.264 + Proposed is the coding performance after integrating the proposed two tools into H.264. Curves marked by TMuC show the coding results using TMuC software and the curves marked by TMuC + Proposed are the screen coding results after integrating the proposed two tools into TMuC. As we can see, the coding gain can be over 10dB by utilizing the two new coding tools. Since TMuC software has a much higher performance than JM, more gain is achieved after integrating the two tools. We use the intra frame configuration. Here, the loop filter is disabled for all schemes to facilitate the comparison. The bit depth is set as 8. Qps are set from 47 to 7 with step of 5.

Actually, the loopfilter in TMuC or JM both will decrease the coding performance for the screen images, especially for some Qps. Thus, in the proposed scheme, we skip the loopfilter for the blocks when the decided mode is one of the proposed modes or when one of its neighboring modes chooses the proposed mode.

| We also give evaluations of the coding performance for screen content video compression. Several screen captured videos at the frame rate of 10 are utilized for testing. The size of Photo and Windows is 1024x768 and the size of WebBrowse is 1280x1024. The length of all the sequences is 100 frames. The configurations of allowing random access and all I frames are utilized for tests separately. The bit depth is set to be 8. Qps are set as 15,25,35 and 45. The loopfilter is enabled. For the random access configuration, the intra period is set to be 16. The rate distortion curves are shown in Fig. 6, Fig.7 and Fig. 8. (a) show the rate-distoriton curves at the random access configuration and (b) show the rate-distortion curves at the all I frames configuration. We can see that becasuse of the impletation of two simple tools, signifiant gains are obtaiend for the screen content video compression. Table I shows the BD-RATE and BD-PSNR gains. |  |
| --- | --- |

(a )				     	                 (b)

Fig. 3. (a) screen image Mix, and (b) coding performance comparison.

(a)							(b)

Fig. 4. (a) screen image Webpage, and (b) coding performance comparison.

(a)							(b)

Fig. 5. (a) screen image Slide, and (b) coding performance comparison.

(a)						     (b)

Fig. 6. Screen video Photo. (a) Random access configureation;  (b) All I frames configuration.

(a)						    (b)

Fig. 7. Screen video Windows. (a) Random access configureation;  (b) All I frames configuration.

(a)						 (b)

Fig. 8. Screen video WebBrowse. (a) Random access configureation;  (b) All I frames configuration.

Table I

| Sequence | BD-bitrate | BD-psnr |
| --- | --- | --- |
| Photo | -21.82 | 2.21 |
| Windows | -11.23 | 0.72 |
| WebBrowse | -27.44 | 3.06 |
| Average | -20.16 | 2.00 |

- Random_access

| Sequence | BD-bitrate | BD-psnr |
| --- | --- | --- |
| Photo | -31.36 | 3.23 |
| Windows | -16.9 | 1.25 |
| WebBrowse | -35.87 | 2.99 |
| Average | -28.04 | 2.49 |

- All_intra

# Conclusions

In this document, we have shown some results of applying two simple new tools to TMuC software, and the coding performance for screen contents can be improved much. Since TMuC software has a much higher performance than JM, more gain is achieve by combining the new screen coding tools and various tools in TMuC together. Besides, the new tools are applied to larger size blocks, being more adaptive to image contents.

There are still many issues in screen coding to be improved. For example, the motion properties of screen videos are also very different from those of natural videos. Thus, the sooner can JCT-VC consider screen coding and start some efforts, the more likely the new standard can be more mature for the new screen contents and facilitate various new application scenarios.

# References

[1] C. Lan, J. Xu, F. Wu and G. Sullivan, “Screen content coding,”  JCT-VC Doc JCT-B084, Geneva, CH, July, 2010.

[2] “Draft Requirements for next-generation video coding project,” ITU-T SG16/Q6/VCEG-AL96, London, UK/Geneva, CH, July 2009.

[3] “Vision, Applications and Requirements for High-Performance Video Coding (HVC),” ISO/IEC JTC1/SC29/WG11/N11096, Kyoto, JP, Jan. 2010.

[4] C. Lan, G. Shi and F. Wu, “Compress compound images in H.264/MPEG-4 AVC by exploiting spatial correlation,” IEEE Trans. on Image Processing, Apr., 2010.

[5] M. Narroschke, H. G. Musmann, “Adaptive prediction error coding in spatial and frequency domain with a fixed scan in the spatial domain,” ITU-T SG16/Q6/VCEG-AB06, Bangkok, TH, Jan. 2006

[6] X. Wu, “Color quantization by dynamic programming and principal analysis,” ACM Trans. Graph., vol. 11, no. 4, pp. 348–372, Oct. 1992.

# Patent rights declaration(s)

Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).