| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C279 WG11 Number: m18322 |
| --- | --- |

| Title: | Opportunistic parallel V2V decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dake He, Gergely Korodi, En-hui Yang, Gaelle Martin-Cocher 175 Columbia St. West Waterloo, Ontario N2V 2V9 Canada | Tel: Email: | +1-519-888-7465 {dhe, gkorodi, ehyang, gmartincocher}@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

A buffer-based technique that distributes codewords produced by V2V codes into interleaved fixed-length phrases is proposed. On the decoder side, since subsequent phrases can be accessed independently without waiting for the decoding of the current phrase to complete, opportunistic parallel decoding can be achieved.

- Introduction

V2V (variable-length-to-variable-length) coding for HEVC (high efficiency video coding) was proposed in JCTVC-A116 and JCTVC-A120, where a finite number of V2V codes are used for entropy coding, each of which corresponds to a unique probability between 0.02 and 0.5. Subsequently, in TMuC [1] a set of 12 V2V codes are described.

In a particular operational mode (hereafter “interleaving mode”), in response to a sequence

(x1, p1) (x2, p2), …, (xi, pi), …, (xn, pn)

where xi denotes the bin symbol to be encoded, and pi denotes the probability estimate as the result of context modeling used to encode xi, V2V coding for HEVC produce an interleaved sequence of variable-length codewords

c1c2, …, cj, …, cm.

In order to make sure decoding is successful, the codeword sequence satisfies the causality constraint as follows. Let xk be the first bin symbol corresponding to cj, then x1…xk-1 must be decoded before cj is to be decoded.

Due to the nature of V2V coding, {cj; j=1, 2, …, m} are of various lengths. As such, the starting position of cj+1 is often not determined until cj is fully decoded for any 0 < j < m. Consequently it is hard if not impossible to take advantage of pipeline architecture to speed up decoding.

In this section, we describe a buffer-based technique that, coupled with V2V codes, in response to

(x1, p1) (x2, p2), …, (xi, pi), …, (xn, pn)

produces an interleaved sequence of phrases

A1A2,…, Aj, …, Am’,

where |Aj| is a constant. Throughout this document, |s| denotes the length of a binary string s. The sequence A1A2,…, Am’ also satisfies the causality constraint to ensure successful decoding. Furthermore, since |Aj| is a constant, the decoder is now possible to read Aj and Aj+1 simultaneously, and start decoding Aj+1 as soon as it becomes possible. In cases where more than one bin decoders are available, opportunistic parallel decoding can then be achieved.

Note that in contrast to the well known variable-length-fixed-length codes like Tunstall codes, Aj above is not a fixed-length codeword itself but rather a fixed-length container, which might contain even incomplete codewords.

# Buffer-Based Encoding

In this section, we describe how the proposed technique produces interleaved fixed-length phrases.

Let (x1, p1) (x2, p2), …, (xi, pi), …, (xn, pn) denote the sequence of bin symbol and probability estimate pairs. For our purpose, pi belongs to a set, denoted by P, of K probabilities. Suppose that K V2V codes are used, each of which is designed for a probability in P. To produce a sequence of interleaved phrases of a constant length, the proposed technique works as follows.

[Parsing Stage]

Step 1: Set i=1, and initialize K empty FIFO (first-in-first-out) bin buffers, labeled 1, …, K, each of which corresponds to a V2V code, i.e., bin buffer k corresponds to the V2V code indexed by k.

Step 2: Place xi into the bin buffer k if pi is the kth member in P.

Step 3: Increase i by 1.

Step 4: Repeat Steps 1-3 until xn is processed.

[Coding Stage]

Step 1: Set j=1;

Step 2: With each buffer k, associate an FIFO codeword buffer called Fk and an FIFO index buffer Ik, and an indicator: all two buffers are initialized empty, and the indicator is set to 0. For bin buffer k, the length of the associated codeword buffer is at least twice of the longest codeword length of the corresponding V2V code.

Step 3: For k= 1, …, K, do the following.

Step 3a: Use the V2V code for bin buffer k to encode the remaining bin symbols in the buffer. Encoded bins are shift out of the bin buffer immediately.

Step 3b: If a complete codeword is reached, send the codeword to Fk and record the index of the first bin symbol mapped to the codeword in Ik.

Step 3c: If Fk contains more than L bits, where L is a pre-determined number known both to the encoder and the decoder, set the indicator to 1.

Step 3d: If the indicator is equal to 1, and the first entry of Ik is the smallest among all {I1,…, IK}, do the following.

Step 3d.1 Shift out the first L bits of Fk as Aj;

Step 3d.2 Shift out t entries from Ik, where t denotes the number of codewords terminating in Aj.

Step 3d.3 Set the indicator to 0.

Step 3d.4 Increase j by 1.

Step 4: Repeat Steps 1-3 until xn is processed.

Step 5: After xn is processed, for k=1, …, K, flush out the remaining incomplete codewords in the codeword buffers by appending the MPS (most probable symbol) symbols to the bin buffers, and if necessary append default bits (e.g. bit 0) so that the indicator is set to 1 in Step 3c.  The order of output is determined again by the index buffers.

# Opportunistic Parallel Decoding

In this section, we describe how to achieve opportunistic decoding given an interleaved sequence of fixed-length phrases

A1A2……Aj……Am.

Note that since |Aj| is a constant, the decoder can access Aj+1 and any other phrases in the future without having to decode Aj. Suppose that Aj+1 is to be decoded by using a different V2V code from that used to decode Aj. Then as soon as the decoder determines the correct V2V code for Aj+1, it can immediately start decoding Aj+1 without waiting for the decoding of Aj to complete. This indicates a natural pipeline implementation of entropy decoding as follows.

[Read Aj     ] [Decode Aj]

[Read Aj+1]	     [Decode Aj+1]

[Read Aj+2]		[Decode Aj+2]

[Read Aj+3]	       	  [Decode Aj+3]

…

Note that the parallelism achieved above is opportunistic because it depends on the content.

# References

[1] Test model under consideration. JCTVC-A205, Dresden, Germany, 2010.

# Patent rights declaration(s)

Research In Motion Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).