| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C280 |
| --- | --- |

| Title: | TE8: Crosscheck Result of the Transcoder for JCTVC-B034 Source Selection for V2V Entropy Coding in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Contact: Shawmin Lei +886-3-567-0766 ext.25555 shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the transcoder implementation of JCTVC-B034  on source selection for V2V entropy coding submitted by Research In Motion (RIM). The verification task has been done successfully and the results match those provided by RIM.

# Verification

RIM provided a transcoder implemented in the TMuC for the V2V code described in JCTVC-B034. The verification is carried out by using the following instructions:

TAppDecoder –b anchor.bin –o dec.yuv –t anchor-transcode.bin

TAppDecoder –b anchor-transcode.bin –o rec-transcode.yuv

The intention of the first instruction is to decode and transcode the anchor bitstream. A transcoded bitstream anchor-transcode.bin will be generated after the first instruction. The second instruction decodes the transcoded bitstream to rec-transcode.yuv. The verification is done by comparing YUV files decoded from transcoded bitstreams and the YUV files decoded from anchor bitstreams.

Three configurations (intra HE, random access HE, low delay HE) were run for the transcoding technique with the provided code. The running of the tests includes transcoding, decoding, and collecting the file sizes from the decoder output.

# Results

The YUV files decoded from transcoded bitstreams match the ones decoded from anchor bitstreams exactly. Our transcoded results are the same as RIM’s results in intra and low delay conditions. However, there are some small variations (up to -0.12%) in random access condition. The sizes of the anchor bitstreams from RIM are different from ours, but our anchor bitstream sizes are the same as those in TE12 [2]. The main cause might be the simulation platform. We used GCC 3.4.6-9(x86_64) and Intel CPUs to run TMuC 0.7.0 on Redhat Linux (x86_64) for the tests. RIM used 32-bit Windows XP PCs. Maybe there are floating point issues for different compilers and platforms, so some of the results are slightly different. However, the overall performance can be still confirmed. The results are listed in JCTVC-C280_r1.xls.

# Conclusion

The results of the transcoder which implements the V2V code in JCTVC-B034 are confirmed.

# References

- Gergely Korodi and Da-ke He, “Dynamic Source Selection,” Document of Joint Collaborative Team on Video Coding, JCTVC-B034, Geneva, Switzerland, Jul. 2010.

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Document of Joint Collaborative Team on Video Coding, JCTVC-B312, Geneva, Switzerland, Jul. 2010.