| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C281 WG11 Number: m18324 |
| --- | --- |

| Title: | TE 12: Evaluation of Fast Adaptive Intra Smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Muhammed Coban Yunfei Zheng Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5673 martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents the test results for the fast adaptive intra smoothing listed in Tool Experiment 12: Evaluation of TMuC Tools.

- Introduction

In the 2nd JCT-VC meeting, Tool Experiment 12 (TE12) [1] was set up to explore the performance of the individual tools in the Test Model under Consideration (TMuC). This contribution is to test the fast adaptive intra smoothing (AIS) of TE 12.

The software version used for this test was TMuC 0.7. The reference was generated by using the default setting [2]. In the default setting, fast AIS is disabled. The alternative setting with fast AIS enabled (#define AIS_TEST_BEST=1) is compared against the reference setting. The executables for encoding and decoding are compiled on 64-bit Linux and the simulations are performed on a Linux cluster.

- Results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.4 | 0.4 | 0.5 | 0.4 | 0.4 |
| Class B | 0.7 | 0.6 | 0.7 | 0.7 | 0.6 | 0.6 |
| Class C | 1.7 | 1.5 | 1.5 | 1.1 | 0.9 | 0.9 |
| Class D | 1.4 | 1.1 | 1.2 | 1.3 | 1.0 | 1.0 |
| Class E | 1.5 | 1.3 | 1.4 | 0.9 | 0.8 | 0.8 |
| All | 1.2 | 1.0 | 1.1 | 0.9 | 0.8 | 0.8 |
| Enc Time[%] | 64% |  |  | 63% |  |  |
| Dec Time[%] | 127% |  |  | 125% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.1 | 0.3 | 0.1 | 0.0 |
| Class B | 0.3 | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class C | 0.6 | 0.5 | 0.5 | 0.5 | 0.4 | 0.3 |
| Class D | 0.6 | 0.7 | 0.5 | 0.4 | 0.4 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.4 | 0.3 | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 87% |  |  | 89% |  |  |
| Dec Time[%] | 114% |  |  | 102% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class C | 0.2 | 0.1 | 0.4 | 0.1 | 0.1 | 0.2 |
| Class D | 0.1 | 0.2 | 0.5 | 0.1 | 0.2 | 0.0 |
| Class E | 0.3 | 0.3 | 0.6 | 0.2 | -0.3 | 0.8 |
| All | 0.2 | 0.2 | 0.4 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 85% |  |  | 92% |  |  |
| Dec Time[%] | 107% |  |  | 124% |  |  |

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).