| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C284 WG11 Number: m18327 |
| --- | --- |

| Title: | TE9-2.1 Report on Force RQT split according to PUs (Sharp, Microsoft) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA, U.S.A. Jizheng Xu No. 49, Zhichun Road, Haidian District Beijing 100190, China | Tel: Email: Tel: Email: | +1-360-817-8487 asegall@sharplabs.com +86-10-59173019 jzxu@microsoft.com |
| Source: | Sharp and Microsoft |  |  |

_____________________________

# Abstract

This document reports the cross verification results of forcing split of TU quad-tree at different kinds of PU boundaries.

- Introduction

In the current TMuC, a CU can be partitioned into two rectangular PUs, four NxN PUs or two asymmetric PUs. In either partitioning, the transform quad-tree can be across the PU boundaries. In TMuC0.7.4 software, several Macros are added to disallow the transform to be across rectangular PUs, NxN PUs and asymmetric PUs. This document reports the coding results when those restrictions are made.

The tests are based on TMuC0.7.4 software. The anchor to be compared is to set (#define HHI_RQT_ROOT 1) in the software. Three cases are tested, listed in table 1.

Table 1. Three cases to be tested.

| Case 1 | Case 2 | Case 3 |
| --- | --- | --- |
| Force split rectangular: Default setting: Disabled Alternative setting: Enabled Parameters for Alternative setting: #define HHI_RQT_ROOT 1 #define HHI_RQT_FORCE_SPLIT_RECT 1 | Force split asymmetric: Default setting: Disabled Alternative setting: Enabled Parameters for Alternative setting: #define HHI_RQT_ROOT 1 #define HHI_RQT_FORCE_SPLIT_ASYM 1 | Force split rectangular: Default setting: Disabled Alternative setting: Enabled Parameters for Alternative setting: #define HHI_RQT_ROOT 1 #define HHI_RQT_FORCE_SPLIT_NxN 1 |

Among them, case 1 and 2 are tested by Microsoft and case 3 is tested by Sharp.

The following platform and compiler are used by Microsoft.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Testing results

## Case 1

The summary of the testing results of case 1 is shown in the following table.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.1 | 0.3 | 0.3 |
| Class C | 0.1 | 0.0 | 0.2 | 0.1 | 0.1 | 0.1 |
| Class D | 0.0 | 0.1 | 0.3 | 0.0 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 | 0.3 |
| Class C | 0.0 | 0.1 | 0.1 | 0.1 | 0.2 | 0.3 |
| Class D | 0.0 | 0.1 | 0.3 | 0.1 | 0.3 | 0.2 |
| Class E | -0.1 | 0.4 | 0.2 | 0.1 | 0.3 | 0.0 |
| All | 0.0 | 0.2 | 0.2 | 0.1 | 0.2 | 0.2 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |

The detail results can be found in the accompanying file JCTVC-C284-case1.xls

## Case 2

The summary of the testing results of case 2 is shown in the following table.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | -0.1 | 0.2 | 0.2 | 0.2 |
| Class B | 0.2 | 0.0 | 0.0 | 0.3 | 0.5 | 0.5 |
| Class C | 0.2 | 0.0 | 0.1 | 0.2 | 0.2 | 0.2 |
| Class D | 0.1 | 0.0 | -0.1 | 0.1 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.0 | 0.0 | 0.2 | 0.3 | 0.3 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.4 | 0.2 | 0.3 | 0.5 | 0.8 |
| Class C | 0.2 | 0.3 | 0.3 | 0.3 | 0.6 | 0.5 |
| Class D | 0.1 | 0.2 | 0.1 | 0.2 | 0.7 | 0.7 |
| Class E | 0.0 | 0.9 | 0.6 | 0.3 | 0.9 | 0.3 |
| All | 0.2 | 0.4 | 0.3 | 0.3 | 0.6 | 0.6 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## The detail results can be found in the accompanying file JCTVC-C284-case2.xls

## Case 3

The summary of the testing results of case 3 is shown in the following table.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.2 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.1 | -0.2 | 0.0 | 0.2 | 0.0 |
| Class D | 0.1 | 0.3 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class E | 0.1 | -0.3 | 0.2 | 0.0 | -0.1 | -0.3 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

The detail results can be found in the accompanying file JCTVC-C284-case3.xls