| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C285 WG11 Number: m18328 |
| --- | --- |

| Title: | Modified Uni-directional Inter Prediction in Generalized P and B Picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Peisong Chen, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes a modification of uni-directional inter-prediction in generalized P and B picture (GPB). The modification restricts the reference list selection for uni-directional inter-prediction. Simulation results show that 0.88% BD rate reduction can be achieved.

- Introduction

According to the current TMuC, when GPB is enabled, the reference picture list 0 (List_0) and reference picture list 1 (List_1) may be identical in a B picture. In a B picture, syntax element inter_pred_idc is sent to specify bi-directional, uni-directional (List_0)  or uni-directional (List_1) is used for the current partition.

# Proposed solution

As previously mentioned, a B picture may have two identical reference picture lists for the inter-prediction. Since two lists are the same, the syntax element inter_pred_idc contains redundancy when specifying reference picture list index for uni-directional inter-prediction. Therefore, we propose a simplified syntax for inter_pred_idc so that it specifies only bi-directional or uni-directional (List_0). Simulation results indicate this modification results in better coding performance. Moreover, it can potentially reduce encoder complexity by eliminating uni-directional motion search on one of the reference lists. When two reference picture lists are different, the syntax element inter_pred_idc remains unchanged.

# Simulation result

The software version used for this test was TMuC 0.7. The anchor was generated by using the default setting [2] for low-delay high-efficiency configuration. The proposed scheme was compared against the anchor. The encoder and decoder executables were compiled on 64-bit Linux and the simulations were performed on the Linux cluster.

| BD-rate | Low-delay high-efficiency |
| --- | --- |
| WQVGA30_RaceHorses | -1.03 |
| WQVGA50_BasketballPass | -0.98 |
| WQVGA50_BlowingBubbles | -0.87 |
| WQVGA60_BQSquare | -0.92 |
| WQVGA AVE | -0.95 |
| WVGA30_RaceHorses | -0.60 |
| WVGA50_BasketballDrill | -0.58 |
| WVGA50_PartyScene | -0.65 |
| WVGA60_BQMall | -1.40 |
| WVGA AVE | -0.81 |
| 720p60_Vidyo1 | -1.55 |
| 720p60_Vidyo3 | -1.30 |
| 720p60_Vidyo4 | -1.22 |
| 720p AVE | -1.36 |
| 1080p24_ParkScene | -0.77 |
| 1080p24_Kimono | -0.51 |
| 1080p50_BasketballDrive | -0.52 |
| 1080p50_Cactus | -0.75 |
| 1080p60_BQTerrace | -0.47 |
| 1080p AVE | -0.60 |
| Average | -0.88 |

# Conclusion

In this contribution, syntax element inter_pred_idc is simplified in a B picture with two identical reference picture lists to improve the coding efficiency. Experimental results show that the proposed modification achieves an average BD-rate reduction of 0.88% for the low-delay high-efficiency configuration. We propose that this modification be included into the JCT-VC test model.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).