| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C286 WG11 Number: m18329 |
| --- | --- |

| Title: | TE12.2: Technicolor & INRIA report on AMP evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | Edouard Francois, Technicolor Laurent Guillo, INRIA | Tel: Email: | +33-2-99-27-32-62 edouard.francois@technicolor.com laurent.guillo@inria.fr |
| Source: | Technicolor, INRIA |  |  |

_____________________________

# Abstract

This document reports the Technicolor & INRIA results for the TE12 AMP evaluation, also involving Huawei / Hisilicon, and NHK.

# Introduction

This document summarizes the Tool Experiment TE12 activities related to the evaluation of Asymmetric Motion Partitioning (AMP) [1].

This evaluation was performed by Huawei/HiSilicon, NHK, and Technicolor/INRIA.

# Tests conditions

Simulation results has been generated by exercising turning off AMP tool in the inter-coding configurations specified for TMuC 0.7 in JCTVC-B300 [2].

The following platforms and compilers have been used:

| company | platform | Compiler |
| --- | --- | --- |
| Huawei/HiSilicon | Windows Server 2003 64 bits | VS2005 |
| NHK | Windows XP 64bits | VS2008 |
| Technicolor/INRIA | Fedora Core 1164 bits | gcc 4.4.1 |

The testers have run each test case with ‘AMP on’ and ‘AMP off’ using computers with the same configuration, in order to obtain comparable encoding and decoding times for both configurations.

Technicolor/INRIA was tester1 for Random Access High Efficiency and Low Delay High Efficiency cases; the tester2 was Huawei/HiSilicon.

Technicolor/INRIA was tester2 for Low Delay Low Complexity case; the tester1 was NHK.

# Cross-checking results

The testers have observed mismatches when using Linux or Windows platforms. However the differences remain tiny, the results from different platforms can be considered unambiguous. The differences are described as the following table.

| Test | Tester1 | Tester2 | avg rate (%) |  |  | min rate (%) |  |  | max rate (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | Y | U | V | Y | U | V |
| RA / HE / AMP on | Techni/INRIA | Huawei/HiSilicon | 0.0 | 0.0 | 0.0 | 0.0 | -0.3 | -0.1 | 0.0 | 0.1 | 0.1 |
| RA / HE / AMP off | Techni/INRIA | Huawei/HiSilicon | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| LD / HE / AMP on | Techni/INRIA | NHK | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.0 |
| LD / HE / AMP off | Techni/INRIA | NHK | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| LD / LC / AMP on | NHK | Techni/INRIA | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| LD / LC / AMP off | NHK | Techni/INRIA | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |

Table legend:

- HE: High Efficiency

- LC: Low Complexity

- RA: Random Access

- LD: Low Delay

- Tester using linux is in Grey cell

- Tester using Windows64 is in White cell

- Mismatches are indicated in red font

The full cross-test results are reported in JCTVC-C030 [3].

# AMP performance

The full evaluation results can be found in file JCTVC-C286_TE12-AMP-TechnicolorInria.xls.

The following tables summarize the AMP performance evaluation obtained by Technicolor/INRIA:

|  |  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |  |  |  |
| Class A | 1.5 | 1.6 | 1.5 |  |  |  |
| Class B | 1.0 | 1.1 | 1.1 |  |  |  |
| Class C | 1.4 | 1.3 | 1.6 |  |  |  |
| Class D | 1.3 | 1.3 | 1.5 |  |  |  |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.3 | 1.4 |  |  |  |
| Enc Time[%] | 65% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.2 | 1.7 | 1.9 | 1.8 | 1.4 | 1.3 |
| Class C | 1.3 | 1.6 | 1.7 | 1.7 | 1.5 | 1.7 |
| Class D | 1.4 | 1.8 | 1.7 | 1.6 | 1.4 | 1.3 |
| Class E | 2.8 | 3.7 | 2.7 | 3.8 | 3.6 | 3.0 |
| All | 1.6 | 2.1 | 2.0 | 2.1 | 1.8 | 1.7 |
| Enc Time[%] | 65% |  |  | 65% |  |  |
| Dec Time[%] | 101% |  |  | 94% |  |  |

According to the tables, it can be seen that turning AMP off leads to average BD-rate increases of respectively 1.3% for Random Access High Efficiency, 1.6% for Low Delay High Efficiency, 2.1% for Low Delay Low Complexity. The maximum loss per class is of 3.8% on classE for Low Delay Low Complexity.

It is also observed that switching AMP off leads to an encoder time decrease of around 65% while the decoder time is slight increased.

# References

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-A312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

- Frank Bossen, “Common test conditions and software reference configurations,” Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010.

- Edouard Francois, Laurent Guillo, Atsuro Ichigaya, Haoping Yu, “TE12 : report on AMP evaluation,” Doc. JCTVC-C030 Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, Oct 2010.