| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C047 WG11 Number: m18330 |
| --- | --- |

| Title: | Cross-check of encoder speedup for bidirectional averaging with rounding control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Cross-check report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking the results reported in JCTVC-C253 [1].

- Simulation environment

## Software

The software used to run the simulations is a modified version of TMuC 0.8 [2]. The binaries were compiled on a 64 bit Ubuntu 9.04 Linux using the included make files and gcc 4.3.3. The coding conditions used for the simulations are specified in [3]. The default settings for low delay / high efficiency, both for configuration file and compile-time, are used. Changes from this anchor configuration are listed in the next sections.

### Alternative configuration file settings

The following parameters are set for all results.

BitIncrement 	: 0

MRG 			: 0

### Alternative compile-time settings

The following macros in TypeDef.h have been changed:

Reference:

//#define ROUNDING_CONTROL_BIPRED ///< From JCTVC-B074

//#define ROUNDING_CONTROL_BIPRED_SPEEDUP

//#define ROUNDING_CONTROL_BIPRED_SPEEDUP_BITEXACT

BiRound:

#define ROUNDING_CONTROL_BIPRED ///< From JCTVC-B074

//#define ROUNDING_CONTROL_BIPRED_SPEEDUP

//#define ROUNDING_CONTROL_BIPRED_SPEEDUP_BITEXACT

BiRound + All Speedups

#define ROUNDING_CONTROL_BIPRED ///< From JCTVC-B074

#define ROUNDING_CONTROL_BIPRED_SPEEDUP

#define ROUNDING_CONTROL_BIPRED_SPEEDUP_BITEXACT

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Linux operating system. Their hardware is listed in  Table 1-1.

Table 1-1 Computers used to run the simulations.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 24 | Intel(R) Xeon(R) CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 9.04, 64 bit |
| 2 | 8 | Intel(R) Xeon(R) CPU X5460 @ 3.16GHz | 32 GB | Ubuntu 9.04, 64 bit |
| 3 | 8 | Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 9.04, 64 bit |

# Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. Detailed results can be found in JCTVC-C287.xls.

|  |  |  |  |  | BiRound |  |  |  |  |  | BiRound+Speedup |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |
|  | Class B |  | Kimono |  | -0.22 |  | -0.41 |  | -0.67 |  | -0.13 |  | -0.40 |  | -0.73 |
|  |  |  | Parkscene |  | -0.22 |  | -0.67 |  | -1.41 |  | -0.18 |  | -0.59 |  | -0.93 |
|  |  |  | Cactus |  | -1.41 |  | -1.85 |  | -2.18 |  | -1.38 |  | -1.63 |  | -2.04 |
|  |  |  | Basketball |  | -0.56 |  | -1.91 |  | -1.15 |  | -0.46 |  | -1.71 |  | -1.04 |
|  |  |  | BQSquare |  | -1.40 |  | -4.39 |  | -7.34 |  | -0.94 |  | -3.92 |  | -6.68 |
|  |  |  | Avg |  | -0.76 |  | -1.85 |  | -2.55 |  | -0.62 |  | -1.65 |  | -2.28 |
|  | Class C |  | Basketball |  | -0.43 |  | -0.80 |  | -0.50 |  | -0.13 |  | -0.76 |  | -0.38 |
|  |  |  | BQSquare |  | -0.73 |  | -0.47 |  | -1.41 |  | -0.57 |  | -0.31 |  | -1.22 |
|  |  |  | Partyscene |  | -0.56 |  | -0.95 |  | -0.79 |  | -0.43 |  | -0.85 |  | -0.72 |
|  |  |  | Racehorse |  | -0.14 |  | -0.37 |  | -0.75 |  | -0.12 |  | -0.52 |  | -0.62 |
|  |  |  | Avg |  | -0.47 |  | -0.65 |  | -0.86 |  | -0.31 |  | -0.61 |  | -0.74 |
|  | Class D |  | Basketball |  | -0.20 |  | -1.38 |  | -0.03 |  | -0.11 |  | -1.01 |  | -0.07 |
|  |  |  | BQSquare |  | -1.21 |  | -1.73 |  | -1.95 |  | -0.98 |  | -0.96 |  | -1.54 |
|  |  |  | Partyscene |  | -0.61 |  | -0.66 |  | -0.81 |  | -0.50 |  | -0.66 |  | -1.13 |
|  |  |  | Racehorse |  | -0.09 |  | -0.38 |  | -0.38 |  | -0.10 |  | -0.81 |  | -0.29 |
|  |  |  | Avg |  | -0.53 |  | -1.04 |  | -0.79 |  | -0.42 |  | -0.86 |  | -0.76 |
|  | Class E |  | Vidyo1 |  | -3.34 |  | -5.26 |  | -6.69 |  | -2.87 |  | -4.56 |  | -5.77 |
|  |  |  | Vidyo3 |  | -3.15 |  | -2.16 |  | -4.75 |  | -2.71 |  | -2.13 |  | -4.82 |
|  |  |  | Vidyo4 |  | -3.52 |  | -6.00 |  | -5.84 |  | -3.36 |  | -5.41 |  | -5.01 |
|  |  |  | Avg |  | -3.34 |  | -4.47 |  | -5.76 |  | -2.98 |  | -4.04 |  | -5.20 |
|  |  |  | All |  | -1.11 |  | -1.84 |  | -2.29 |  | -0.94 |  | -1.64 |  | -2.06 |
|  |  |  | Enc Time[%] |  | 104% |  |  |  |  |  | 101% |  |  |  |  |
|  |  |  | Dec Time[%] |  | 99% |  |  |  |  |  | 100% |  |  |  |  |

# References

- JCT-VC, “Encoder speedup for bidirectional averaging with rounding control”, JCTVC-C253, Input document to JCT-VC Meeting, Guangzhou, October 2010.

- JCT-VC, “Test Model under Consideration Software 0.8”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.8

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.