| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C288 WG11 Number: m18331 |
| --- | --- |

| Title: | TE4: Verification results of JCTVC-C262 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution shows verification results of JCTVC-C262 (Report on VLC for coded block flag) for TE4. Results are slightly different but not dominant. No mismatch between encoder and decoder is observed. Encoding and decoding time comparison with the TMuC 0.7.4 anchor (including macro change) is matched.

# Introduction

This contribution is a report of verification results of JCTVC-C262 [1], which is a contribution to TE4.

Verification is performed under the TE4 test conditions, but TMuC 0.7.4 is used with the followings macro modification:

LCEC_PHASE2 = 1

HHI_RQT = 0

HHI_RQT_CHROMA_CBF_MOD = 0

HHI_RQT_INTRA = 0

# Results

Table 1  BD-rate (%) and relative encoding and decoding time (%)

|  |  |  | Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Intra only | Random Access | Low Delay |
| A | S01 | Traffic | -0.2 | -0.2 | N/A |
|  | S02 | PeopleOnStreet | -0.1 | -0.1 | N/A |
| B | S03 | Kimono | -0.2 | -0.1 | -0.1 |
|  | S04 | ParkScene | -0.2 | -0.2 | -0.2 |
|  | S05 | Cactus | -0.2 | -0.2 | -0.2 |
|  | S06 | BasketballDrive | -0.2 | -0.1 | -0.2 |
|  | S07 | BQTerrace | -0.3 | -0.2 | -0.2 |
| C | S08 | BasketballDrill | -0.1 | 0.0 | -0.2 |
|  | S09 | BQMall | -0.2 | -0.1 | -0.1 |
|  | S10 | PartyScene | -0.4 | -0.2 | -0.2 |
|  | S11 | RaceHorses | -0.2 | -0.2 | -0.2 |
| D | S12 | BasketballPass | -0.2 | -0.2 | -0.2 |
|  | S13 | BQSquare | -0.3 | -0.2 | -0.1 |
|  | S14 | BlowingBubbles | -0.4 | -0.2 | -0.2 |
|  | S15 | RaceHorses | -0.3 | -0.1 | -0.1 |
| E | S16 | Vidyo1 | -0.1 | N/A | 0.0 |
|  | S17 | Vidyo3 | -0.1 | N/A | -0.2 |
|  | S18 | Vidyo4 | -0.2 | N/A | -0.1 |
| Class A |  |  | -0.2 | -0.1 | N/A |
| Class B |  |  | -0.2 | -0.2 | -0.2 |
| Class C |  |  | -0.2 | -0.1 | -0.2 |
| Class D |  |  | -0.3 | -0.2 | -0.2 |
| Class E |  |  | -0.2 | N/A | -0.1 |
| Total |  |  | -0.2 | -0.2 | -0.2 |
| Encoding time |  |  | 100 | 100 | 100 |
| Decoding time |  |  | 100 | 100 | 100 |

There is no mismatch between encoder and decoder.

Relative encoding and decoding time is the same as that of proponent’s

There is no mismatch of overall BD-rate between the verification results and the proponent’s results. However, there are slight mismatches of coding results (bitrate and/or PSNR) for several cases. A potential reason is that the proponent is using Linux OS and the verifier used Windows OS.

# Conclusion

This contribution shows verification results of JCTVC-C262. Results are exactly matched each other. No mismatch between encoder and decoder is observed. Encoding and decoding time comparison with the TMuC 0.7.4 anchor (including macro change) does not show large differences.

# Reference

[1]	X. Wang, M. Karczewicz, W.-J. Chien, “TE4: Report on VLC for coded block flag”, JCTVC-C262, October 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).