| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C289 WG11 Number: m18332 |
| --- | --- |

| Title: | TE11: Report on experiment 3.1.a: disabling MV Competition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Jung, G. Clare, S. Pateux Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com gordon.clare@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Introduction

As requested during the Geneva meeting, this experiment assesses the gain provided by MV-Competition. The competition for the Inter, the Skip and the Direct modes is disabled, only the median predictor is enabled, as in AVC. The document reports an average loss of 2.6% to 4.1% when the MV-Competition is disabled.

# Experiments

RD-Results:

Reference: MV-Comp enabled on Inter, Skip and Direct modes (MRG=0 - HHI_AMVP_OFF=0). Negative values in the tables indicate a BD-rate loss for median-only prediction against this reference.

Random Access:

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -4,1 | -3,5 | -3,5 |
| Class B | -3,8 | -2,9 | -2,8 |
| Class C | -3,1 | -3,0 | -3,2 |
| Class D | -3,2 | -2,5 | -2,9 |
| Class E | - | - | - |
| Average | -3,5 | -2,9 | -3,0 |

Random Access LOCO:

|  | Random Access LOCO |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -5,1 | -4,5 | -4,5 |
| Class B | -4,2 | -2,9 | -2,9 |
| Class C | -3,8 | -3,9 | -4,2 |
| Class D | -3,8 | -3,2 | -3,5 |
| Class E |  |  |  |
| Average | -4,1 | -3,5 | -3,6 |

Low Delay:

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -2,6 | -1,9 | -1,6 |
| Class C | -2,9 | -2,8 | -2,6 |
| Class D | -2,4 | -1,5 | -1,8 |
| Class E | -2,7 | -1,6 | -2,5 |
| Average | -2,6 | -2,0 | -2,1 |

Low Delay LOCO:

|  | Low Delay LOCO |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -4,0 | -3,5 | -3,7 |
| Class C | -3,8 | -4,0 | -4,2 |
| Class D | -3,3 | -3,2 | -3,4 |
| Class E | -4,9 | -3,9 | -4,7 |
| Average | -3,9 | -3,6 | -3,9 |

# Conclusion

Conclusion: removing the MV competition on Inter, Skip and Direct modes provides an average coding loss of 2.6% to 4.1% depending on the configuration. The loss is systematic, whatever the bitrate or the sequence tested.