| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C291 WG11 Number: m18334 |
| --- | --- |

| Title: | TE11: Report on experiment 3.3.b: ‘temporally oriented’ set of predictors for MV-Competition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Jung, G. Clare, S. Pateux Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com gordon.clare@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Introduction

In contribution JCTVC-B081 by Sony, it is mentioned that the temporal predictor is efficient between a moving object and stationary background. A dynamic code number assignment for each of the motion predictor candidate was proposed. With camera-fixed-type sequences assignment of smaller code numbers for the temporal predictor can improve visual quality as well as coding efficiency. The contribution asked the ability to adapt code number assignment (can be seen as the modification of the set of predictors) in the Slice Header (only possible at the SPS level with the current TMuC), as the best predictor may change if scene change occurs in the sequence. Additionally, in the MV-Competition contribution [1] adopted in the KTA software in 2006, different set of predictors were used for Inter or Skip competition.

This contribution reports that changing the order of the predictors in the set (or use different sets of predictors) brings gains up to 1.5% (sequence dependant), and that it is appropriate to signal such changes at the slice level to adapt the prediction to the sequence characteristic.

# Experiment 3.3.b: temporal predictor moved to 1st position

2.1- Experiment description

The default set of predictors in the reference TMuC is the following:

- M (median of A, B, C below)

- A (1st left neighbor found)

- B (1st top neighbor found)

- C (a corner, 1st of C, E, D neighbours)

- T (a temporal, found by mapping the origin of the block to a collocated frame)

The above is the initial order – the median can be swapped with either A or B depending on partitioning of the block and the partition being coded. It can be seen that the above set is “spatially oriented” – it is more expensive to signal the use of T as its index in the set is the largest.

In this experiment, the following set of predictors is used:

- T (a temporal, found by mapping the origin of the block to a collocated frame)

- M (median of A, B, C below)

- A (1st left neighbor found)

- B (1st top neighbor found)

- C (a corner, 1st of C, E, D neighbours)

The above is the initial order – as before the median can be swapped with either A or B, but the temporal vector always remains first in the set. It can be seen that this set is a “temporally oriented” set – it is cheaper than before to signal the use of the temporal predictor as its index is 0.

2.2- BD-rate results

Experiments performed on RAHE configuration only Class E sequences have been added as it contains well known fixed-camera type sequences. The reference is either 3.2.b or the TMuC reference. Negative values indicate a BD-rate gain for 3.3.b against this reference.

|  | Random Access | Random Access |
| --- | --- | --- |
|  | Y BD-rate | Y BD-rate |
|  | Ref: 3.2.b | TMuC Ref |
| ClassA | -0,4 | -1.2 |
| ClassB | -0.4 | -1.2 |
| ClassC | -0.3 | -1.1 |
| ClassD | -0.4 | -1.1 |
| ClassE | -0,9 | x |
| AVG | -0,5 |  |

Although the average gain is low, the class E results suggest having a closer look at sequence by sequence results. In particular, it can be observed that sequences with the highest gains correspond to sequences with moving objects on a static background:

| Sequence | Vidyo1 | Cactus | Vidyo4 | BQMall |
| --- | --- | --- | --- | --- |
| Y BD-rate | -1.5 | -1.0 | -0.7 | -0.6 |

In addition, it can be observed that the sequences that do not provide any gain correspond to non static background sequences.

| Sequence | BQTerrace | RaceHorses |
| --- | --- | --- |
| Y BD-rate | 0 | 0 |

2.3- Second by second analysis of “temporal” prediction

If we select a sequence with characteristics that vary during the time, such as “Kimono” (scene cut) or “BasketballPass” (static background followed by camera motion), we can observe that the average gain is low (0.5% and 0.4% respectively). However, when drawing the BD-rate gain second by second, it can be seen that it is completely correlated with the camera motion.

In the case of “Kimono”, there are 1% gains using the temporal in position zero, but only at the end of the sequence after the scene cut. In the case of “Basketballpass”, there are 1% gains during the initial second (where there is no camera movement).  Gains are reduced when there are panning movements.  When the panning stops temporarily (around the 6s mark) gains improve.

# Conclusion

The experiment performed in this document show that:

- The ability to change the order of predictors (use different sets of predictors) brings significant gains (up to 1.5%)

- The ability to signal such changes at the frame level is necessary to adapt the behavior to the sequence characteristic.

# References

- G. Laroche, J. Jung, Competition-Based Scheme for Motion Vector Selection and Coding, VCEG-AC06 contribution, 31th Meeting: Klagenfurt, Austria, 17-18 July 2006.