| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C296 WG11 Number: m18341 |
| --- | --- |

| Title: | Decoding Improvement on the PA-Coder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Tel: Email: | hurricanezhb@hotmail.com |
| Source: | Hongbo Zhu |  |  |

_____________________________

# Abstract

The proposal provides a fast channel for MPS decoding of the PA-coder [1][2]. Once a new bit is decoded, a threshold value is computed using a table lookup in the previous design. Compare the threshold value with the current value to decide the MPS or LPS. The procedure is as follows:

int prbadd1 = lgprobaddvl[bi_ct->state][1];

int rngprob = dep->rngprob;

unsigned int thrshld = dvsn_interval(prbadd1+rngprob);

int symbol = bi_ct->MPS;

if(thrshld <= dep->upvalue)

{

int prbadd0 = lgprobaddvl[bi_ct->state][0];

dep->upvalue -= thrshld;

dep->rngprob += prbadd0;

symbol ^= 1;

if (!state)          // switch meaning of MPS if necessary

bi_ct->MPS ^= 0x01;

bi_ct->state = AC_next_state_LPS_64[state]; // next state

}

else

{

dep->rngprob += prbadd1;

bi_ct->state = AC_next_state_MPS_64[state]; // next state

}

The new algorithm maps the upvalue to a rngprbOfVal once the upvalue is changed. So in most cases when decoding the MPS, the threshold value is not needed with the price of mapping the upvalue to logprob when the upvalue is changed. The mapping follows the following rules:maxrngprbOfVal|IrngprbOfVal>upvalue, in which Ix denotes the range of the log prob x. The procedure is as follows:

const unsigned short val2lngprb[256] = {

32736,28640,26272,24544,23232,22176,21248,20448,19776,19136,18592,18080,17600,17152,16736,16352,

16000,15680,15360,15040,14752,14496,14208,13984,13728,13504,13280,13056,12864,12640,12448,12256,

12096,11904,11744,11584,11424,11264,11104,10944,10816,10656,10528,10400,10272,10112,10016, 9888,

9760, 9632, 9504, 9408, 9280, 9184, 9056, 8960, 8864, 8768, 8672, 8544, 8448, 8352, 8256, 8160,

8096, 8000, 7904, 7808, 7744, 7648, 7552, 7488, 7392, 7328, 7232, 7168, 7072, 7008, 6944, 6848,

6784, 6720, 6624, 6560, 6496, 6432, 6368, 6304, 6240, 6176, 6112, 6016, 5952, 5920, 5856, 5792,

5728, 5664, 5600, 5536, 5472, 5408, 5376, 5312, 5248, 5184, 5152, 5088, 5024, 4960, 4928, 4864,

4832, 4768, 4704, 4672, 4608, 4576, 4512, 4448, 4416, 4352, 4320, 4256, 4224, 4160, 4128, 4064,

4032, 4000, 3936, 3904, 3840, 3808, 3776, 3712, 3680, 3648, 3584, 3552, 3520, 3456, 3424, 3392,

3328, 3296, 3264, 3232, 3168, 3136, 3104, 3072, 3040, 2976, 2944, 2912, 2880, 2848, 2784, 2752,

2720, 2688, 2656, 2624, 2592, 2528, 2496, 2464, 2432, 2400, 2368, 2336, 2304, 2272, 2240, 2208,

2176, 2144, 2112, 2080, 2048, 2016, 1952, 1920, 1888, 1856, 1824, 1824, 1792, 1760, 1728, 1696,

1664, 1632, 1600, 1568, 1536, 1504, 1472, 1440, 1408, 1376, 1344, 1312, 1312, 1280, 1248, 1216,

1184, 1152, 1120, 1088, 1056, 1056, 1024,  992,  960,  928,  896,  864,  864,  832,  800,  768,

736,  736,  704,  672,  640,  608,  576,  576,  544,  512,  480,  480,  448,  416,  384,  352,

352,  320,  288,  256,  256,  224,  192,  160,  160,  128,   96,   64,   64,   32,    0,    0

};

#define mapVal2LngPrb(val, curlngprb, lngprb) {int high4=curlngprb>>12; lngprb=(curlngprb&0xf000)+val2lngprb[val>>(16-high4)];} //map upvalue to lngprob

int prbadd1 = lgprobaddvl[bi_ct->state][1];

int rngprob = dep->rngprob;

unsigned int thrshld = dvsn_interval(prbadd1+rngprob);

int symbol = bi_ct->MPS;

if(dep->rngprbOfVal >= prbadd1+rngprob)//fast channel for MPS

{

dep->rngprob += prbadd1;

bi_ct->state = AC_next_state_MPS_64[state]; // next state

}

else

{

if(thrshld <= dep->upvalue)

{

int prbadd0 = lgprobaddvl[bi_ct->state][0];

dep->upvalue -= thrshld;

dep->rngprob += prbadd0;

symbol ^= 1;

if (!state)          // switch meaning of MPS if necessary

bi_ct->MPS ^= 0x01;

bi_ct->state = AC_next_state_LPS_64[state]; // next state

mapVal2LngPrb(dep->upvalue, dep->rngprob, dep->rngprbOfVal)

}

else

{

dep->rngprob += prbadd1;

bi_ct->state = AC_next_state_MPS_64[state]; // next state

}

}

- Reference

- Hongbo Zhu, “Arithmetic coding based on probability-aggregation and delayed-subdivision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010, Document JCTVC-A027.

- Hongbo Zhu, “Some improvement of the arithmetic coding based on probability aggregation”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 2st Meeting: Geneva, CH, 21-28 July, 2010, Document JCTVC-B091.

# Patent rights declaration(s)

Hongbo Zhu may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).