| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C297 WG11 Number: m18344 |
| --- | --- |

| Title: | Cross-verification of JCTVC-C213: additional results for combined intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woo-Jin Han | Email: | wjhan.han@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution is the cross-verification report of JCTVC-C213: additional results for combined intra prediction. Provided source code was investigated. Both encoder and decoder running were done.

- Cross-verification

TMuC 0.7 software has been used and the provided patch file (several lines of changes) was applied to the software. Provided patch includes (1) adjustment of weights between open-loop and close-loop predictors (2) removal of encoder-side penalty due to the fast estimation process of the combined intra prediction. Both changes seem not relevant to the coding tool itself.

For the configuration parameter, TE12 default parameters were used.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -2.6 | -2.8 | -1.2 | -2.4 | -2.3 |
| Class B | -0.6 | -1.5 | -1.5 | -0.8 | -1.5 | -1.5 |
| Class C | -0.8 | -1.2 | -1.2 | -0.9 | -1.1 | -1.1 |
| Class D | -0.8 | -1.0 | -1.0 | -1.0 | -1.1 | -1.1 |
| Class E | -0.8 | -2.2 | -2.2 | -0.9 | -2.0 | -1.9 |
| All | -0.7 | -1.5 | -1.6 | -0.9 | -1.5 | -1.5 |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.9 | -0.8 | -0.4 | -0.9 | -1.1 |
| Class B | -0.3 | -0.5 | -0.4 | -0.2 | -0.6 | -0.7 |
| Class C | -0.5 | -0.4 | -0.7 | -0.3 | -0.5 | -0.5 |
| Class D | -0.4 | -0.7 | -0.6 | -0.3 | -0.4 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.6 | -0.6 | -0.3 | -0.6 | -0.7 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.3 | -0.5 | 0.0 | -0.2 | -0.1 |
| Class C | -0.2 | -0.1 | -0.2 | -0.1 | -0.2 | -0.1 |
| Class D | -0.2 | -0.2 | 0.1 | -0.1 | -0.2 | 0.0 |
| Class E | 0.3 | 0.4 | 0.2 | 0.1 | -1.0 | 0.1 |
| All | -0.1 | -0.1 | -0.2 | 0.0 | -0.4 | 0.0 |

- Conclusions

All results are perfectly matched to those of JCTVC-C213.