| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C302 WG11 Number: m18517 |
| --- | --- |

| Title: | Improved intra smoothing for UDI and new AIS fast mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tung Nguyen Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this contribution, an adaption of intra smoothing scheme for UDI is proposed to improve the overall coding performance. Furthermore, this contribution presents an improved fast mode for AIS.

- Introduction

In ADI, reference pixel smoothing depends on a lookup table [3]. This table contains the number of recursive applications of the [1 2 1]/4 low-pass filter, where no filtering, single-pass filtering and two-pass filtering are possible. In contrast, no such table is used for UDI and a single-pass filtering is always performed except for intra DC mode. This behavior leads to large loss in case of default intra smoothing (DEFAULT_IS=1 and AIS [2] is disabled). Short experiments indicated a loss for about 2.23% compared to no smoothing case in Intra only high efficiency configuration. When AIS is enabled, a gain for about 0.4% was observed compared to no smoothing. This leads to large gain by comparing default intra smoothing against AIS in [4] and [5].

# Proposed scheme

The proposed scheme identifies the default intra smoothing approach of UDI as inefficiency result in reported loss compared to no smoothing case. Therefore, for this proposal the usage of intra smoothing table is forced and new values for the table are derived. Additionally, the context model selection for AIS is adapted to UDI. Furthermore, a new fast mode of AIS is presented, which reduces the complexity of AIS for about 65% by retaining about 90% of the efficiency of AIS.

## Smoothing table

This contribution proposed following smoothing table when UDI is used:

const UChar g_aucIntraFilter[7][40] =

{

{

0,0,0,0,0,  0,0,0,0,0,    // 2x2

0,0,0,0,0,  0,0,0,0,0,

0,0,0,0,0,  0,0,0,0,0,

0,0,0,0,0,  0,0,0,0,0

},

{

0,0,0,0,0,  0,0,0,0,0,    // 4x4

0,0,0,0,0,  0,0,0,0,0,

0,0,0,0,0,  0,0,0,0,0,

0,0,0,0,0,  0,0,0,0,0

},

{

0,0,0,2,2,  2,2,2,2,0,    // 8x8

0,0,0,0,0,  0,0,0,0,0,

0,0,0,0,0,  0,0,0,0,0,

0,0,0,0,0,  0,0,0,0,0,

},

{

0,0,0,2,2,  2,2,2,2,2,  // 16x16

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

},

{

2,2,0,2,2,  2,2,2,2,2,   // 32x32

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

},

{

2,2,0,2,2,  2,2,2,2,2,  // 64x64

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

},

{

2,2,0,2,2,  2,2,2,2,2,   // 128x128

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

2,2,2,2,2,  2,2,2,2,2,

}

};

## Context Modeling for AIS

The context model selection is based on the intra prediction mode. In this proposed scheme, the number of contexts is extended from 5 to 15; where horizontal, vertical, angular prediction modes and block size are distinguished.

## Proposed AIS fast mode

In the current AIS fast mode, only the best candidate is tested. Instead of testing only the best candidate, the proposed scheme tests a several number of best candidates. For this, the RD cost range (rd_range) between best (rd_best) and worst candidate is calculated and only candidates with RD costs lower than 3/8 * rd_range + rd_best are tested. The threshold value is derived by extensive simulations and has a good trade-off between complexity and efficiency. To estimate the complexity of the proposed fast mode, the number of candidates tested by AIS and the total number of candidates was counted. For Intra only configurations in high efficiency, only 35% of all possible candidates are tested.

# Results

The branch 0.8-hhi-bugfix is used to compare the proposed scheme. Conditions as specified in [1] were used and the reference is the AIS enabled configuration.

Table 1: No Smoothing

Table 2: No AIS

Table 3: Proposed Fast Mode

# References

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- JCT-VC, “Video coding technology proposal by Samsung (and BBC)”, JCTVC-A124, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- JCT-VC, “TE12.3: Adaptive intra smoothing (AIS) test (slow vs. fast) by Fraunhofer HHI”, JCTVC-053, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- JCT-VC, “TE12.3: Adaptive intra smoothing (AIS) test (off vs. on) by Fraunhofer HHI”, JCTVC-C054, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).