| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C305 WG11 Number: m18544 |
| --- | --- |

| Title: | TE7: Results for Simplification of MDDT Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wenpeng Ding, Yunhui Shi, Baocai Yin Pingleyuan 100# Chaoyang district Beijing, China | Tel: Email: | wpding@bjut.edu.cn |
| Source: | Beijing University of Technology |  |  |

_____________________________

# Abstract

MDDT is adopted in TMUC to improve the transform efficiency by using different transform matrices for residual block of different intra mode. However, MDDT is implemented as directly matrix multiplication, which results in higher computation cost than that of DCT. In this contribution, we designed two symmetric transform matrices to simplify the MDDT matrices. And the proposed symmetric transform matrices can be implemented using lifting structure. Experimental results demonstrate that this proposal reduce the run-time complexity of MDDT without significant coding performance loss.

- Introduction

To simplify MDDT [1], the transform matrices are approximated using eq (1).

SN=PNU00VIJJ−I.		(1)

where U and V can be any orthogonal matrices and PN is a known reordering matrix defined as

PN(m,n)=1,                      m=2n,  n<N21,  m=(n−N2)∗2+1,n≥N20,                              otherwise.

For N=8, the reordering matrix P8 is

P8=1000000000001000010000000000010000100000000000100001000000000001.

The implementation of SN is illustrated in Fig.1 for N=8. The whole transform is the concatenation of a simple butterfly transform, two separate transform U and V of halved data size, and a data reordering P.

Fig. 1. The butterfly implementation structure for S8.

Thus we obtained one symmetric 8x8 transform matrix and one symmetric 4x4 transform matrix. The matrices are mapped to intra prediction mode as follows

Table I: Vertical transform matrices mapping

| 0 | KLT |
| --- | --- |
| 1 | DCT |
| 2 | DCT |
| 3 | S |
| 4 | S |
| 5 | S |
| 6 | S |
| 7 | S |
| 8 | S |

Table II: Horizontal transform matrices mapping

| 0 | DCT |
| --- | --- |
| 1 | KLT |
| 2 | DCT |
| 3 | S |
| 4 | S |
| 5 | S |
| 6 | S |
| 7 | S |
| 8 | S |

The designed symmetric transform matrices are adopted for intra mode 3~8.

- Simulation environment

The proposed scheme is tested against the test conditions as specified in TE7. The implementation is based on TMuC 0.7. Tests were conducted using intra high efficiency coding configurations [5].

We compare our proposed scheme with the TMuC anchor. And the transform matrices are mapped as in TMuC 0.7. Only the transform matrices are replaced with new ones.

- Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C305.xls. Table III summaries the test results. Symmetric transform matrices will result in about 0.5 percent BD-rate increases.

Table III: Average BD-rate for all test sequences

| HE Intra |  |  |  |
| --- | --- | --- | --- |
| BD-rate | Y | U | V |
| Class A | 0.6 | 0.5 | 0.4 |
| Class B | 0.5 | 0.5 | 0.5 |
| Class C | 0.5 | 0.5 | 0.5 |
| Class D | 0.4 | 0.5 | 0.4 |
| Class E | 0.5 | 0.5 | 0.5 |
| All | 0.5 | 0.5 | 0.5 |

…

- Conclusion

Experimental results demonstrate that this proposal reduce the run-time complexity of MDDT without significant coding performance loss.

[1] Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[2]W. Ding, Y. Shi and B. Yin, “Fast Mode Dependent Directional Transform via Butterfly-Style Transform and Integer Lifting Steps”, Geneva, Switzerland, Jul 2010.

[3] JCT-VC, “Test Model under Consideration (TMuC),” JCTVC-B205, Geneva, Switzerland, Jul 2010.

[4] R. Cohen et al., “Tool Experiment 7: MDDT Simplification,” JCTVC-B307, Geneva, Switzerland, Jul 2010.

[5] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-B300, Geneva, Switzerland, Jul 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Beijing University of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).