| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C307 WG11 Number: m18548 |
| --- | --- |

| Title: | Cross Verification of Low Complexity Adaptive Coefficient Scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinwen Zan Jin Meng Mohammad Towhidul Islam Dake He 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 jzan@rim.com dhe@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on “low complexity adaptive coefficient scanning” as described in JCTVC-C250.

Adaptive coefficient scanning has been employed to exploit the DCT coefficient statistics based on the intra prediction mode. After encoding each CU, scanning orders are updated with a full sorting and statistics normalization. However, this operation introduces significant complexity, especially to the decoder.

- Tested tools

In order to reduce the computational complexity of the adaptive scanning, a partial sorting algorithm has been introduced in JCTVC-C250.

A modified version of TMuC-0.8 software provided by Qualcomm is used for the cross verification. To enable/disable the algorithm in JCTVC-C250, a macro

#define FAST_ADAPTIVE_SCAN              1

has been define: enabling it tests the performance of the algorithm proposed in JCTVC-C250; and disabling it gives the performance of the TMuC-0.8, which is used as anchor data for this test.

The encoding tests were run on a 64-bit Linux cluster, and decoding tests on Windows-XP (32-bit). Running decoding tests on Windows was suggested by Qualcomm, as big variance on decoding time had been reported.

# Cross verification results

Due to the rather late request of cross verification from Qualcomm, RIM agreed and was able to test 5 sequences from each class; and for each test sequence, two qp values (22 and 37) for the “intra” and “intra-loco” modes.

The test results have been compared with that from Qualcomm: 1) for the tests that macro “FAST_ADAPTIVE_SCAN” were enabled, both results matched on BD-Rate, and PSNRs on all test points; 2) for the anchor tests that macro“FAST_ADAPTIVE_SCAN” were disabled, initially mismatched were observed between the RIM anchor results and that from Qualcomm in the “intra_loco” mode except for the Class D sequence, although the difference is very minor (<0.1%). After the mismatch being reported, Qualcomm ran another round of anchor tests. The new anchor results obtained so far for Classes C, D, E(1 missing point) match the RIM anchor results.

On decoding time, for the “intra_loco” mode, an average of 43% decoding time was recorded on all available test points, and the Qualcomm data was an average of 37% on all qps and all sequences; for the “intra” mode, an average of 60% decoding time was recorded on all available test points, and the Qualcomm data was an average of 51% on all qps and all sequences.

# Tool analysis

It is noted that the tests could only be done on some selected sequences, with the lowest and highest qp values, due to the very tight time frame. From the data collected on these test points, it is shown that the decoding time is greatly reduced.

# Patent rights declaration(s)

Research In Motion Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).