| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C309 |
| --- | --- |

| Title: | Cross verification of DCT-IF for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda and Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross check results on DCT-IF for chroma. In the test, the coding performance and complexity was measured under common test conditions, which is defined in JCTVC-B300[1]. Proposed tools have been evaluated on the common condition. Detailed results are summarized in the attached Excel sheet. The source code was given by Samsung to cross check. We ran simulation using the source code.

# Experiments

All experiments for all sequences based on the common conditions, JCTVC-B300 have been conducted.

## Result

As objective quality performance measure, BD-Rates are computed the basis of fitting 4 test points with the three order polynomial. The results are summarized in the following tables.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -2.4 | -3.2 | -0.3 | -8.1 | -8.7 |
| Class B | -0.1 | -5.0 | -6.7 | -0.3 | -12.7 | -14.9 |
| Class C | -0.2 | -5.8 | -5.6 | -0.5 | -12.6 | -12.3 |
| Class D | -0.3 | -7.3 | -9.2 | -0.5 | -15.0 | -17.6 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -5.5 | -6.6 | -0.4 | -12.7 | -14.1 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

Table 1: Results for random access

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -1.8 | -2.0 | -1.3 | -13.5 | -16.5 |
| Class C | -0.5 | -3.5 | -3.7 | -1.6 | -13.3 | -12.4 |
| Class D | -0.9 | -3.2 | -4.5 | -1.9 | -14.6 | -16.8 |
| Class E | -0.1 | -1.3 | -1.3 | -0.5 | -6.5 | -15.1 |
| All | -0.5 | -2.5 | -2.9 | -1.4 | -12.4 | -15.3 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |

Table 2: Results for low delay

# Conclusion

In this contribution, the cross check results of DCT-IF chroma is reported. The results reported in C201 and C248 were confirmed.

# References

[1] F. Bossen, " Common test conditions and software reference configurations,” JCT-VC document JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2 nd Meeting: Geneva, CH, 21-28 July, 2010

[2] F. Bossen, et al. "AHG report: Software development and TMuC software technical evaluation” JCT-VC document JCTVC-B003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2 nd Meeting: Geneva, CH, 21-28 July, 2010

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).