| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C311 WG11 Number: m18569 |
| --- | --- |

| Title: | Fast intra encoding for fixed maximum depth of transform quadtree |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | B. Bross H. Kirchhoffer H. Schwarz T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: |  |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document describes an encoder-side complexity reduction for intra encoding. The presented fast encoding method produces bitstreams that are conforming to the specification of [1] while achieving a similar encoder complexity as for the transform quadtree disabled case.

# Fast Intra Encoding

Two fast modes have been implemented:

In speedup mode 1, the different intra prediction modes are tested only on the maximum allowed transform block size. The best performing mode is tested with the full transform tree partitioning and the best performing partitioning is chosen.

Speedup mode 2 does basically the same except testing the full transform tree partitioning with the two best performing intra modes instead of the best one.

# Implementation in TMuC 0.8.1

The following macros are added in TypeDef.h to enable the two modes:

#define HHI_RQT_INTRA_SPEEDUP		0 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD 	0 // tests two best modes with full rqt

Other changes are made in TEncSearch.h and TEncSearch.cpp und all changes are put under the two above listed macros. The two modified files are attached as JCTVC-C311-TEncSearch.h and JCTVC-C311-TEncSearch.cpp.

# Results

The results being reported are generated for the intra high efficiency case using 50 frames. Detailed results can be found in JCTVC-C311-NoIntraSpeedUp.xls, JCTVC-C311-Depth2_NoIntraSpeedUp.xls, JCTVC-C311-IntraSpeedUp1.xls, JCTVC-C311-Depth2_IntraSpeedUp1.xls, JCTVC-C311-IntraSpeedUp2.xls, JCTVC-C311-Depth2_IntraSpeedUp2.xls. TMuC 0.8.1 software and the following macro settings have been used as reference:

No RQT reference:

#define V0_7_COMPAT 		1

QuadtreeTUFlag : 0

RQT no speedup:

#define V0_7_COMPAT 			1

#define HHI_RQT_DEPTH         		1

#define HHI_RQT_INTRA_SPEEDUP		0 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD	0 // tests two best modes with full rqt

QuadtreeTUFlag	: 1

QuadtreeTUMaxDepth	: 3

|  | Intra HE 50fr |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -1.1 | 1.3 |
| Class B | -0.9 | 0.1 | 0.8 |
| Class C | -1.2 | -0.3 | 0.1 |
| Class D | -1.0 | -0.1 | 0.2 |
| Class E | -1.5 | 0.6 | 0.3 |
| All | -1.1 | -0.1 | 0.5 |
| Enc Time[%] | 206% |  |  |
| Dec Time[%] | 102% |  |  |

QuadtreeTUMaxDepth	: 2

|  | Intra HE 50fr |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.7 | 1.7 |
| Class B | -0.7 | 0.3 | 1.0 |
| Class C | -0.9 | 0.0 | 0.4 |
| Class D | -0.8 | 0.1 | 0.4 |
| Class E | -1.1 | 1.1 | 0.9 |
| All | -0.8 | 0.2 | 0.8 |
| Enc Time[%] | 175% |  |  |
| Dec Time[%] | 124% |  |  |

The speedup contains the following changes to TMuC 0.8.1:

Speedup mode 1:

#define V0_7_COMPAT 			1

#define HHI_RQT_DEPTH         		1

#define HHI_RQT_INTRA_SPEEDUP		1 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD	0 // tests two best modes with full rqt

QuadtreeTUFlag	: 1

QuadtreeTUMaxDepth	: 3

|  | Intra HE 50fr mode1 |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | 2.2 |
| Class B | -0.2 | 0.7 | 1.4 |
| Class C | -0.3 | 0.6 | 1.0 |
| Class D | -0.2 | 0.8 | 1.0 |
| Class E | -0.4 | 1.7 | 1.4 |
| All | -0.3 | 0.8 | 1.3 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |

QuadtreeTUMaxDepth	: 2

|  | Intra HE 50fr mode1 |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 2.5 |
| Class B | -0.1 | 0.9 | 1.6 |
| Class C | -0.1 | 0.9 | 1.2 |
| Class D | -0.1 | 1.0 | 1.1 |
| Class E | -0.3 | 2.2 | 1.8 |
| All | -0.1 | 1.0 | 1.5 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 109% |  |  |

Speedup mode 2:

#define V0_7_COMPAT 			1

#define HHI_RQT_DEPTH         		1

#define HHI_RQT_INTRA_SPEEDUP		1 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD	1 // tests two best modes with full rqt

QuadtreeTUFlag	: 1

QuadtreeTUMaxDepth	: 3

|  | Intra HE 50fr mode2 |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.4 | 1.9 |
| Class B | -0.4 | 0.5 | 1.2 |
| Class C | -0.6 | 0.4 | 0.8 |
| Class D | -0.4 | 0.7 | 0.9 |
| Class E | -0.7 | 1.4 | 1.1 |
| All | -0.5 | 0.6 | 1.1 |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 102% |  |  |

QuadtreeTUMaxDepth	: 2

|  | Intra HE 50fr mode2 |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.1 | 2.3 |
| Class B | -0.3 | 0.7 | 1.5 |
| Class C | -0.3 | 0.7 | 1.1 |
| Class D | -0.3 | 0.9 | 1.2 |
| Class E | -0.4 | 1.9 | 1.6 |
| All | -0.3 | 0.9 | 1.4 |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 112% |  |  |

## Disabling MDDT, ROT, edge based prediction and intra smoothing (IS)

The results being reported are generated for the intra high efficiency case using 50 frames. Detailed results can be found in JCTVC-C311-noIntraTools_NoIntraSpeedUp.xls, JCTVC-C311- noIntraTools_IntraSpeedUp1.xls and JCTVC-C311- noIntraTools_IntraSpeedUp2.xls. Intra smoothing, edge-based intra prediction, MDDT and ROT are disabled. To do so, TMuC 0.8.1 software and the following macro settings have been used:

#define V0_7_COMPAT 		1

#define DEFAULT_IS			0

#define QC_MDDT			0

//#define EDGE_BASED_PREDICTION   // Enable edge based prediction for intra

AIS					: 0

ROT				: 0

No RQT reference:

QuadtreeTUFlag	: 0

RQT no speedup:

#define HHI_RQT_DEPTH         		1

#define HHI_RQT_INTRA_SPEEDUP		0 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD	0 // tests two best modes with full rqt

QuadtreeTUFlag	: 1

QuadtreeTUMaxDepth	: 3

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.0 | 0.7 | 0.1 | 1.4 | 3.9 |
| Class B | -1.0 | 0.2 | 0.8 | -0.3 | 1.0 | 1.9 |
| Class C | -1.2 | -0.3 | 0.0 | -1.4 | 0.4 | 0.5 |
| Class D | -1.1 | 0.1 | 0.2 | -1.5 | 0.2 | 0.3 |
| Class E | -1.4 | 0.9 | 0.5 | 0.4 | 9.3 | 6.4 |
| All | -1.1 | 0.1 | 0.4 | -0.7 | 2.1 | 2.2 |
| Enc Time[%] | 158% |  |  | 182% |  |  |
| Dec Time[%] | 103% |  |  | 107% |  |  |

Speedup mode 1:

#define V0_7_COMPAT 			1

#define HHI_RQT_DEPTH         		1

#define HHI_RQT_INTRA_SPEEDUP		1 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD	0 // tests two best modes with full rqt

QuadtreeTUFlag	: 1

QuadtreeTUMaxDepth	: 3

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.2 | 1.5 | 0.9 | 2.0 | 4.4 |
| Class B | -0.5 | 0.9 | 1.4 | 0.5 | 1.4 | 2.3 |
| Class C | -0.5 | 0.4 | 0.7 | -0.6 | 0.9 | 1.1 |
| Class D | -0.5 | 0.6 | 1.0 | -0.7 | 0.7 | 1.0 |
| Class E | -0.6 | 1.5 | 1.1 | 1.2 | 9.4 | 6.6 |
| All | -0.5 | 0.7 | 1.1 | 0.1 | 2.6 | 2.7 |
| Enc Time[%] | 96% |  |  | 111% |  |  |
| Dec Time[%] | 99% |  |  | 106% |  |  |

Speedup mode 2:

#define V0_7_COMPAT 			1

#define HHI_RQT_DEPTH         		1

#define HHI_RQT_INTRA_SPEEDUP		1 // tests one best mode with full rqt

#define HHI_RQT_INTRA_SPEEDUP_MOD	1 // tests two best modes with full rqt

QuadtreeTUFlag	: 1

QuadtreeTUMaxDepth	: 3

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.5 | 1.2 | 0.6 | 1.8 | 4.3 |
| Class B | -0.7 | 0.7 | 1.2 | 0.1 | 1.3 | 2.2 |
| Class C | -0.8 | 0.2 | 0.4 | -1.0 | 0.7 | 0.9 |
| Class D | -0.7 | 0.4 | 0.7 | -1.1 | 0.5 | 0.7 |
| Class E | -1.0 | 1.1 | 0.9 | 0.8 | 9.5 | 6.6 |
| All | -0.8 | 0.4 | 0.8 | -0.2 | 2.4 | 2.5 |
| Enc Time[%] | 116% |  |  | 132% |  |  |
| Dec Time[%] | 100% |  |  | 107% |  |  |

# Conclusion

It can be concluded that for the proposed transform quadtree depth of 3 the intra encoding complexity can be reduced to achieve similar complexity as without quadtree and still having coding gains for intra coding.

# References

- JCT-VC, “TE9: Simulation results for various max. number of transform quadtree dept”, JCTVC-C198, JCT-VC Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).