| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C315 WG11 Number: m18581 |
| --- | --- |

| Title: | On the computation of PSNR and BD-Rate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dzung Hoang | Tel: Email: | dzung.hoang@zenverge.com |
| Source: | Zenverge, Inc. |  |  |

_____________________________

# Abstract

The computations of PSNR and BD-Rate by the current TMuC software and spreadsheet macros are reviewed.  Flaws in the computations are identified and recommendations are made on how to improve these computations.

- Problem Statement

## Computation of PSNR

The current TMuC software computes the sequence-level PSNR as the average of the frame-level PSNR in the following way:

PSNRseq=iPSNRframe(i)N	()

This is an abuse of the PSNR concept because PSNR is clearly originally intended as a logarithmic scaling of the ratio between the peak signal and the MSE. The definition of PSNR for 8-bit unsigned data is as follows:

PSNR=10log2552MSE	()

where MSE is the well-known mean square error. As applied to a frame of video, PSNR is computed from the MSE of the pixels that comprise the frame. When applied to a video sequence, the PSNR should be computed from the MSE of the pixels that comprise the video sequence. This is not what is computed in Equation 1. What is computed by Equation 1 can be interpreted as the logarithmic scaling of the geometric mean of the frame-level MSE. If this is indeed the intent agreed to by the committee, it should be explained why this is desired as opposed to Equation 2.

## Computation of BD-Rate

Many input documents compute BD-Rate for the Y, Cr, and Cb components using the same sequence bit rate. Ideally, the BD-Rate for each component should be computed using the component-specific bit rate. A fault with this approach is that any change in the coding efficiency of just one component will affect the BD-Rate of the other components. If a change affects more than one component, using the sequence bit rate introduces even further confusion in the causal relationship as indicated by the BD-Rate values.

# Proposal

## Computation of PSNR

It is proposed to change the computation of PSNR to align with Equation 2 in both TMuC and the to-be-defined Test Model. This proposal is in line with another proposal submitted for consideration by ITU-T in [1].

## Computation of BD-Rate

For the BD-Rate computation, one approach is to measure the bit rate separately by component. A difficulty with the approach is how to split the bit rate for coded information common to the components, such as motion vectors and other side information. In acknowledgement of this difficulty, we propose not to change the computation of BD-Rate for Y. For Cr and Cb, we propose to measure the bit rate for coding the transform coefficients for Cr and Cb separately and to use these in the computation of BD-Rate for Cr and Cb. Since BD-Rate for Y already accounts for the coding of the common information, omitting these from the rate measurements for Cr and Cb should not be a concern. In this way, the BD-Rate for Cr and Cb can be aligned more closely to actual coding efficiency changes for Cr and Cb. There may be an issue if the coding rate for Cr and Cb are measured to be zero, but this can be addressed by adding a small offset to prevent a zero rate value, for example.

# References

[1]	Stephen Wolf and Margaret Pinson, “Reference Algorithm for Computing Peak Signal to Noise Ratio (PSNR) of a Video Sequence with a Constant Delay,” Available URL: http://www.its.bldrdoc.gov/pub/n3/video/itut_com9_c6.pdf

# Patent rights declaration(s)

There is no patent rights declaration.