| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C318 WG11 Number: m18586 |
| --- | --- |

| Title: | TMuC text on max CU size seems broken |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden | Tel: Email: | rickard sjoberg @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

There are several issues with the syntax and semantics that specifies the largest and smallest coding unit sizes in the current TMuC specification. Therefore this document proposes to revert that part of the specification to the text of JCTVC-B205_draft000.doc.

- Problem statement

We identified the following problems with the current TMuC specification [1]:

- There are no rectangular CUs in the TMuC, only square CUs. The individual signaling of width and height is not necessary

- max_coding_unit_width and max_coding_unit_height are unconstrained

- The while loop design is unnecessarily complicated

- The while loop does nothing and could result in an infinite loop

- There are no restrictions on MaxCodingUnitHierarchyDepth

Here is the current TMuC specification for signaling the maximum and minimum CU and TU sizes:

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| max_coding_unit_width | 0 | ue(v) |
| max_coding_unit_height | 0 | ue(v) |
| max_coding_unit_hierarchy_depth_correct | 0 | u(1) |
| log2_min_transform_unit_size_minus2 | 0 | ue(v) |
| if( log2_min_transform_unit_size_minus2 < 4 ) |  |  |
| max_transform_unit_hierarchy_depth | 0 | ue(v) |

max_coding_unit_width specifies the maximum width of a coding unit

max_coding_unit_height specifies the maximum height of a coding unit

max_coding_unit_hierarchy_depth_correct equal to 0 specifies that the coding unit hierarchy depth is not incremented. max_coding_unit_hierarchy_depth_correct equal to 1 specifies that the coding unit hierarchy depth is incremented by 1.

The variable MaxCodingUnitDepth is derived as

AddCUDepth = 0
        While  ( max_coding_unit_width >> max_coding_unit_hierarchy_depth_correct ) >
                    ( 1 << ( log2_min_transform_unit_size_minus2 ) + AddCUDepth ) ) ) {
              AddCuDepth
          }
          MaxCodingUnitHierarchyDepth = max_coding_unit_hierarchy_depth_correct + AddCUDepth

log2_min_transform_unit_size_minus2 specifies the minimum size of a transform unit.

The variable MinTransformUnitSize is derived as

MinTransformUnitSize = 1 << ( log2_min_transform_unit_size_minus2 + 2 )

max_transform_unit_hierarchy_depth specifies the maximum hierarchy depth for transform units. If not parsed, this element is set to 0.

The variable MaxTransformUnitSize is derived as

MaxTransformUnitSize = 1 << ( log2_min_transform_unit_size_minus2 + 2 +
                           max_transform_unit_hierarchy_depth )	(1)

The bitstream shall not contain data that result in a value of MaxTransformUnitSize that is greater than MaxCodingUnitSize.

# Proposal

We propose to revert the text describing the maximum and minimum CU sizes to the description in JCTVC-B205_draft000.doc and change the TM software accordingly. We believe this solves all the listed issues regarding the current TMuC text. The draft000 syntax is explicit and the CUs are constrained between 8x8 and 128x128. The TUs are constrained between 4x4 and the max CU size. The maximum TU should be changed according to the decisions made at the 3rd JCTVC meeting, possibly 64 or 32.

Here is the draft000 text:

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| log2_min_coding_unit_size_minus3 | 0 | ue(v) |
| max_coding_unit_hierarchy_depth | 0 | ue(v) |
| log2_min_transform_unit_size_minus2 | 0 | ue(v) |
| max_transform_unit_hierarchy_depth | 0 | ue(v) |

log2_min_coding_unit_size_minus3 specifies the minimum size of a coding unit.

The variable MinCodingUnitSize is derived as

MinCodingUnitSize = 1 << ( log2_min_coding_unit_size_minus3 + 3 )

max_coding_unit_hierarchy_depth specifies the maximum hierarchy depth for coding units.

The variable MaxCodingUnitSize is derived as

MaxCodingUnitSize = 1 << ( log2_min_coding_unit_size_minus3 + 3 +
                                                        max_coding_unit_hierarchy_depth )

The bitstream shall not contain data that result in a value of MaxCodingUnitSize that is greater than 128.

log2_min_transform_unit_size_minus2 specifies the minimum size of a transform unit.

The variable MinTransformUnitSize is derived as

MinTransformUnitSize = 1 << ( log2_min_transform_unit_size_minus2 + 2 )

max_transform_unit_hierarchy_depth specifies the maximum hierarchy depth for transform units.

The variable MaxTransformUnitSize is derived as

MaxTransformUnitSize = 1 << ( log2_min_transform_unit_size_minus2 + 2 +
                                                     max_transform_unit_hierarchy_depth )

The bitstream shall not contain data that result in a value of MaxTransformUnitSize that is greater than MaxCodingUnitSize.

- References

- "Test Model under Consideration", JCTVC-B205_draft007, Geneva, Switzerland, July 2010.