| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C319 WG11 Number: m18590 |
| --- | --- |

| Title: | BoG report: residual quadtree structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Wiegand H. Schwarz B. Bross (Fraunhofer HHI) A. Fuldseth (Cisco) X. Wang (Qualcomm) W.-J. Han (Samsung) | Email: | thomas.wiegand@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de arilfuld@cisco.com xianglin@qualcomm.com wjhan.han@samsung.com |
| Source: | Break-out group on residual quadtree structure |  |  |

_____________________________

# Abstract

This document contains the report of the break-out group on residual quadtree structure. The group intended to clarify that, for the low-complexity case, the HHI residual quadtree can be configured to be identical with the Samsung 2-level transform unit structure. Therefore, there is no need to keep two separate branches in the TM software.

# Residual quadtree structure (informative)

The residual quadtree is a recursive representation of the partitioning of a coding unit into transform units. There is only one residual quadtree for both luma and chroma components of each coding unit. The transform unit block that corresponds to the root node of the residual quadtree is identical to the given coding unit block. The possible splittings of a coding unit are described for both low-complexity and high-efficiency cases as follows.

## Low-complexity case

In the low-complexity case, there are two possible partitionings of the coding unit. The corresponding residual quadtree is restricted to 2 levels. The partitioning depends on the value of the syntax element split_transform_flag (or its inferred values) as follows.

–	If split_transform_flag is equal to 1, the coding unit is split into four quadratic transform units, each having half the horizontal and vertical size of the coding unit.

–	Otherwise (split_transform_flag is equal to 0), the coding unit is not split, resulting in a transform unit that has the same size as the coding unit.

For intra-coded coding units, split_transform_flag is not present in the bitstream, but inferred to be equal to intra_split_flag. The syntax element intra_split_flag is signaled in the prediction unit syntax and is available before parsing the syntax elements related to transform coding.

For inter-coded coding units, split_transform_flag is present in the bitstream, unless the supported physical transform sizes (as signaled in the sequence parameter set) do not allow a splitting of the coding unit (in which case split_transform_flag is inferred to be equal to 0) or do not allow the coding of the coding unit as a single transform unit (in which case split_transform_flag is inferred to be equal to 1).

A splitting level for which split_transform_flag is inferred to be equal to 1 because the coding unit size is greater than the maximum supported transform size is not counted for the hierarchy depth. In this case, the split_transform_flag is present for all blocks of the maximum supported transform size.

## High-efficiency case

In the high-efficiency case, the partitioning of a coding unit into transform units is restricted to 3 levels of the residual quadtree. The partitioning is done recursively depending on the value of split_transform_flag. Initially the transform unit size is set equal to the coding unit size. Then, the following partitioning is done recursively:

–	If split_transform_flag is equal to 1, the current transform unit is split into four quadratic transform units, each having half the horizontal and vertical size of the current transform unit. When the obtained transform unit sizes are larger than the minimum supported transform size and the maximum hierarchy level is not yet achieved, for each of the four quadratic transform units a split_transform_flag is signaled.

–	Otherwise (split_transform_flag is equal to 0), the current transform unit is not split further.

For intra-coded coding units, split_transform_flag for the first level is inferred in the same way as described in 1.1 for the low-complexity case. This splitting, which is signaled using intra_split_flag, is not counted for the transform hierarchy depth. When split_transform_flag is equal to 1, the split_transform_flag for the next level is present in the bitstream, unless the supported physical transform sizes (as signaled in the sequence parameter set) do not allow a splitting of the coding unit (in which case split_transform_flag is inferred to be equal to 0) or do not allow the coding of the coding unit as a single transform unit (in which case split_transform_flag is inferred to be equal to 1) or the maximum hierarchy depth has been achieved (in which case split_transform_flag is inferred to be equal to 0).

For inter-coded coding units, split_transform_flag is generally parsed in a recursive way for both levels. It is not present when the supported physical transform sizes (as signaled in the sequence parameter set) do not allow a splitting of the coding unit (in which case split_transform_flag is inferred to be equal to 0) or do not allow the coding of the coding unit as a single transform unit (in which case split_transform_flag is inferred to be equal to 1) or the maximum hierarchy depth has been achieved (in which case split_transform_flag is inferred to be equal to 0).

Splitting levels for which split_transform_flag is inferred to be equal to 1 (intra_split_flag is equal to 1 or the block size is greater than the maximum supported transform size) are not counted for the hierarchy depth.

- Syntax and Semantics

## Syntax

### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| ... |  |  |
| log2_min_coding_unit_size_minus3 | 0 | ue(v) |
| log2_diff_max_min_coding_unit_size | 0 | ue(v) |
| ... |  |  |
| log2_min_transform_size_minus2 | 0 | ue(v) |
| log2_diff_max_min_transform_size | 0 | ue(v) |
| max_transform_hierarchy_depth_inter | 0 | ue(v) |
| max_transform_hierarchy_depth_intra | 0 | ue(v) |
| ... |  |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

[Ed. Note(HS): The SPS syntax elements are a suggestion and might be made less general (I tried to suggest a general concept, which can be made more special in an easy way). In particular, the following aspects could be considered:

–	log2_min_transform_size_minus2, which specifies the minimum supported transform size, could be always set equal to 0 (resulting in a minimum transform size of 4x4), and doesn't need to be signaled in the SPS in this case. I don't see a particular advantage of allowing minimum transform sizes larger than 4x4, but included it to be more general. Note that this parameter doesn't specify the maximum transform depth for a particular coding unit, but the size of the minimum supported transform;

–	log2_min_transform_size_minus2 +  log2_diff_max_min_transform_size, which specifies the maximum supported transform size, could be always set to a fixed value (e.g., equal to 6 specifying a maximum transform size of 64x64, which seems to be the maximum that is currently supported in TMuC), and doesn't need to be signaled in the SPS in this case. I believe there is value of sending such a parameter in the SPS, in order to restrict the complexity for certain profiles and/or levels;

–	max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra could be declared as variables that are derived based on profile_idc. However, I believe it is conceptually clearer to include these syntax elements and specify constraints based on profile_idc. Furthermore, a low-complexity encoder may not evaluate the full transform depth (as allowed by the corresponding profile) and hence wants to avoid the sending of splitting flags that are always equal to 0;

–	the hierarchy depth could be specified with one syntax element for inter and intra. However, encoders might want to test different depth for intra and inter and avoid the signaling of splitting flags that are always equal to 0.

]

### Slice data syntax

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| ... |  |  |
| CurrMbAddr = first_lctb_in_slice |  |  |
| moreDataFlag = 1 |  |  |
| do { |  |  |
| xCU = HorLumaLocation( CurrMbAddr ) |  |  |
| yCU = VerLumaLocation( CurrMbAddr ) |  |  |
| coding_unit( xCU, yCU, Log2MaxCUSize ) | 2 ¦ 3 ¦ 4 |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | C | Descriptor |
| --- | --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && log2CUSize > Log2MinCUSize ) |  |  |
| split_coding_unit_flag | 2 | u(1) ¦ ae(v) |
| if( split_coding_unit_flag ) { |  |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |  |
| coding_unit( x0, y0, log2CUSize − 1 ) | 2 ¦ 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL ) |  |  |
| coding_unit( x1, y0, log2CUSize − 1 ) | 2 ¦ 3 ¦ 4 |  |
| if( y1 < PicHeightInSamplesL ) { |  |  |
| coding_unit( x0, y1, log2CUSize − 1 ) | 2 ¦ 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL ) |  |  |
| coding_unit( x1, y1, log2CUSize − 1 ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| } else { |  |  |
| prediction_unit( x0, y0, log2CUSize ) | 2 |  |
| if( PredMode != MODE_SKIP ) { |  |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) | 3 ¦ 4 |  |
| if( !no_residual_data_flag ) { |  |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) /* luma */ | 3 ¦ 4 |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) /* chroma Cb */ | 3 ¦ 4 |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) /* chroma Cr */ | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Transform tree syntax

| transform_tree( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | C | Descriptor |
| --- | --- | --- |
| if(trafoDepth = = 0 && intra_split_flag == 0){ |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| cbp_yuv_root |  | vlc(n,v) |
| cbf_luma[ x0 ][ y0 ] [trafoDepth ] = cbp_yuv_root & 1 |  |  |
| cbf_cb[x0][y0] [trafoDepth ] = (cbp_yuv_root >> 1) & 1 |  |  |
| cbf_cr[x0][y0] [trafoDepth ] = (cbp_yuv_root >> 2) & 1 |  |  |
| residualDataPresentFlag = (cbp_yuv_root != 0) |  |  |
| } |  |  |
| else { |  |  |
| if( PredMode != MODE_INTRA ) |  |  |
| no_residual_data_flag | 3 ¦ 4 | u(1) ¦ ae(v) |
| residualDataPresentFlag = !no_residual_data_flagb |  |  |
| } |  |  |
| } else { |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| if( blkIdx = = 0) { |  |  |
| cbp_luma[ trafoDepth ] |  | vlc(n,v) |
| cbp_cb[ trafoDepth ] |  | vlc(n,v) |
| cbp_cr[ trafoDepth ] |  | vlc(n,v) |
| } |  |  |
| cbf_luma[ x0 ][ y0 ] [trafoDepth] = (cbp_luma[ trafoDepth]) >> blkIdx ) & 1 |  |  |
| cbf_cb[ x0 ][ y0 ] [trafoDepth] = (cbp_cb[ trafoDepth ]) >> blkIdx ) & 1 |  |  |
| cbf_cf[ x0 ][ y0 ] [trafoDepth] = (cbp_cf[ trafoDepth]) >> blkIdx ) & 1 |  |  |
| residualDataPresentFlag = (cbp_yuv_root != 0) |  |  |
| } |  |  |
| residualDataPresentFlag = true |  |  |
| } |  |  |
| if( residualDataPresentFlag) { |  |  |
| intraSplitFlag = ( intra_split_flag && trafoDepth = = 0 ? 1 : 0 ) |  |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + intra_split_flag : max_transform_hierarchy_depth_inter ) |  |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag ) |  |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |  |
| xBase = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |  |
| yBase = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |  |
| if( firstChromaCbf ¦ ¦ coded_block_flag_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |  |
| coded_block_flag_cb[ x0 ][ x1 ][ trafoDepth ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| if( firstChromaCbf ¦ ¦ coded_block_flag_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |  |
| coded_block_flag_cr[ x0 ][ x1 ][ trafoDepth ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |  |
| if ( !entropy_coding_mode_flag) { |  |  |
| cbp_luma[ trafoDepth+1 ] |  |  |
| cbp_cb[ trafoDepth+1 ] |  |  |
| cbp_cr[ trafoDepth+1 ] |  |  |
| } | 3 ¦ 4 |  |
| transform_tree( x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) | 3 ¦ 4 |  |
| transform_tree( x1, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) | 3 ¦ 4 |  |
| transform_tree( x0, y1, log2TrafoSize − 1, trafoDepth + 1, 2 ) | 3 ¦ 4 |  |
| transform_tree( x1, y1, log2TrafoSize − 1, trafoDepth + 1, 3 ) | 3 ¦ 4 |  |
| } else if(entropy_coding_mode_flag ){ |  |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |  |
| coded_block_flag_luma[ x0 ][ y0 ][ trafoDepth ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| if( PredMode = = MODE_INTRA ) |  |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |  |
| coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| } else if( blkIdx = = 0 ) { |  |  |
| coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | 3 ¦ 4 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | C | Descriptor |
| --- | --- | --- |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) | 3 ¦ 4 |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) | 3 ¦ 4 |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) | 3 ¦ 4 |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) | 3 ¦ 4 |  |
| } else { |  |  |
| nMax = ( 1 << ( log2TrafoSize << 1 ) ) >> ( cIdx > 0 ? 2 : 0 ) |  |  |
| for( n = 0; n < nMax; n++ ) |  |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ] = 0 |  |  |
| if (entropy_coding_mode_flag && ( ( cIdx= =0 && coded_block_flag_luma[ x0 ][ y0 ][ trafoDepth ] )¦ ¦ ( cIdx= =1 && coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx= =2 && coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) |  |  |
| residual_coding_pipe( x0, y0, log2TrafoSize, trafoDepth, cIdx ) | 3 ¦ 4 |  |
| else if (!entropy_coding_mode_flag && ( ( cIdx= =0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx= =1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx= =2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) |  |  |
| residual_coding_lcec( x0, y0, log2TrafoSize, trafoDepth, cIdx ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |

## Semantics

### Sequence parameter set RBSP semantics

profile_idc indicates the profile to which the coded video sequence conforms.

...

log2_min_coding_unit_size_minus3 specifies the minimum size of a coding unit.

The variable Log2MinCUSize is set equal to log2_min_coding_unit_size_minus3 + 3.

log2_diff_max_min_coding_unit_size specifies  the difference between the maximum and minimum coding unit size.

The variable Log2MaxCUSize is set equal to log2_min_coding_unit_size_minus3 + 3 + log2_diff_max_min_coding_unit_size.

The bitstream shall not contain data that result in Log2MaxCUSize greater than 7.

...

log2_min_transform_size_minus2 specifies the minimum transform size.

The variable Log2MinTrafoSize is set equal to log2_min_transform_size_minus 2 + 2.

The bitstream shall not contain data that result in Log2MinTrafoSize greater than or equal to Log2MinCUSize.

log2_diff_max_min_transform_size specifies  the difference between the maximum and minimum transform size.

The variable Log2MaxTrafoSize is set equal to log2_min_transform_size_minus 2 + 2 + log2_diff_max_min_transform_size.

The bitstream shall not contain data that result in Log2MaxTrafoSize greater than Log2MaxCUSize.

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform units of coding units coded in intra prediction mode.

Depending on profile_idc, the following applies:

–	If profile_idc is equal to "low-complexity profile", the value of max_transform_hierarchy_depth_intra shall be equal to 0.

–	Otherwise (profile_idc is not equal to "low-complexity profile"), the value of max_transform_hierarchy_depth_intra shall not be greater than 2.

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode.

Depending on profile_idc, the following applies:

–	If profile_idc is equal to "low-complexity profile", the value of max_transform_hierarchy_depth_inter shall not be greater than 1.

–	Otherwise (profile_idc is not equal to "low-complexity profile"), the value of max_transform_hierarchy_depth_inter shall not be greater than 2.

### Slice data semantics

...

end_of_slice_flag equal to 0 specifies that another macroblock is following in the slice. end_of_slice_flag equal to 1 specifies the end of the slice and that no further macroblock follows.

### Coding unit semantics

split_coding_unit_flag specifies whether a coding unit is split into coding units with half horizontal and vertical size.

When split_coding_unit_flag is not present, its value is inferred as specified in the following:

–	If log2CUSize is greater than Log2MinCUSize, the value of split_coding_unit_flag is inferred to be equal to 1.

–	Otherwise (log2CUSize is equal to Log2MinCUSize), the value of split_coding_unit_flag is inferred to be equal to 0.

### Transform tree semantics

no_residual_data_flag equal to 1 specifies that no residual data are present for the current coding unit. no_residual_data_flag equal to 0 specifies that residual data are present for the current coding unit.

When no_residual_data_flag is not present, its value shall be inferred to be equal to 0.

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into blocks with half horizontal and vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

–	If log2TrafoSize is greater than Log2MaxTrafoSize or intraSplitFlag is equal to 1, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

–	Otherwise (log2TrafoSize is less than or equal to Log2MaxTrafoSize and intraSplitFlag is equal to 0), the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

coded_block_flag_luma[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] is not present and PredMode is not equal to MODE_INTRA, the value of coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] is not present and PredMode is not equal to MODE_INTRA, the value of coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

cbp_yuv_root equal to 1 specifies that the coding unit contains at least one transform block with one or more transform coefficients not equal to 0.

cbp_luma[ trafoDepth] equal to 1 specifies that one of the four luma transform blocks at the subdivision level trafoDepth contains one or more transform coefficient levels not equal to 0. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

cbp_cb[ trafoDepth] equal to 1 specifies that one of the four cb transform blocks at the subdivision level trafoDepth contains one or more transform coefficient levels not equal to 0. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

cbp_cr[ trafoDepth] equal to 1 specifies that one of the four cr transform blocks at the subdivision level trafoDepth contains one or more transform coefficient levels not equal to 0. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

### Transform coefficient semantics

The transform coefficient levels are parsed into the arrays transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array index n specifies the scanning position of the transform coefficient levels.

- Decoding process

## Decoding process for coding units coded in intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is:

–	a modified reconstructed picture before deblocking filtering.

Depending on intra_split_flag, the following applies:

–	If intra_split_flag is equal to 1, the following ordered steps apply:

- The derivation process for the luma intra prediction mode is invoked with the luma location ( xB, yB ) as the input and the output is the variable lumaPredMode.

- The decoding process for intra luma blocks as specified in subclause 3.1.1 is invoked with the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2CUSize, the variable trafoDepth set equal to 0, and the luma intra prediction mode lumaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (intra_split_flag is equal to 0), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xBS is set equal to xB + ( (1 << log2CUSize ) >> 1 ) * ( blkIdx % 2 ).

- The variable yBS is set equal to yB + ( (1 << log2CUSize ) >> 1 ) * ( blkIdx / 2 ).

- The derivation process for the luma intra prediction mode is invoked with the luma location ( xBS, yBS ) as the input and the output is the variable lumaPredMode.

- The decoding process for intra luma blocks as specified in subclause 3.1.1 is invoked with the luma location ( xBS, yBS ), the variable log2TrafoSize set equal to log2CUSize − 1, the variable trafoDepth set equal to 1, and the luma intra prediction mode lumaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

The decoding process for intra chroma blocks as specified in subclause 3.1.2 is invoked with the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2CUSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 1, and the chroma intra prediction mode chromaPredMode set equal to intra_chroma_pred_mode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

The decoding process for intra chroma blocks as specified in subclause 3.1.2 is invoked with the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2CUSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 2, and the chroma intra prediction mode chromaPredMode set equal to intra_chroma_pred_mode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

### Decoding process for intra luma blocks

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable lumaPredMode specifying the luma intra prediction mode.

Output of this process is:

–	a modified reconstructed picture before deblocking filtering.

Depending split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variable xB1 is set equal to xB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The variable yB1 is set equal to yB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The decoding process for intra luma blocks as specified in this subclause is invoked with the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, and the luma intra prediction mode lumaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- The decoding process for intra luma blocks as specified in this subclause is invoked with the luma location ( xB1, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, and the luma intra prediction mode lumaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- The decoding process for intra luma blocks as specified in this subclause is invoked with the luma location ( xB, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, and the luma intra prediction mode lumaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- The decoding process for intra luma blocks as specified in this subclause is invoked with the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, and the luma intra prediction mode lumaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0), the following ordered steps apply:

- The variable nS is set equal to 1 << log2TrafoSize.

- The intra luma sample prediction process as specified in subclause XXX is invoked with the luma location ( xB, yB ), the luma intra prediction mode lumaPredMode, and the prediction size predSize set equal to nS as the inputs and the output is a (nS)x(nS) array predSamples.

- The scaling and transformation process as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable trafoDepth, the variable cIdx set equal to 0, and the transform size trafoSize set equal to nS as the inputs and the output is a (nS)x(nS) array resSamples.

- The residual signal accumulation process as specified in subclause XXX is invoked with the variable arraySize set equal to nS, the (nS)x(nS) array predSamples, and the (nS)x(nS) array resSamples as the inputs and the output is a (nS)x(nS) array recSamples.

- The picture reconstruction process for a component before deblocking filtering as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable arraySize set equal to nS, the variable cIdx set equal to 0, and the (nS)x(nS) array recSamples as the inputs and the output is a modified reconstructed picture before deblocking filtering.

### Decoding process for intra chroma blocks

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable chromaPredMode specifying the chroma intra prediction mode.

Output of this process is:

–	a modified reconstructed picture before deblocking filtering.

The variable splitChromaFlag is derived as follows:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1 and log2TrafoSize is greater than Log2MinTrafoSize + 1, splitChromaFlag is set equal to 1.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0 or log2TrafoSize is equal to Log2MinTrafoSize + 1), splitChromaFlag is set equal to 0.

Depending splitChromaFlag, the following applies:

–	If splitChromaFlag is equal to 1, the following ordered steps apply:

- The variable xB1 is set equal to xB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The variable yB1 is set equal to yB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The decoding process for intra chroma blocks as specified in this subclause is invoked with the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, and the chroma intra prediction mode chromaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- The decoding process for intra chroma blocks as specified in this subclause is invoked with the luma location ( xB1, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, and the chroma intra prediction mode chromaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- The decoding process for intra chroma blocks as specified in this subclause is invoked with the luma location ( xB, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, and the chroma intra prediction mode chromaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- The decoding process for intra chroma blocks as specified in this subclause is invoked with the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, and the chroma intra prediction mode chromaPredMode as the inputs and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (splitChromaFlag is equal to 0), the following ordered steps apply:

- The variable nS is set equal to ( 1 << log2TrafoSize ) >> 1.

- The intra chroma sample prediction process as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable cIdx, the chroma intra prediction mode chromaPredMode, and the prediction size predSize set equal to nS as the inputs and the output is a (nS)x(nS) array predSamples.

- The scaling and transformation process as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nS as the inputs and the output is a (nS)x(nS) array resSamples.

- The residual signal accumulation process as specified in subclause XXX is invoked with the variable arraySize set equal to nS, the (nS)x(nS) array predSamples, and the (nS)x(nS) array resSamples as the inputs and the output is a (nS)x(nS) array recSamples.

- The picture reconstruction process for a component before deblocking filtering as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable arraySize set equal to nS, the variable cIdx, and the (nS)x(nS) array recSamples as the inputs and the output is a modified reconstructed picture before deblocking filtering.

## Decoding process for the residual signal of coding units coded in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array resSamplesL of luma residual samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array resSamplesCb of chroma residual samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array resSamplesCr of chroma residual samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to 1 << log2CUSize and the variable nCSC is set equal to ( 1 << log2CUSize ) >> 1.

Let resSamplesL be a (nCSL)x(nCSL) array of luma residual samples and let resSamplesCb and resSamplesCr be two (nCSC)x(nCSC) arrays of chroma residual samples.

Depending on no_residual_data_flag, the following applies:

–	If no_residual_data_flag is equal to 1, all samples of the (nCSL)x(nCSL) array resSamplesL and all samples of the two (nCSC)x(nCSC) arrays resSamplesCb and resSamplesCr are set equal to 0.

–	Otherwise (no_residual_data_flag is equal to 0), the following ordered steps apply:

- The decoding process for luma residual blocks as specified in subclause 3.2.1 below is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CUSize, the variable trafoDepth set equal to 0, the variable nCS set equal to nCSL, and the (nCSL)x(nCSL) array resSamplesL as the inputs and the output is a modified version of the (nCSL)x(nCSL) array resSamplesL.

- The decoding process for chroma residual blocks as specified in subclause 3.2.2 below is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CUSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 1, the variable nCS set equal to nCSC, and the (nCSC)x(nCSC) array resSamplesCb as the inputs and the output is a modified version of the (nCSC)x(nCSC) array resSamplesCb.

- The decoding process for chroma residual blocks as specified in subclause 3.2.2 below is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CUSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 2, the variable nCS set equal to nCSC, and the (nCSC)x(nCSC) array resSamplesCr as the inputs and the output is a modified version of the (nCSC)x(nCSC) array resSamplesCr.

### Decoding process for luma residual blocks

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoSize specifying the size of the current block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable nCS specifying the size, in luma samples, of the current coding unit,

–	a (nCS)x(nCS) array resSamples of luma residual samples.

Output of this process is:

–	a modified version of the (nCS)x(nCS) array of luma residual samples.

Depending split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variable xB1 is set equal to xB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The variable yB1 is set equal to yB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB1, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0), the following ordered steps apply:

- The variable nS is set equal to 1 << log2TrafoSize.

- The scaling and transformation process as specified in subclause XXX is invoked with the luma location ( xC + xB, yC +yB ), the variable trafoDepth, the variable cIdx set equal to 0, and the transform size trafoSize set equal to nS as the inputs and the output is a (nS)x(nS) array resSamplesBlock.

- The array construction process as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable cIdx set equal to 0, the variable arraySize set equal to nS, the (nS)x(nS) array resSamplesBlock, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

### Decoding process for chroma residual blocks

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoSize specifying the size of the current block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nCS specifying the size, in chroma samples, of the current coding unit,

–	a (nCS)x(nCS) array resSamples of chroma residual samples.

Output of this process is:

–	a modified version of the (nCS)x(nCS) array of chroma residual samples.

The variable splitChromaFlag is derived as follows:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1 and log2TrafoSize is greater than Log2MinTrafoSize + 1, splitChromaFlag is set equal to 1.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0 or log2TrafoSize is equal to Log2MinTrafoSize + 1), splitChromaFlag is set equal to 0.

Depending splitChromaFlag, the following applies:

–	If splitChromaFlag is equal to 1, the following ordered steps apply:

- The variable xB1 is set equal to xB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The variable yB1 is set equal to yB + ( ( 1 << log2TrafoSize ) >> 1 ).

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB1, yB ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

–	Otherwise (splitChromaFlag is equal to 0), the following ordered steps apply:

- The variable nS is set equal to ( 1 << log2TrafoSize ) >> 1.

- The scaling and transformation process as specified in subclause XXX is invoked with the luma location ( xC + xB, yC +yB ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nS as the inputs and the output is a (nS)x(nS) array resSamplesBlock.

- The array construction process as specified in subclause XXX is invoked with the luma location ( xB, yB ), the variable cIdx, the variable arraySize set equal to nS, the (nS)x(nS) array resSamplesBlock, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

# Encoding process (informative)

The encoding process for intra-coded coding units can be summarized as follows.

–	The luma intra prediction mode (or modes for intra_split_flag equal to 1) is (are) determined using the residual coding with the largest applicable transform size.

–	Given the determined luma intra prediction mode (or modes for intra_split_flag equal to 1), the transform tree and the corresponding luma transform coefficient levels are determined using an exhaustive subdivision process, taking into account the maximum allowed transform hierarchy depth and considering only the luma component.

–	The chroma intra prediction mode and the corresponding chroma transform coefficient levels are determined given the determined transform tree.

The encoding process for inter-coded coding units can be summarized as follows.

–	The transform tree and the corresponding luma and chroma transform coefficient levels are determined using an exhaustive subdivision process, taking into account the maximum allowed transform hierarchy depth and considering both the luma component and the chroma components.