| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C320 WG11 Number: m18597 |
| --- | --- |

| Title: | TE12: Summary of results of evaluation of TMuC tools in TE12 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ken McCann (Samsung/ZetaCast) | Email: | ken@zetacast.com |
| Source: | TE12 Coordinator |  |  |

_____________________________

# Abstract

This report is a summary overview of the results of tool experiment 12 (TE12), on exploring the performance of individual tools in the Test Model under Consideration (TMuC) [1].  Detailed results are reported by the TE12 participants in individual input documents.

- Introduction

The starting points used in TE12 were the default configurations of the TMuC specified in JCTVC-B300[2], since these represent the current best estimate of realistic use cases.

Each tool in the TMuC was tested individually as follows in TE12 [3].

- Some tools are on/off in nature.  This type of tool was tested by being turned on (if off in the default configuration) or off (if on in the default configuration).

- Some tools have direct alternatives either within the TMuC itself or else added to the software for comparison purposes.  This type of tool was tested by being replaced by direct alternatives.

- Some tools can have minimum or maximum limits defined (e.g. Large Transform).  This type of tool was tested by having either the minimum or maximum limit made stricter as appropriate.

Tools for which there is no alternative in the TMuC (e.g. inter transforms) were not tested.

A summary of the tests used to evaluate the TMuC tools in TE12 is given in [4] and the detailed results are reported by the TE12 participants in individual input documents.

This document summarizes the results of the various tests, in section 2 below. In each case, the results are expressed in terms of the incremental benefit of each tool (in terms of BD-rate reduction) and the incremental cost (in terms of effect on the encoder and decoder runtime).  However, in several cases the actual test was performed by disabling a tool, so some processing of the data has been performed.

It should be noted that the various tools in TMuC interact in a complex non-linear manner, so that careful interpretation of the results is required.

- Tests Performed

The following format was used for all numbers provided in the tables below:

BD-rate luma reduction (%) Relative encoding time Relative decoding time

BD-rate luma reduction indicates the bit-rate reduction when the tested coding tool is used. Positive number means that the tested coding tool reduces the bit-rate compared to the bit-rate without the tested coding tool and vice versa. Please note that there is a non-trivial margin of error on the encoder and decoder runtime in many reported results, particularly those using a cluster.

  - Unit Definitions

The results of the tests of unit definitions are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| 64x64 max CU size (relative to 32x32) | 0.4 141% 43% [a] | 0.5 | 2.7 132% 38% [a] | 7.0 130% | 2.6 133% 38% [a] | 8.8 | 3.7 134% 40% |
| Quadtree transform (relative to 2-level) | 1.2 256% 103% | 0.6 323% 104% | 0.5 98% 91% | -0.2 102% 96% | 0.2 109% 101% | 0.8 108% 97% | 0.5 166% 99% |
| 64x64 max TU (relative to 32x32) | 0.2 99% 98% | 0.1 104% 108% | 0.3 94% 92% | 0.2 102% 102% | 0.5 96% 98% | 0.6 102% 106% | 0.3 100% 101% |
| Asymmetric PU |  |  | 1.3 154% 100% | 1.8 152% 100% | 1.6 152% 100% | 2.1 152% 100% | 1.7 153% 100% |
| CU-based Block Merging (MRG) |  |  | -0.7 79% 86% | -2.6 85% 92% | 0.1 94% 104% | -2.9 91% 99% | -1.5 87% 95% |

Notes:

[a] Decoder runtime results for HE are believed to be inaccurate due to a bug in the MDDT implementation (ticket #91)

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C029 | m18040 | TE12.1: Report on 64x64 versus 32x32 maximum coding unit size for low complexity configuration | P. Pandit (Harmonic) |
| JCTVC-C030 | m18051 | TE12.1: Report on AMP evaluation | E. Francois (Technicolor), L. Guillo (INRIA), A. Ichigaya (NHK), H. Yu (Huawei) |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C052 | m18073 | TE12.1: Block merging (MRG) test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C056 | m18079 | TE12.1: Evaluation of transform unit (TU) size | M. Zhou, V. Sze (TI) |
| JCTVC-C104 | m18127 | TE12.1: Huawei & HiSilicon report on AMP evaluation | X. Zheng (HiSilicon), J. Zhou, H. Yu (Huawei) |
| JCTVC-C105 | m18128 | TE12.1: Huawei report on max transform unit size evaluation | J. Zhou, D. Wang, H. Yu (Huawei) |
| JCTVC-C109 | m18132 | TE12.1: Results for PU-based merging by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C118 | m18141 | TE12.1 cross-check report on block merging (MRG) on/off | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C149 | m18175 | TE12.1: Report on asymmetric motion prediction unit (AMP) on/off | A. Ichigaya, K. Iguchi (NHK) |
| JCTVC-C200 | m18238 | TE12.1: Experimental results of transform unit quadtree/2-level test | T. Lee, J. Chen, W.-J. Han (Samsung) |
| JCTVC-C231 | m18271 | TE12.1: Results for PU based merging | T.K. Tan |
| JCTVC-C259 | m18300 | TE12.1: Results for experiments on Max CU size | Y. Yu, K. Panusopone, L. Wang, S.-T. Hsiang, F. Ishtiaq (Motorola) |
| JCTVC-C264 | m18305 | TE 12.1: Evaluation of block merging (MRG) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C286 | m18329 | TE12.1: Technicolor & INRIA report on AMP evaluation | E. Francois (Technicolor), L. Guillo (INRIA) |

  - Motion Representation

The results of the tests on motion representation aspects are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| MV prediction IMVP (relative to AMVP) |  |  | -0.7 103% 101% | -0.9 104% 101% | -0.2 103% 101% | -0.4 105% 101% | -0.6 104% 101% |
| Adaptive motion vector resolution (AMVRES) |  |  | 0.5 114% 102% |  | 0.4 115% 103% |  | 0.5 115% 103% |
| Partition-based intensity comp. (PBIC) |  |  | 0.1 102% 105% | -1.6 105% 104% | 0.2 102% 102% | -1.0 103% 105% | -0.6 103% 104% |
| Interpolation filter DCT-IF 6 tap (relative to DIF) |  |  |  | 2.8 100% 98% |  | 0.7 101% 98% | 1.8 101% 98% |
| Interpolation filter DCT-IF 8 tap (relative to DIF) |  |  |  | 4.6 112% 103% |  | 2.5 112% 102% | 3.6 112% 103% |
| Interpolation filter DCT-IF 8 tap (relative to SIFO 12 tap) |  |  | -0.2 62% 94% |  | -1.1 67% 95% |  | -0.7 65% 95% |
| Interpolation filter DCT-IF 12 tap (relative to SIFO 12 tap) |  |  | 0.3 65% 94% |  | -0.1 70% 95% |  | 0.1 68% 95% |
| Interpolation filter SIFO 6 tap (relative to DIF) |  |  |  | 3.4 174% 106% |  | 2.6 152% 103% | 3.0 163% 105% |
| Interpolation filter SIFO 12 tap (relative to DIF) |  |  |  | 5.5 198% 115% |  | 5.9 176% 110% | 5.7 187% 113% |
| Interpolation filter MOMS (relative to SIFO 12 tap) |  |  | 0.7 68% 122% |  | -1.0 62% 111% |  | -0.1 65% 116% |
| Interpolation filter MOMS (relative to DIF) |  |  |  | 5.8 129% 133% |  | 6.3 119% 131% | 6.0 124% 132% |

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C041 | m18062 | TE12.2: Performance of partition based illumination compensation (PBIC) | A. Fujibayashi, S. Kanumuri, F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C050 | m18071 | TE12.2: Interpolation filter SIFO and DIF tested against MOMS by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C087 | m18110 | TE12.2: Experimental results of interpolation filter | T. Chujoh, K. Kanou, T. Yamakage (Toshiba) |
| JCTVC-C119 | m18142 | TE12.2 report on MV prediction AMVP/IMVP | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C162 | m18188 | TE12.2: Results on MC interpolation filters | K. Kondo, T. Suzuki (Sony) |
| JCTVC-C163 | m18189 | TE12.2: Results on MV prediction | K. Sato, T. Suzuki (Sony) |
| JCTVC-C182 | m18211 | TE12.2: Cross-check of adaptive motion vector resolution (AMVRES) technology in the TmuC software | A. Segall (Sharp) |
| JCTVC-C197 | m18235 | TE12.2: Results for SIFO-6Tap with DIF tests by Nokia and Qualcomm | K. Ugur, J. Lainema (Nokia), R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C201 | m18239 | TE12.2: Experimental results of SIFO (DIF) / DCT-IF | E. Alshina, J. Chen, N. Shlyakhov, A. Alshin, W.-J. Han (Samsung) |
| JCTVC-C232 | m18272 | TE12.2: Verification of JCTVC-C041 (partition based illumination compensation – PBIC) | N. Sprljan, S. Paschalakis, P. Wu (Mitsubishi Electric) |
| JCTVC-C239 | m18280 | TE12.2: Report on interpolation filter: SIFO/DIF | R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C241 | m18282 | TE12.2: Crosscheck on interpolation filter: SIFO(DIF)/MOMS | R. Panchal, M. Karczewicz (Qualcomm) |
| JCTVC-C246 | m18287 | TE12.2: Cross-verification of partition-based illumination compensation (PBIC) | P. Onno (Canon) |
| JCTVC-C248 | m18289 | TE12.2: Cross verification on DCT-IF for chroma | T. Chujoh, K. Kanou, T. Yamakage (Toshiba) |
| JCTVC-C265 | m18306 | TE12.2: Evaluation of adaptive motion vector resolution (AMVRES) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |

  - Intra-frame Prediction

The results of the tests on intra-frame prediction are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| Adaptive Intra Smoothing (AIS) - fast version | 1.2 132% 93% | 0.9 139% 95% | 0.5 99% 98% | 0.4 100% 95% | 0.2 98% 102% | 0.1 95% 93% | 0.6 111% 96% |
| Adaptive Intra Smoothing (AIS) - full version | 2.4 164% 100% | 4.0 164% 100% | 0.9 82% 90% | 1.5 82% 84% | 0.4 108% 103% | 0.7 109% 102% | 1.7 118% 97% |
| Combined Intra Prediction (CIP) - preliminary | 0.3 123% 101% | 0.2 125% 100% | 0.2 101% 100% | 0.1 101% 100% | 0.1 101% 100% | 0.0 100% 100% | 0.2 109% 100% |
| Combined Intra Prediction (CIP) - enhanced | 0.7 [a] | 0.9 [a] | 0.4 [a] | 0.3 [a] | 0.1 [a] | 0.0 [a] | 0.4 [a] |
| Planar prediction | 0.0 98% 102% | -0.9 95% 101% | 0.0 108% 112% | -0.2 108% 104% | 0.0 98% 93% | 0.0 96% 95% | -0.2 101% 101% |
| Edge-based prediction | -0.1 114% 105% | -0.1 110% 104% | -0.1 125% 103% | 0.0 102% 100% | 0.0 115% 103% | 0.0 102% 100% | -0.1 111% 103% |

Notes:

[a] Encoding time and decoding time are not available.

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C053 | m18074 | TE12.3: Adaptive intra smoothing (AIS) test (slow vs. fast) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C054 | m18075 | TE12.3: Adaptive intra smoothing (AIS) test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C092 | m18115 | TE12.3: Report on combined intra prediction evaluation | K. Chono, K. Senzaki, H. Aoki, J. Tajime, Y. Senda (NEC) |
| JCTVC-C120 | m18143 | TE12.3 cross-check report on AIS on/off | K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi Electric) |
| JCTVC-C171 | m18200 | TE12.3: Experimental results of edge based prediction | A. Tabatabai, C. Auyeung (Sony) |
| JCTVC-C174 | m18203 | TE12.3: Report on combined intra prediction | T. Yamamoto, A. Segall (SHARP) |
| JCTVC-C188 | m18219 | TE12.3: Results for planar prediction tests by Nokia | J. Lainema, K. Ugur, O. Bici (Nokia) |
| JTVC-C213 | m18251 | - TE12.3: Additional results for combined intra prediction | M. Mrak, T.Davies, D. Flynn, A. Gabriellini (BBC) |
| JCTVC-C216 | m18256 | TE12.3: Results for edge based prediction | V. Drugeon (Panasonic) |
| JCTVC-C244 | m18285 | TE12.3: Report on planar intra prediction | D. Flynn (BBC) |
| JCTVC-C281 | m18324 | TE12.3: Evaluation of fast adaptive intra smoothing | M. Coban, Y. Zheng, M. Karczewicz (Qualcomm) |
| JCTVC- C297 | m18344 | TE12.3: Cross-verification of JCTVC-C213: additional results for combined intra prediction | W.-J. Han (Samsung) |

  - Transforms

The results of the tests on transforms are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| Combined ROT and MDDT (relative to MDDT off, ROT off) | 2.1 182% 222% | 2.7 189% 370% | 0.8 103% 294% | 0.9 108% 588% | 0.3 102% 294% | 0.4 111% 667% | 1.2 133% 406% |
| ROT (relative to MDDT off, ROT off) | 2.1 242% 102% | 2.4 258% 104% | 0.9 105% 100% | 0.9 107% 99% | 0.3 101% 100% | 0.2 102% 100% | 1.1 152% 101% |
| MDDT (relative to MDDT off, ROT off) | 1.6 121% 193% | 2.1 117% 344% | 0.7 106% 234% | 0.8 112% 507% | 0.2 107% 256% | 0.3 113% 477% | 1.0 113% 335% |
| Transform coeff coding | 1.4 109% 95% |  | 1.3 91% 83% |  | 2.0 103% 102% |  | 1.6 101% 93% |
| Transform Skip Flag (relative to off case) |  |  | 0.1 106% 103% | 3.7 93% 91% | 0.0 100% 99% | 3.6 97% 102% | 1.9 99% 99% |

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C047 | m18068 | TE12.4: Transform skip flag test (off vs. on) by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C049 | m18070 | TE12.4: Transform coding HHI tested against Samsung proposal by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C059 | m18082 | TE12.4: Evaluation of transform coefficient coding (HHI_TRANSFORM_CODING) with tool breakdown | V. Sze, M. Budagavi, M. Zhou (TI) |
| JCTVC-C110 | m18133 | TE12.4: Transform unit quadtree tested against 2-level by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C136 | m18160 | TE12.4: Mode dependent direnctional transform (MDDT) test (off vs. on) by HKUST | X. Zhang (HKUST) |
| JCTVC-C181 | m18210 | TE12.4: Cross-check of rotational transform (ROT) | K. Misra, A. Segall (Sharp) |
| JCTVC-C202 | m18240 | TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm | E. Alshina, A. Alshin, W.-J. Han (Samsung), R. Joshi, M. Coban (Qualcomm) |
| JCTVC-C203 | m18241 | TE12.4: Experimental results of transform coefficient coding | J. Chen, V. Seregin, W.-J. Han (Samsung) |
| JCTVC-C258 | m18299 | TE12.4: Evaluation of transform skip flag (RQT_ROOT) | W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C268 | m18310 | TE12.4: Report on evaluation of MDDT and ROT | R. Joshi, M. Coban, M. Karczewicz (Qualcomm) |

  - Filtering

The results of the tests on filtering are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| Deblocking filter | -1.1 100% 103% | 0.6 100% 104% | 0.4 101% 100% | 1.1 101% 103% | 0.4 100% 100% | 0.2 97% 102% | 0.3 100% 102% |
| Adaptive loop filter (ALF) | 2.2 99% 128% | 2.2 143% 127% | 3.7 103% 116% | 4.8 133% 119% | 5.2 101% 116% | 4.3 125% 122% | 3.3 117% 127% |
| 3-input ALF (relative to ALF off) | 3.5 141% 120% | 3.4 215% [b] | 4.7 101% 114% | 5.9 250% 135% | 5.6 106% 114% | 5.2 250% 141% | 4.7 [a] 177% 128% |

Notes:

[a] In subjective viewing 1 out of 4 sequences was considered to be “slightly better”, 3 were considered to be “similar” compared to ALF

[b] Decoding run times are not available for Intra LC

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C129 | m18153 | TE12.5: Cross verification for adaptive loop filter (ALF) (on vs. off) | Y.-J. Chiu, L. Xu, W. Zhang, H. Jiang (Intel) , |
| JCTVC-C148 | m18174 | TE12.5: Crosscheck on deblocking filter in TMuC0.7 by MediaTek | Y.-L. Chang, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-C153 | m18179 | TE12.5: Report on deblocking filter | P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C214 | m18254 | TE12.5: Results for adaptive loop filter using prediction and residual | M. Narroschke (Panasonic) |
| JCTVC-C230 | m18270 | TE12.5: Cross check result of Panasonic's 3-input-ALF | I. S. Chong, M. Karczewicz (Qualcomm) |
| JCTVC-C235 | m18276 | TE12.5: Evaluation of adaptive in-loop filter | I. S. Chong, M. Karczewicz (Qualcomm) |

  - Entropy Coding

The results of the tests on entropy coding are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| PIPE (relative to CABAC) | -0.2 102% 103% |  | -0.3 88% 90% |  | -0.3 103% 102% |  | -0.3 98% 98% |
| LCEC phase 1 (relative to CABAC) |  | -12.5 59% 100% |  | -15.2 71% 88% |  | -15.5 85% 104% | -14.4 72% 97% |
| LCEC phase 2 (relative to CABAC) |  | -12.2 59% 94% |  | -14.8 65% 68% |  | -13.7 73% 90% | -13.6 66% 84% |

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C048 | m18069 | TE12.6: PIPE and LCEC tested against CABAC by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C051 | m18072 | TE12.6: PIPE tested against LCEC by Fraunhofer HHI | B. Bross (Fraunhofer HHI) |
| JCTVC-C058 | m18081 | TE12.6: Evaluation of entropy coders: PIPE tested against CABAC | V. Sze, M. Budagavi (TI) |
| JCTVC-C160 | m18186 | TE12.6: Crosscheck on LCEC phase 2 | P. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-C184 | m18215 | TE12.6: PIPE tested against LCEC by Cisco | A. Fuldseth (Cisco) |
| JCTVC-C196 | m18233 | TE12.6: Results for LCEC_PHASE2 tests by Nokia | K. Ugur, A. Hallapuro, J. Lainema (Nokia) |

  - Other

The results of the tests other decoder issues are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| Internal bit depth increase (IBDI) | 0.8 101% 101% | 1.0 102% 101% | 2.2 100% 101% | 2.0 100% 101% | 4.7 100% 101% | 3.1 100% 101% | 2.3 101% 101% |
| Transform Precision Extension (TPE) | 0.9 100% 99% | 1.1 101% 99% | 0.8 100% 100% | 0.9 100% 100% | 1.4 100% 100% | 1.3 94% 92% | 1.1 99% 98% |

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C057 | m18080 | TE12.7: Evaluation of IBDI and TPE (transform precision extension) | M. Zhou (TI) |
| JCTVC-C088 | m18111 | TE12.7: Experimental results of IBDI | T. Chujoh, T. Yamakage (Toshiba) |
| JCTVC-C252 | m18293 | TE12.7: Report on evaluation of internal bit depth increase and transform precision extension | R. Joshi, M. Karczewicz (Qualcomm) |

  - Encoder-only Tools

The results of the encoder-only tests are summarized below.

|  | Luma BD-Rate reduction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Average |
|  | HE | LC | HE | LC | HE | LC |  |
| Rate Distortion Optimized Quant. (RDOQ) | 4.7 145% 99% | 1.9 135% 99% | 4.4 109% 98% | 3.2 115% 94% | 2.5 109% 101% | 0.8 112% 100% | 2.9 121% 99% |

The information was extracted from the following documents:

| JCT-VC number | MPEG number | Title | Source |
| --- | --- | --- | --- |
| JCTVC-C044 | m18065 | TE12.1, 12.3, 12.8: Results for experiments on max CU size, RDOQ and AIS | F. Bossen, T. K. Tan (NTT DoCoMo) |
| JCTVC-C159 | m18185 | TE12.8: Cross verification of RDOQ | K. Andersson (Ericsson) |
| JCTVC-C238 | m18279 | TE12.8: Results on RDOQ in high efficiency settings | J. Zan, J. Meng, M. T. Islam, D. He (RIM) |

- References

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312rev6, JCT-VC Meeting, Geneva, July 2010

- JCT-VC, “TE12: Summary of evaluation of TMuC tools in TE12”, JCTVC-C225 JCT-VC Meeting, Guangzhou, October 2010