| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C501 WG11 Number: m18604 |
| --- | --- |

| Title: | Core Experiment 1: Decoder-Side Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Description |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu Intel Corp. Haoping Yu Huawei Technologies Co., Ltd. Yu-Wen Huang Mediatek Inc. Shun-ichi Sekiguchi Mitsubishi | Tel: Email: | yi-jen.chiu@intel.com haopingyu@huawei.com yuwen.huang@mediatek.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |
| Source: | Intel Corp., Huawei Technologies Co., Ltd., Mediatek Inc., Mitsubishi Electric Corp. |  |  |

_____________________________

# Introduction

This core experiment targets at comfirming the performance and value of the technique of the decoder-side motion vector derivation (DMVD) on the HEVC Test Model (HM) software platform, and the primary goal of this CE is to test algorithms that can improve the coding efficiency of HM software with feasible complexity of video encoder and video decoder in various implementation platforms. The objectives of this CE include:

# DMVD performance evaluation in the new software environment, i.e. HM of HEVC (technically TMuC 0.9)

# Complexity assessment

# Test interactions between DMVD and other relevant tools

# Participants List

| Participant | Person, Email address | Notes |
| --- | --- | --- |
| Huawei | Haoping Yu, haopingyu@huawei.com Mingyuan Yang, myyang@huawei.com Sixin Lin, linsx@huawei.com | P, C |
| Intel | Yi-Jen Chiu, yi-jen.chiu@intel.com Lidong Xu, lidong.xu@intel.com Wenhao Zhang, wenhao.zhang@intel.com | P |
| JVC Kenwood | Satoru Sakazume, sakazume.satoru@jk-holdings.com Shigeru Fukushima, fukushima.shigeru@jk-holdings.com Motoharu Ueda, ueda.motoharu@jk-holdings.com Hiroya Nakamura, nakamura.hiroya@jk-holdings.com | P |
| Leibniz Univ. Hannover | Sven Klomp, klomp@tnt.uni-hannover.de Holger Meuel, meuel@tnt.uni-hannover.de | C |
| MediaTek | Shawmin Lei, shawmin.lei@mediatek.com Yu-Wen Huang, yuwen.huang@mediatek.com Chih-Wei Hsu, cw.hsu@mediatek.com Ching-Yeh Chen, chingyeh.chen@mediatek.com Jian-Liang Lin, jl.lin@mediatek.com Shan Liu, shan.liu@mediatek.com | P |
| Mitsubishi Electric Corp. | Shun-ichi Sekiguchi, Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Yusuke Itani, Itani.Yusuke@dx.MitsubishiElectric.co.jp | P |
| Peking University | Yue Wang, wangyue@jdl.ac.cn Siwei Ma, swma@jdl.ac.cn Li Zhang, zhanglili@jdl.ac.cn Wen Gao, wgao@pku.edu.cn | P |
| Qualcomm | Rajan Joshi, rajanj@qualcomm.com Wei-Jung Chien, wchien@qualcomm.com | C |
| RWTH Aachen University | Mathias Wien, wien@ient.rwth-aachen.de | C |
| Samsung | Woo-Jin Han, wjhan.han@samsung.com Elena Alshina, elena.a.alshina@samsung.com | P, C |
| Zhejiang University | Lu Yu, yul@zju.edu.cn Shangwen Li, dylanwenster@gmail.com Binbin Yu, zjuybb@zju.edu.cn | C |
| Motorola | Panusopone Krit, krit@motorola.com Yue Yu, yyu@motorola.com | C |

P/C = Proponent / Crosschecking CE partner

The participant group will either bring in a proposal to the next meeting or serve as a cross-checking partner to the next meeting.

# Tools under Test

## Tools under Experiments

### Candidate-based reference block prediction DMVD

| Proposal | Source | Tester | Notes |
| --- | --- | --- | --- |
| JCTVC-C097, B037 | Huawei, Peking Univ. | Motorola(?) | Fast algorithms in B037, Intel could help on cross-checking this proposal |
| JCTVC-C124 | Mitsubishi | Intel |  |
| JCTVC-C127 | Intel | Mitsubishi | Leibniz Univ. Hannover Huawei and Samsung could help on cross-checking this proposal |
| JCTVC-B076 | MediaTek | Leibniz Univ. Hannover |  |

Template matching based DMVD

| Proposal | Source | Tester | Notes |
| --- | --- | --- | --- |
| JCTVC-C097, B037 | Huawei, Peking Univ. | Motorola | Fast algorithms in B037 |
| JCTVC-C127 | Intel | Mitsubishi | Leibniz Univ. Hannover Huawei and Samsung could help on cross-checking this proposal |
| JCTVC-B076 | MediaTek | JVC Kenwood |  |

Per-pixel optical flow based DMVD

| Proposal | Source | Tester | Notes |
| --- | --- | --- | --- |
| JCTVC-C204 | Samsung | MediaTek |  |

### Refinement techniques based on transmitted MVs

| Proposal | Source | Tester | Notes |
| --- | --- | --- | --- |
| JCTVC-C138 | JVC Kenwood | Samsung |  |

## List of Individual proposal

List of proposals to be tested in this CE, including reference to corresponding JCT-VC documents (and draft definition of sub experiments (list in order of document number))

- JCTVC-C097 (m18120) TE1：Huawei report on DMVD improvements in TMuC, [M. Yang, S. Lin, D. Wang, J. Zhou, H. Yu (Huawei)]

  - Template matching based DMVD technique (TM-DMVD) to obtain motion information at decoder. It works in both random access and low delay cases.

  - An extended DMVD approach (STDM): a translational motion model is also used jointly with DMVD to obtain the motion information.

- JCTVC-C124 (m18147) TE1: Implicit direct vector derivation [S. Sekiguchi, Y. Itani (Mitsubishi Electric)]

This technique is a spatio-temporal adaptive direct vector derivation for B-skip/B-direct modes without explicit signaling, which do not require any side information for locally adaptive direct MV derivation relying on a simple decoder-side decision.

- JCTVC-C127 (m18151) TE1: Report of self derivation of motion estimation improvement in TMuC [Y. Chiu, L. Xu, W. Zhang, H. Jiang (Intel)].

  - This contribution reports that the technique of the Candidate-based SDME/DMVD to improve the coding efficiency by utilizing on the mirror-based metric to derive motion vector at video decoder side. Different variants of the simplification techniques was investigated to reduce the complexity of SDME/DMVD and was tested with TMuC 0.7 Software.

- JCTVC-C138 (m18162) TE1.a: Implementation report of refinement motion compensation using DMVD on TMuC [M. Ueda (JVC Kenwood)]

  - In this technique, decoder receives motion vector for uni-predictive to obtain other motion vectors for mixed prediction using DMVD approach. The reference block indicated by the received motion vector is used as template for DMVD.

- JCTVC-C204 (m18242) Bi-directional optical flow [Elena Alshina, Alexander Alshin(Samsung)]

  - New method improving B-slice prediction is proposed in this contribution. By combining the optical flow concept and high accuracy gradients evaluation this proposed technique allows pixel-wise refinement of motion. Proposed approach does not require any signaling about motion vector refinement to decoder. The performance of this tool as well as encoding/decoding complexity will be evaluated in this core experiment.

- JCTVC-B037 [S. Lin, M. Yang, J. Zhou, J. Song, D. Wang, H. Yang, J. Fu, H. Yu (Huawei), W. Yue, L. Zhang, S. Ma, W. Gao (Peking Univ.)]

  - TE1: Huawei report on DMVD improvements
A simplified approach where a bilinear filter is used in the process of determining the motion information in DMVD and STDM. Compared with the conventional DMVD/STDM, this approach has shown a potential for significant complexity reduction while maintaining the coding efficiency..

- JCTVC-B076 TE1: Decoder-Side Motion Vector Derivation with Switchable Template Matching [Y.W. Huang et al. (MediaTek)]

  - MediaTek proposed a switchable template matching (TM) method. Two DMVD modes, DMVD_DIRECT and DMVD_TM, are provided. For DMVD_DIRECT, TM is disabled. A corresponding decoder utilizes only reference picture indices and motion vectors (MVs) of neighboring coding units (CUs) to derive motion information of a current CU with help from a corresponding encoder telling which spatial-temporal neighbors should be used. For DMVD_TM, TM is enabled. The TM search algorithm begins with an initial stage followed by a refinement stage. Moreover, adaptive template shape and boundary weighting are newly developed to improve the coding efficiency with low complexity overhead.

# Core Experiment Tasks

The following tasks shall be performed in this Core experiment.

- Implementation of proposed tools into HEVC HM, the version available on Nov. 1 (technically TMuC 0.9).

- Confirm the  DMVD performance in HEVC HM

- Test DMVD interaction with other relevant tools listed in the HM

# Test Sequences and Test Points

Testing in this CE shall be performed according to the coding conditions and test sequences specified in JCTVC-C500. Intra-only tests won’t be included.

All of the proposal to this CE shall provide simulation results for the sequences specified above. The performance of the proposed tools shall be evaluated in comparison to competing tools. Participants are encouraged to provide additional results (more sequences, more tests configurations) on top of the proposed reference points.

# Target Time-line and Responsibilities

[SONM=Start of next meeting, EOCM=End of Current Meeting, numbers are counting weeks]

- EOCM + 2 (October, 29, 2010):		Final CE-description, deadline for expressing interest in joining CE

- SONM – 4 (December, 23, 2010): 	Completion of tool integration, preparing for testing

- SONM – 3 (December, 30, 2010): 	Start Cross-check

- SONM - 1 (January, 10, 2011):		Complete the task of cross-check, and report the result of cross-check to CE coordinators

- SONM - 1 (January, 13, 2011):	Input Document upload

# Software and Configuration

The CE tools are going to be implemented in the new HEVC HM software available on November 1, 2010, and they will be tested and evaluated under the official common test configurations in JCTVC-C500. Data are requested to compare with the below anchor.

HM anchor: HM common test condition (JCTVC-C500)

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

Objective rate-distortion measurements using PSNR, BD-PSNR, BD-Rate [1]

The visual quality will be assessed by the involved experts.

## Complexity Considerations

For assessment of the complexity impact, the following numbers should be provided:

- Algorithm description including tool configuration

- Number of operations (adds, multiplications, …) on number of reference frames

- Memory bandwidth consumption

- Encoding time (relative to the tool switched off)

- Decoding time (relative to the tool switched off)

# References

1.	Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

2.	Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

3.	JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, 2nd JCT-VC Meeting, Geneva, July 2010

4.	JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd  JCT-VC Meeting, Guangzhou, October 2010