| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C502 WG11 Number: m18602 |
| --- | --- |

| Title: | Core Experiment 2: Flexible Motion Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Edouard Francois (coordinator), Xiaozhen Zheng, Peisong Chen (vice-coordinators) | Tel: Email: | +33 2 99 27 32 62 edouard.francois@technicolor.com peisongc@qualcomm.com zhengxiaozhen@huawei.com |
| Source: | Technicolor, Qualcomm, Huawei & HiSilicon Technologies |  |  |

_____________________________

- Introduction

This Core Experiment (CE) is an ongoing work of previous Tool Experiment 3 subset2 – Inter prediction described in [1]. Its goal is to further investigate Flexible Motion Partitioning. This relates to the partitioning on square blocks into rectangular and non-rectangular partitions. Different solutions have been proposed in previous meetings, [2][3][4][5]. Promising BD-rate performance are reported, , but with concerns regarding encoding complexity. One of the considered solutions (AMP), already present in the TMuC, can be considered as a subset of the two other ones, which actually are variants of the same basic concept.

- Participants List

| Participant | Person, Email address |
| --- | --- |
| Technicolor | Edouard Francois, edouard.francois@technicolor.com Philippe Bordes, philippe.bordes@technicolor.com |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Liwei Guo, liweig@qualcomm.com |
| Huawei/HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Ghent University | Kenneth Vermeirsch, kenneth.vermeirsch@elis.ugent.be |
| MediaTek | Shan Liu, shan.liu@mediatek.com Yu-Wen Huang, yuwen.huang@mediatek.com |
| ETRI | Hui Yong KIM, hykim5@etri.re.kr Jongho Kim, pooney@etri.re.kr |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com |

# Tools under Test

Three related techniques are considered:

- Asymmetric Motion Partitioning (AMP), initially proposed in JCTVC-A124 [2] and integrated into TMuC0.8;

- Motion Compensation with Adaptable Block Shapes (MCABP), proposed by Huawei/Hisilicon, JCTVC-C099 [3];

- Geometry Block Partitioning (GEO), JCTVC-C301 [4], and its simplification (SGEO), JCTVC-C031 [5], proposed by Qualcomm and Technicolor; the tool is integrated into TMuC0.8 Qualcomm branch (rev278).

## Asymmetric Motion Partitioning

The related JCTVC contribution is:

- JCTVC-A124 [2], by Samsung.

Cross-checkers: Huawei & HiSilicon.

Asymmetric motion partition (AMP) was initially proposed in JCTVC-A124 [2]. The tool is implemented in TMuC0.8. The tool consists of partitioning a square block into two non-symmetric horizontal or vertical rectangles shapes, as illustrated in Figure 1 for the 64x64 block size. In the current TMuC implementation (version 0.8), the square block is split using AMP in one rectangle of 3/4 of the square width or height, and another rectangle of 1/4 of the square width or height.

Figure 1: example of asymmetric motion partitioning for a 64x64 block.

## Asymmetric Motion Partitioning with OBMC

The related JCTVC contribution is:

- JCTVC-C251[9], by Qualcomm.

Cross-checkers: Technicolor.

AMP with OBMC was initially proposed in JCTVC-C251[9]. For AMP, the selected transform may be of the same size as the CU and it will span multiple prediction units. In this case, OBMC is used to smooth the boundary between AMP PUs. Let the two AMP prediction units in a CU denoted by region 1 and region 2. A pixel from region 1 (2) is defined to be a boundary pixel if any of its four connected neighbors (left, top, right, and bottom) belongs to region 2 (1). If a pixel A is a boundary pixel, the motion compensation is performed using a weighted sum of itself and the neighboring pixel in the other unit, B.

## Motion compensation with adaptable block shapes

The related JCTVC contribution is:

- JCTVC-C099 [3], by Huawei & HiSilicon.

Cross-checkers: ETRI.

### Tool description

To improve the precision of inter-frame coding, a square block can be divided into two blocks with arbitrary shape. To represent various horizontal, vertical, and non-rectangular partitions, quad-tree design is used for signaling.

In the case of horizontal and vertical partitioning, the syntax element inter_partitioning_idc at TMuC is used to identify the shape of partitions. Based on this design, the square block can be divided into one rectangle with 1/8, 1/4, 3/8 of the square width or height, and the other one rectangle with 7/8, 3/4, 5/8, of the square width or height.

In the case of non-rectangular partitioning, the ratio of two triangles’ edge is used for signaling. For an example, in Figure 2, AO and BO are two edges of the triangle AOB, and the ratio of BO and AO is 2:1. Indicate the ratio’s value, the shape of two non-rectangular blocks can be signaled. At the current design, the value of ratio can be 4:1, 2:1, 1:1, 1:2 and 1:4.

Figure 2: The ratio of two triangles’ edge

### Current implementation and further works

The horizontal and vertical partitioning cases have been implemented in the TMuC0.7. In general, this new design can provide more flexible and accurate asymmetric partitions to TMuC. The addition of these new horizontal and vertical partitions is a straightforward extension of the existing motion partitioning representation in TMuC. This new partitioning design is compatible with all of the tools in the TMuC0.7 software.

In current implementation, besides original PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N asymmetric partitions, additional 8 partitioning type are added to TMuC, which bring more accurate and efficient partitions corresponding to irregular image patterns.

Future work will be focused on:

- Implementation of non-rectangular partitioning;

- Encoding algorithm simplification ;

- Algorithms for non-rectangular deblocking filtering.

## Geometry Motion Partitioning (GEO)

The related JCTVC contributions are:

- JCTVC-C301 [4], by Qualcomm,

- JCTVC-C031 [5], by Technicolor.

Cross-checkers: MediaTek, Huawei & HiSilicon, possibly INRIA.

In addition an informative contribution has also been submitted in 3rd JCTVC meeting:

- JCTVC-C236 [6], by Ghent University.

### Tool description

Geometry motion partition (GEO) is a generic tool enabling to partition a square block into 2 regions using a straight line. One motion vector is sent for each region. The boundary separating the 2 regions is defined by a straight line. Assuming the origin is at the center of the block, each geometry partition is defined by a line passing through the origin that is perpendicular to the line defining the partition boundary. This is shown in Figure 3. The geometry partition is defined by the angle subtended by the perpendicular line with the X axis θ and the distance of the partition line from the originρ. The equation of the line defining the partition boundary can be specified as:  y=−1tanθx+ρsinθ=mx+c

Figure 3: parameters defining a geometry motion partition.

To reduce artifacts along the line, overlapped block motion compensation is used. Let the two regions created by a geometry partition be denoted by region 1 and region 2. A pixel from region 1 (2) is defined to be a boundary pixel if any of its four connected neighbors (left, top, right, and bottom) belongs to region 2 (1). Figure 4 shows an example where grey-shaded pixels belong to the boundary of region 1 (grey region) and white-shade pixels belong to the boundary of region2 (white region). If a pixel is a boundary pixel, the motion compensation is performed using a weighted sum of the motion predictions from the two motion vectors. The weights are 3/4 for the prediction using the motion vector of the region containing the boundary pixel and 1/4 for the prediction using the motion vector of the other region. The overlapping boundaries improve the visual quality of the reconstructed video while also providing BD-rate gain.

Figure 4: Overlapped motion compensation for geometry partitions.

Transform and Quantization of the square block residue is then applied using the actual TMuC process.

GEO is a superset of AMP, or more generally of rectangular partitioning, and therefore can emulate this rectangular partitioning mode. It has been implemented into TMuC0.7.2 and is available in TMuC8.0, Qualcomm branch, revision 278, [4]. In addition one variant of this tool has been proposed in the 3rd JCTVC meeting, [5].

### Current implementation and further works

Considered modes

In current TMuC implementation (TMuC0.8, rev278), GEO is parameterized as follows:

- Block 16x16:  = ,  = 1  238 modes

- Block 32x32: = ,  = 1  494 modes

- Block 64x64: only AMP is used

A simplified version proposed in [5] consists of reducing the number of possible combinations as follows (a similar simplification has been suggested in [6]):

- Block 16x16: Δρ=2, ρmax=6, Δθ=45  26 modes

- Block 32x32 Δρ=4, ρmax=12, Δθ=45  26 modes

- Block 64x64: only AMP is used

Signaling

When GEO is used, it is signaled by the codeword ‘010’. The chosen GEO mode is signaled in the bitstream using FLC, of size 8 bits for 16x16 blocks and 9 bits for 32x32 blocks.

Motion vector coding

Motion vector predictors consist of motion vectors from neighboring blocks, as depicted in Figure 5.

Figure 5: GEO motion vectors predictors.

Motion vector competition applies as for the common square and rectangular partitions. An index is coded for each GEO partition to indicate which of the MV candidates is chosen.

Motion estimation and rate-distortion optimization

Motion estimation and rate-distortion optimization is performed as follows:

- Ordinary motion Estimation is performed for all the square and rectangular motion partitions before motion search for GEO;

- Then for each GEO partition:

  - The motion vector from the largest rectangular block entirely inside the region (when available) is used;

  - The motion cost is evaluated using SAD

- Then the N partitions (practically N is set to 8), giving the minimum SADs, are chosen

- The final motion vector search and RD optimization is performed from this limited set of candidates, and within a limited search window.

Future work will be focused on:

- Encoding algorithm simplification ;

- Identification of the most valuable partitions in order to limit the added complexity and to remove statistically useless modes.

- Experimental Conditions

## Software

The CE tools are going to be implemented into the TMuC 0.9 software, as recommended by the JCT-VC software group at the end of the 3rd JCT-VC meeting.

## Test Sequences, Bit Rates and Coding Conditions

The CE tools will be tested and evaluated under the common official configurations recommended by JCT-VC at the conclusion of the 3rd JCT-VC meeting in document JCTVC-C500 [7]. In particular the reference anchors will be generated using the recommended configuration files provided by the JCT-VC software group.

Restrictions / modifications for this TE:

- No test with intra-only configuration

- Only high coding efficiency configuration

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and encoding/decoding run time comparisons as defined in document JCTVC-B312 [8].

Following comparisons will be made:

- Proposal against TMuC 0.9 default configuration;

- Proposal against TMuC 0.9 default configuration, with AMP switched On.

# Time-line

[NM=Start of next meeting, numbers are counting weeks]

- Fri. Oct. 30th:	Final CE-description

- Nov 1st:		TmuC0.9 delivered – start tool integration

- NM -4: 		Completion of tool integration, start of testing

- NM -4: 		S	tart of cross-checking

- NM -1:		Input Documents upload

# References

- A.Krutz, et al, “Tool Experiment 3: Inter Prediction in HEVC”, Doc. JCTVC-C303, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

- K.McCann, W.-J.Han, I.-K.Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, Doc. JCTVC-A124, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Dresden, Germany, April 2010.

- X.Zheng (HiSilicon), H.Yu (Huawei), “TE3.2: Huawei & HiSilicon report on flexible motion partitioning coding”, Doc. JCTVC-C099 (m18122), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- P.Chen, L.Guo, M.Karczewicz , (Qualcomm), “Geometry motion partition”, Doc. JCTVC-C301 (m18516), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- E.Francois, D.Thoreau, P.Bordes (Technicolor), L.Guillo (INRIA), “TE3 subtest 2: Report on simplified geometry block partitioning”, Doc. JCTVC-C031 (m18052), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- K.Vermeirsch, J.De Cock, R.Van de Walle (Ghent Univ. - IBBT), “Report of complexity analysis of geometric partitioning”, Doc. JCTVC-C236 (m18277), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-C500 (m18607), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- K.McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-B312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

- P.Chen, R. Panchal W.-J. Chien, M.Karczewicz , (Qualcomm), “Overlapped block motion compensation in TMuC”, Doc. JCTVC-C251 (m18292), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.