| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-504 |
| --- | --- |

| Title: | Core Experiment 4 : Interpolation for MC (Chroma) |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Elena Alshina Samsung Yung-Lyul Lee Sejong University Peisong chen, Qualcomm Haricharan Lakshman, HHI | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com +82-2-3408-3753 yllee@sejong.ac.kr peisongc@qualcomm.com haricharan.lakshman@hhi.fraunhofer.de |
| Source: | Samsung, Sejong University, SK Telecom, Qualcomm, Nokia, Toshiba, HHI |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of various interpolation filters (longer tap, finer motion vector accuracy) for the chroma components in first HEVC test model (HM1).

Only default HM1 chroma interpolation filter will be changed in this core experiment.

The objectives of this CE include:

- Coding performance verification of the proposed interpolation filters for chroma components

- Complexity analysis of the proposed interpolation filters

# Participants List

| Participant | Person, Email address | P/C |
| --- | --- | --- |
| Samsung | Jianle Chen, jianle.chen@samsung.com Elena Alshina, elena_a.alshina@samsung.com (coordinator) | P/C |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Rajan Joshi, rajanj@qualcomm.com | P/C |
| Sejong University, SK Telecom | Yung-Lyul Lee, yllee@sejong.ac.kr Dae-Yeon Kim, dykim@dms.sejong.ac.kr, Jung-Pil Kim, jpkim@dms.sejong.ac.kr, | P/C |
| Nokia | Kemal Ugur, kemal.ugur@nokia.com | P/C |
| Toshiba | Takeshi Chujoh, takeshi.chujoh@toshiba.co.jp | C |
| HHI | Haricharan Lakshman, haricharan.lakshman@hhi.fraunhofer.de | P/C |
| Media Tek | Xun Guo, xun.guo@mediatek.com Jicheng An, jicheng.an@mediatek.com | C |
| Cisco | Arild Fuldseth, arilfuld@cisco.com | P/C |
| Sony | Teruhiko Suzuki, teruhikos@jp.sony.com | C |
| LG | Ji-Wook Jung, jiwook.jung@lge.com | C |

P/C = Proponent / Crosschecking TE partner

The participant group will either bring in a contribution to the next meeting or serve as a cross-checking partner to the next meeting.

# Tools under Test

The following tools will be considered in this CE.

| Proponent | Document | Tool description | Cross-checker |
| --- | --- | --- | --- |
| Samsung | JCTVC-C201 | Bi-linear interpolation for chroma is replaced by DCT IF with longer number of taps. | Toshiba Sejong University, Sony |
| Qualcomm | JCTVC-A121 | 1/8 pel accuracy high precision interpolation filter | Media Tek |
| Sejong University, SK Telecom | JCTVC-C126 | Chroma Interpolation Filtering using High Precision Filter | Nokia |
| Nokia | JCTVC-A119 | Chroma interpolation using directional filters | HHI, Samsung |
| HHI | JCTVC-A116 | Chroma interpolation using MOMS | LG, Samsung |
| Cisco | JCTVC-A119 | Two-stage averaging filter for chroma | Qualcomm |

## JCTVC-C201: "TE12.2: Experimental results of SIFO (DIF) / DCT-IF chroma extension ", Samsung Electronics

For the first time DCT IF was applied for chroma motion compensation in JCTVC-A124. In the framework of TE12 the JCTVC-C201 presented experimental result of DCT-IF interpolation filter applied for chroma motion compensation. In the experiments, MV accuracy for chroma component is 1/8 pel and DCT-IF 6 tap was used to replace existing default chroma interpolation in TMuC. This tool experiment will verify coding efficiency and complexity impact when DCT-IF is used as chroma interpolation filter in HM1.

## JCTVC-A121: ”Video coding technology proposal by Qualcomm Inc”

JCTVC-A121 proposed using 3 interpolation filters for 1/4th, 1/2, and 3/4th pixel position. 1/8th pixel position uses direct filters which are derived from the 1/4th pixel position filters. The highest accuracy for the chroma motion vectors is 1/8th pixel.

This tool experiment will verify coding efficiency of high precision filters for chroma.

## JCTVC-C126: " Chroma Interpolation Filtering using High Precision Filter ", Sejong University and SK Telecom

JCTVC-C126 presented the experimental results of chroma interpolation filter with high precision. In the experiments, high precision 6 tap filter is used to interpolate half pixels, and linear and bilinear interpolation filters with high precision are used to interpolate 1/4 pixels and 1/8 pixels. This tool experiment will verify coding efficiency and complexity compared with chroma interpolation filter in TM.

## JCTVC-A119 "Video coding technology proposal by Tandberg, Nokia, Ercisson",

### Chroma interpolation using averaging

JCTVC-A119 proposed using a two-stage averaging filter for chroma interpolation. This method first rounds the chroma motion vectors to quarter-pixel accuracy and then uses simple averaging operations to interpolate the chroma samples. This experiment will verify coding efficiency and complexity impact of using the proposed filter.

### Chroma interpolation using directional filter

JCTVC-A119 proposed 1D directional filters for luma interpolation. This experiment will replace the averaging filter using 1D directional filters and test its performance. In addition it will also test utilizing directional filters used for luma interpolation in JCTVC-A119 for chroma interpolation

## JCTVC-A116 " Description of video coding technology proposal by Fraunhofer HHI",

The proposed interpolation scheme is based on MOMS algorithm and implemented in fixed point arithmetic. Each color plane of the decoded picture is up-sampled separately. So this tool can be tested independently for Chroma. The up-sampling consists of FIR filtering using MOMS basis

# Core Experiment Tasks

The following tasks shall be performed in this Core experiment.

- Implementation of proposed tools into HEVC TM, the version available on Nov. 1.

- Analysis of proposed Chroma interpolation filter performance in HEVC HM (without changing the Luma interpolation filter)

- Complexity analysis for tested tools

# Software and Configuration

The software specified by JCTVC-C500 (the version available on Nov. 1) will be used for testing in this CE. Participants should use common conditions and test sequences as defined by JCTVC-C500. Proposed Chroma interpolation filters will be evaluated for

- low delay & high efficiency,

- low delay & low complexity,

- random access & high efficiency,

- random access & low complexity

test configurations.

# Time-line and Responsibilities

- Fri. Oct. 29th:       Final CE-description

- Nov. 1st : Common test s/w available

- Dec 22. Completion of tool integration, test results representation form announcement and start of cross-check

- Jan. 7:    Complete cross-checking

- Jan. 14:  Input Document upload

- Cross-verifiers are requested to:

- Receive source code from proponent;

- Examine the software to determine if it is implementing the proposed technique;

- Check provided description for number of operations required for

- Compile the software and run the tests described in this CE;

- Compare their results to those provided by the proponent;

- Make comments on the proposed technique and on the results.

# Definition of Performance Measurement Criteria

The performance measurements will be evaluated by switching the individual tools on and off to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

Only chroma interpolation filter should be changed compared to the reference during this CE. No other part changes are allowed (for example, both Luma and Chroma QP settings should be consistent).

Objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate [7] for tools in Section 3.1- 3.5 should be provided. . Tools that will be investigated in this CE modify only chroma MC part. Results of BD-Rate measurements should be reported for each color component separately.

Together with each component BD-rates, the YUV BD-rate should be provided which should be calculated based on mean square errors for all components and averaged PSNRYUV.  The formula for PSNRYUV calculation is specified below.

;

An effect on the subjective quality introduced by the tested tool should be briefly described by the contributors.

## Throughput and Complexity Considerations

In order to make an unbiased comparison of the different interpolation filters, proponents are requested to use the same bit depth for filtering operation.

All test results will be provided by using the same interpolation filter for Luma (DCT IF 12 taps /DIF as it was adopted for first test model of HEVC). The usage of the same test configuration by all proponents will make the decision easier.

The contributors should accompany their RD test results with encoding/decoding time measurement and a detailed description of the number of operations required for the proposed interpolation filter. Number of operations should include both cases: (1) multiplications and additions and (2) shifts and additions to check the validity when multiplication-free implementation is used.

.

# References

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- Elena Alshina, Jianle Chen, Nikolay Shlyakhov, Alexander Alshin, Woo-Jin Han, “TE12.2: Experimental results of SIFO (DIF) / DCT-IF chroma extension,” JCTVC-C201, Guangzhou, October, 2010.

- D. Y. Kim, J. P. Kim, Y. L. Lee (Sejong University), J. Lim, J. Song (SKT) “Chroma interpolation filtering using high precision filter” , JCTVC-C126, Guangzhou, October, 2010.

- Marta Karczewicz, Peisong Chen, Rajan Joshi, Xianglin Wang, Wei-Jung Chien, Rahul Panchal, “Video coding technology proposal by Qualcomm Inc,” JCTVC-A121, Dresden, Germany, April 2010.

- K. Ugur, K. Andersson, A. Fuldseth, "Video coding technology proposal by Tandberg, Nokia, Ercisson”, JCTVC-A119, Dresden, Germany, April 2010.

- T. Wiegand, et. al., " Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, Dresden, Germany, April 2010

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.