| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C507_r3 WG11 Number: m18601 |
| --- | --- |

| Title: | CE7: Alternative Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com |
| Source: | Mitsubishi Electric Research Laboratories, Institute for Infocomm Research, Qualcomm |  |  |

_____________________________

# Introduction

The purpose of CE7 is to investigate the performance of transforms other than the conventional integer DCT-like transform currently in HM. This CE would address the compression efficiency, complexity, and subjective performance aspects of these alternative transforms, along with coefficient scanning techniques related to these alternative transforms. Architectural improvements and simplifications for the conventional transform would be investigated via formal experiments and discussions conducted in the AHG on Spatial Transforms.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Robert Cohen (coordinator) | Mitsubishi Electric Research Labs | cohen@merl.com |
| Chuohao Yeo (coordinator) | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Muhammed Coban | Qualcomm | mcoban@qualcomm.com |
| Joel Solé Rojals | Qualcomm | joels@qualcomm.com |
| Akiyuki Tanizawa | Toshiba | akiyuki.tanizawa@toshiba.co.jp |
| Jun Yamaguchi | Toshiba | jun2.yamaguchi@toshiba.co.jp |
| Elena Alshina | Samsung | elena_a.alshina@samsung.com |
| Felix Fernandes | Samsung | ffernandes@sta.samsung.com |
| Ankur Saxena | Samsung | asaxena@sta.samsung.com |
| Il-Koo Kim | Samsung | ilkoo.kim@samsung.com |
| Wenpeng Ding | Beijing Univ. of Tech. | wpding@gmail.com |
| Yunhui Shi | Beijing Univ. of Tech. | syhzm@bjut.edu.cn |
| Baocai Yin | Beijing Univ. of Tech. | ybc@bjut.edu.cn |
| Haitao Yang | Huawei | haitaoyang@huawei.com |
| Jin Song | Huawei | songjinmail@huawei.com |
| Haoping Yu | Huawei | haopingyu@huawei.com |
| Xin Zhao | Peking Univ. | xzhao@jdl.ac.cn |
| Li Zhang | Peking Univ. | zhanglili@jdl.ac.cn |
| Siwei Ma | Peking Univ. | swma@jdl.ac.cn |
| Ying Weng | BBC | ying.weng@bbc.co.uk |
| Thomas Davies | BBC | Thomas.Davies@bbc.co.uk |
| Xun Guo | Mediatek | xun.guo@mediatek.com |
| Kai Zhang | Mediatek | kai.zhang@mediatek.com |
| Jizheng Xu | Microsoft | jzxu@microsoft.com |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Kazuhisa Iguchi | NHK | iguchi.k-eq@nhk.or.jp |

# Core Experiment tasks

The following tasks shall be performed in this Core Experiment:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency, complexity, and subjective performance of the alternative transforms studied in this CE;

- Investigate interactions between alternative transforms and coefficient scanning schemes;

- Report the results and conclusions of these experiments.

# Description of tools

The following tools shall be evaluated for CE 7:

| Tool # | Proponent | Related Docs. | Tool Abstract | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1 | Toshiba | JCTVC-C080 JCTVC-B042 JCTVC-A117 | One-Dimensional Directional Unified Transform (1DDUT) is a spatial transform scheme for intra coding. 1DDUT has only two 1-D transform matrices, which are Discrete Cosine Transform (DCT) and the directional transform based on Karhunen-Loeve Transform (KLT). The directional transform was trained offline and pre-determined. Four combinations of these two 1-D transform matrices are selected for each intra prediction mode. The intra prediction modes are assigned into four categories, which are three directions and no direction (DC prediction). After intra prediction, the residual signal of the intra prediction is transformed using the set of 1-D transform matrices derived according to the above categories. Then, transform coefficients are converted into 1D signals from 2D signals. Previous experiments indicate that 1DDUT can improve the coding efficiency for intra coding with minimum complexity increase. | 1. Qualcomm 2. Samsung 3. … |
| 2 | I2R | JCTVC-C039 JCTVC-B024 | The proposed tool uses two transform matrices for intra prediction mode-dependent transform coding of intra predicted residuals: a DCT and a derived KLT (DST Type-VII). The choice of row and column transforms is further specified for the 34 intra prediction modes in the unified intra prediction in TMuC. | 1. Samsung 2. Huawei 3. Toshiba 4. … |
| 3 | Samsung | JCTVC-C202 JCTVC-A124 JCTVC-C096 | Rotational Transform (ROT) was proposed to be an adaptive transform for efficient Intra residual encoding. In TE12 the performance of this tool was reported, and high encoding complexity was indicated as the main disadvantage of this tool. This experiment is targeted to investigate a fast version of ROT implemented on top of HM. | 1. MERL 2. Qualcomm 3. Mediatek 4. … |
| 4 | Samsung & Qualcomm | JCTVC-C202 | The proposed tool combines MDDT and ROT for 4x4 and 8x8 blocks by applying KLT and adaptive coefficient scanning with no additional secondary transform when ROT index = 0, and applying DCT with a rotational transform for other ROT indices. For larger block sizes, the proposed tool always applies DCT, and an appropriate rotational transform (or none) is applied as indicated by the ROT index. | 1. Toshiba 2. I2R 3. Peking Univ. 3. … |
| 5 | Samsung | JCTVC-C108 | The proposed tool applies either the conventional DCT or a derived DST separably based on various intra-prediction modes. The proposed transform requires no additional signaling information, works in a single-pass and does not incur any additional computational complexity relative to conventional DCT implementation. No training was needed to derive the transform and it requires the storage of only one DST matrix in addition to DCT. | 1. Toshiba 2. BBC 3. NHK 4. … |
| 6 | Qualcomm | JCTVC-C268 JCTVC-C250 | In TE12, the performance of MDDT was reported (MDDT=ON/OFF) (JCTVC-C268). But the performance was severely impacted by two factors. First, as specified by TMuC, the MDDT was used only for 4x4 and 8x8 blocks. Second, the adaptive coefficient scanning implementation was very slow. In this CE it is proposed that the performance of MDDT be investigated by extending it to higher block sizes and using it with faster adaptive coefficient scanning proposed in JCTVC-C250. Further simplifications of MDDT will also be explored. | 1. Huawei 2. Samsung 3. … |
| 7 | Beijing Univ. Tech. | JCTVC-C305 JCTVC-B073 | We proposed two symmetric transform matrices that previous experiments show improve the performance of the transform, with a comparable complexity to the DCT. The proposed symmetric transform matrices can be implemented using a lifting structure. The proposed transform matrices will be used for intra-predicted residuals. | 1. Microsoft 2. … |
| 8 | Huawei | JCTVC-C106 JCTVC-C102 | The symmetry-based simplification of MDDT and scan orders was proposed in JCTVC-C102 and JCTVC-C106, respectively. The number of transforms and scan orders can be reduced while the coding performance is maintained. | 1. I2R 2. … |
| 9 | Peking Univ. | JCTVC-C089 | The proposed tool applies a certain kind of reordering on the residual samples for each directional intra prediction mode, and a single KLT matrix is employed for all the 33 directional intra prediction modes defined in HM. Prior experiments in JCTVC-C089 suggest that superior coding performance over the HM anchor configuration is achieved using the proposed tool, with an applicable increase of algorithm complexity | 1. Mediatek 2. … |

:

- Test conditions

The following test conditions shall be used for this CE:

## Software

TMuC version 0.9 shall be used for these experiments, as specified in JCTVC-C500 [1].

## Test configurations

This CE shall use the same encoder and decoder configurations as described in JCTVC-C500. Experiments shall be run for the Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity coding configurations.

## Test sequences

The test sequences as described in JCTVC-C500 shall be used for these tests.

## Training of transforms (if applicable)

The test sequences used from Section 5.3 shall not be used for training any proposed alternative transforms. Proponents shall report which sequences they use for training of their transforms, if applicable.

# Evaluation Criteria

## Quantitative metrics

To obtain objective rate-distortion measurements, four point BD-Rate and BD-PSNR metrics [2] shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. The references for these computations shall be generated from the unmodified TMuC version 0.9 software.

## Qualitative evaluation

If sequences coded using a proposed alternative transform exhibit subjective improvements or degradations, proponents shall describe these subjective differences.

## Complexity metrics

For the understanding of complexity, the following shall be provided:

- Operations counts (e.g., number of multiplies and additions) for individual transforms

- Memory requirements for transforms and/or scanning orders

- Specify the minimum bit-precision required by the transforms

- Other operations and memory requirements relevant to the proposed tool, e.g., scaling matrix modification in quantization

- Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided.

# Participant task checklist

Participants shall:

- Write source code or receive source code for their corresponding tool. Typically proponents write the source code and cross-verifiers receive it.

- Examine the software to determine if it is implementing the proposed technique;

- Compile the software and run the tests described in this CE;

- Cross-verifiers shall compare their results to those provided by the proponent;

- Comment on the proposed technique and on the results.

# Timeline

November 1, 2010: TMuC 0.9 software expected to be available

November 2, 2010: Final CE7 description uploaded

December 28, 2010: Cross-verification begins by this date

January 12, 2011: Reporting of all results to CE7 coordinators complete

January 14, 2011: Input documents uploaded

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd JCT-VC Meeting, Guangzhou, China, Oct., 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001.