| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C508_r1 WG11 Number: m18612 |
| --- | --- |

| Title: | Description of CE8: In-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takeshi Chujoh1 Yu-Wen Huang2 Keiichi Chono3 In Suk Chong4 Tomoyuki Yamamoto5 Jeongyeon Lim7 Byeungwoo Jeon6 Jizheng Xu8 Matthias Narroschke9 |  | tomoo.yamakage@toshiba.co.jp takeshi.chujoh@toshiba.co.jp yuwen.huang@mediatek.com chono@ct.jp.nec.com ichong@qualcomm.com yamamoto.tomoyuki@sharp.co.jp jeongyeon@sktelecom.com bjeon@skku.edu jzxu@microsoft.com Matthias.Narroschke@eu.panasonic.com |
| Source: | Toshiba1, MediaTek2, NEC3, Qualcomm4, SHARP5, SK Telecom7, SKKU6, Microsoft8, Panasonic9 |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to improve and/or enhance in-loop filtering techniques in the HEVC Test Model 1. Since some tools interacting in-loop filtering included TMuC 0.7 are not included in TMuC 0.9, it is expected to change the coding efficiency gain as well as relative encoding/decoding time. In order to confirm the coding efficiency gain by Wiener-based in-loop filtering in TMuC 0.9, the similar information reported in TE12 TMuC filtering (TE12.5) at the Guangzhou meeting will also be provided for the anchor.

Techniques covered by this CE are deblocking/debanding filters, Wiener-based in-loop filters and image clipping and offset.

Concerning the deblocking/debanding filters, techniques are evaluated according to BD-rate gain as well as subjective picture quality. The first evaluated method is in-loop deblocking filtering for intra blocks where intra prediction mode and direction are considered in addition to the conventional AVC deblocking process when deciding Bs values for intra blocks. The second evaluated method is Improved Deblocking Filter. The third evaluated method is conditional joint deblocking-debanding filter. The fourth evaluated method is intra de-blocking filter. The first two methods also adjust the strength of the deblocking filter. These are organized in Subset 1 and participants in this activity are SK Telecom, Sungkyunkwan University (SKKU), MediaTek, NEC and Microsoft.

Concerning the Wiener-based in-loop filters, techniques are evaluated according to BD-rate gain, complexity. The first evaluated method is adaptive filter based on combination of sum-modified Laplacian filter indexing whose encoding scheme is simplified. The second evaluated method is parallel adaptive loop filter. The third evaluated method is Quadtree-based Adaptive Restoration. The fourth evaluated method is 3-input Adaptive Loop Filter. The fifth evaluated method is Quadtree-based Adaptive Loop Filter. These are organized in Subset 2 and participants in this activity are Qualcomm, SHARP, MediaTek, Panasonic and Toshiba.

Concerning the image clipping and offset, techniques are evaluated according to BD-rate gain. The first evaluated method is Picture-based adaptive offset. The second evaluated method is Picture-based adaptive clipping. These are organized in Subset 3 and participant in this activity is MediaTek.

The software basis for this CE is TMuC 0.9. For the test sequences, test conditions in JCTVC-C500 are used, except that Subset 1 additionally requires the results turning off ALF for both anchor and proposal.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 | Tomoo Yamakage (coordinator) Keiichi Chono (coordinator) Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Jicheng An Kai Zhang Shawmin Lei Xun Guo Yu-Wen Huang (coordinator) Jizheng Xu Matthias Narroschke (coordinator) Marta Karczewicz In Suk Chong (coordinator) Wei-Jung Chien Andrew Segall Tomohiro Ikai Tomoyuki Yamamoto Jeongyeon Lim Jungyoup Yang Byeungwoo Jeon Takeshi Chujoh Lidong Xu Wenhao Zhang Yi-jen Chiu Ping Wu Andrey Norkin Kenneth Andersson Il-Koo Kim Hirofumi Aoki Kenta Senzaki | Toshiba NEC MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek Microsoft Panasonic Qualcomm Qualcomm Qualcomm SHARP SHARP SHARP SK Telecom SKKU SKKU Toshiba Intel Intel Intel Mitsubishi Electric Ericsson Ericsson Samsung NEC NEC | tomoo.yamakage@toshiba.co.jp chono@ct.jp.nec.com chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com jicheng.an@mediatek.com kai.zhang@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com jzxu@microsoft.com Matthias.Narroschke@panasonic.de martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com asegall@sharplabs.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp jeongyeon@sktelecom.com binbak1@ece.skku.ac.kr bjeon@skku.edu takeshi.chujoh@toshiba.co.jp lidong.xu@intel.com wenhao.zhang@intel.com yi-jen.chiu@intel.com p.wu@uk.merce.mee.com andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com ilkoo.kim@samsung.com hao@bu.jp.nec.com k-senzaki@bp.jp.nec.com; | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

TMuC 0.9 shall be used.  (Will be provided on Nov. 1, 2010.)

## Test Sequences, Bit Rates and Coding Conditions

Follow the recommended sequences, configurations and test cases described in JCTVC-C500.

For Subset 1, results for all six test cases and additional results using ALF off anchor for Random Access cases shall be provided.

For Subsets 2 and 3, all Random Access cases shall be tested.

Providing additional experimental results is encouraged.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures against the anchor.

## Evaluation of Complexity

For the complexity measurement, the TMuC 0.9 software with the anchor configuration and the proposal implemented on the TMuC 0.9 software should be executed on the same machine, and the computational time will be measured for each test sequence and test case. Then, relative computational time is calculated against the anchor. In addition, number of additional encoding passes (to design filter coefficients or calculate offsets) and number of operations in one encoding pass will be used for Subset 2 and 3. Further details for Subset 2 will be described in 4.2.3.

# Description of Tool Experiment

## Subset 1: Deblocking/Debanding Filters

### Functionality Addressed

- Reduction of blocking artifacts

- Reduction of banding-noise, which is noticeable even when coded video PSNR is higher than 40 dB

- Improvement of coding efficiency

### Expected Gains

- Subjective picture quality improvement

- BD-rate reduction

### Evaluation Criteria

- Measure impact on Bitrate/PSNR relative to the anchor(s)

- Complexity (Encoding time measurement, Decoding time measure, Memory Storage, etc.)

- Visual impact on representative output images of sequences encoded at representative rates (“Kimono” 144th frame is one of the representative output images.)

Note1: In Subset 1, two anchors exist, the TMuC 0.9 anchor and the configuration of TMuC 0.9 with ALF off.

Note2: In Subset1, cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipments, and time-slot for informal viewing are available in the Daegu meeting, informal viewing will be conducted in a similar way as the Guangzhou meeting. The selection of test sequences, QP values, and coding conditions for the potential informal viewing in the Daegu meeting are decided before the cross-check start date. Test sequences are selected from 1080 24p, ClassC, and Class E since it is difficult to playback ClassA and 1080 50/60p real-time on computers. The selection should be conducted by CE8 Subtest1 participants including non-proponents.

### Improved Deblocking Filter (JCTVC-C142, MediaTek)

#### General Concept

MediaTek proposed “improved deblocking filter” (IDF) based on H.264 deblocking filter (DF). IDF improves intra block boundaries by using intra mode dependent deblocking filter (MDDF) and modified boundary strength and thresholds (MBST). IDF outperforms TMuC0.7 anchor in all the six test configurations. Bitrate reductions are 1.6%, 1.2%, 1.3%, 1.4%, 0.6%, and 0.5% for intra, random access, low delay, intra LoCo, random access LoCo, and low delay LoCo, respectively. Though the visual differences between the IDF and TMuC0.7 anchor are small, some improvements still can be observed, especially in the image areas with obvious texture directions. The encoding time decreases by 1.8%, and the decoding time increases by 6%, compared to TMuC0.7.

#### Bitstream organization

No need to modify the bitstream syntax.  (i.e. only require the modification of decoding process)

### Conditional Joint Deblocking-Debanding Filter (JCTVC-C091, NEC)

#### General Concept

The proposal injects small pseudo noise into images around an intra-block boundary which is supposed to be a part of areas of low detail with subtle changes in pixel intensity. The injected small pseudo noise masks banding-noise around intra-block boundaries while keeping the rate-distortion performance with the help of the subsequent Wiener filter. Furthermore, when IBDI is used, it also introduces random quantization effect on the IBDI output image. Thereby the conditional joint deblocking-debanding filter significantly improves visual quality of decoded videos, especially when IBDI is used.

#### Bitstream organization

No need to modify the bitstream syntax.  (i.e. only require the modification of decoding process)

### Content-adaptive de-blocking (JCTVC-C273, Microsoft)

#### General Concept

Figure 4 shows the flow chart of the proposed de-blocking scheme. Given a decoded frame after inverse DCT and motion compensation, its luminance component is first processed with the H.264/AVC de-blocking filter. Then a global orientation energy edge detection (OEED) is conducted on the initially de-blocked image to get a local directional feature (LDF) partition. Based on the LDF partition, different de-blocking modes are decided for all blocks. Finally, the content-adaptive de-blocking is performed on both the luminance and chroma components of the decoded frame, with some parameters (e.g., filter orientation and thresholds) updated by the de-blocking mode decision, while others (e.g., boundary strength) directly inherited from the initial H.264/AVC de-blocking.

Figure 4  Flow chart of the proposed de-blocking scheme

The LDF partition is to decide an appropriate deblocking mode for each block. In total 8 deblocking modes are defined: the H.264/AVC mode (mode A), an extra smoothing mode (mode B), and 6 directional modes (mode C-m, m = 2,3,4,6,7,8) in accordance with the filter bank of OEED except the horizontal and vertical directions. The mode decision is based on the following rules.

Mode C: for each 4x4 or 8x8 transformed intra block BI with its upper and left neighbors BU and BL available, if edge pixels appear in BI, the average orientation index mI (an integer between 1 to 8) and the orientation variance vI  on these edge pixels then are calculated. Similarly mU, vU and mL, vL, can be got in case there are also edge pixels in the two neighbors (otherwise they are set to 0). Denote D(·) a distance metric between different orientation indices, once

|  | (1) |
| --- | --- |

it suggests the current block has consistent directional features with its neighbors, and thus BI is decided to be with mode C-mI.

Mode B: for each 16x16 transformed intra block BI with its upper and left neighbors BU and BL available, if no edge pixels appear in BI, BU and BL, then the intensity variances vI, vU and vL in the three blocks are calculated. Once

|  | (2) |
| --- | --- |

it suggests the current block is inside a consecutive non-LDF region, and BI is decided to be with mode B.

Mode A: the remaining blocks are decided to be with mode A.

For deblocking mode C, the filtering involved pixels are no longer in a line perpendicular to the block boundary, but along 6 different orientations. Suppose  are the selected pixels for a 1D filter, during directional deblocking their values are modified as

|  | (3) |
| --- | --- |

The above filter coefficients  are the same as that used in H.264/AVC.

In H.264/AVC there are two thresholds α and β determined by the average QP employed over the block boundary, and deblocking only takes place if

|  | (4) |
| --- | --- |

As the deblocking performance highly depends on these thresholds, they should be carefully designed for our directional deblocking, not only considering QP but also the directional mode. Since our directional deblocking is always performed along the edge orientation, the filter decision thresholds to distinguish edges from blocking degradation should be relaxed. Generally there are

|  | (5) |
| --- | --- |

where A is a magnification parameter related to QP, and B is a modulation parameter related to the directional mode mI.

The magnification parameter A is trained on a set of images containing typical directional edges. At high bit-rate (low QP), image details are still well preserved, so directional deblocking is less encouraged to avoid smoothing anti-directional texture, while at low bit-rate (high QP), directional deblocking can be much stronger as it will not blur the remaining edges.

The modulation parameter B is defined based on the following observation. Since the blocking degradation is caused by different quantization in two adjacent blocks, it is most severe across the block boundary and least severe along the boundary. Thus deblocking should be stronger if its direction is more perpendicular to the boundary. Suppose  is the angle between the directional mode mI and the block boundary, then there is

|  | (6) |
| --- | --- |

For deblocking mode B, the extra smoothing is performed along lines perpendicular to the upper and left block boundaries. Suppose  are the pixels in the same row or column of two adjacent blocks, during extra smoothing their values are modified as

|  | (7) |
| --- | --- |

This is a heavy and long distance smoothing across the block boundary and all pixels in the consecutive non-LDF regions will be filtered. For the extra smoothing mode, no boundary strength and filter thresholds are needed.

Once the deblocking mode decision is done, the 8 different deblocking filters are simultaneously employed in a raster-scan order at the macroblock level for both luminance and chroma components. The reconstructed frame is obtained in a single deblocking pass.

Compared with the H.264/AVC deblocking, our method provides not only filter orientation flexibility but also filter thresholds adaptivity.

#### Bitstream organization

No need to modify the bitstream syntax.  (i.e. only require the modification of decoding process)

### Intra deblocking filter (JCTVC-C130, SKT/SKKU)

#### General Concept

The H.264/AVC deblocking filter assigns the strongest deblocking boundary strength to intra coded blocks and it does not take intra coded conditions such as coded coefficients and intra prediction mode in deblocking filtering of intra blocks. Since deblocking filtering of intra-coded blocks not only affects coding efficiency of intra pictures but also that of inter-coded blocks, it is proposed to apply deblocking filtering to intra blocks considering various intra coding conditions as in Figure 1.

Figure 1  Bs decision of the proposed deblocking filter

If P and/or Q are intra coded and their macroblock types are different, relatively strong deblocking filtering with Bs=3 is applied since the boundary between them has strong blocking artifacts. Otherwise, intra prediction modes of P and Q are taken into account to jointly decide Bs. Different intra prediction modes of P and Q indicate that P and Q are predicted in different directions, thus the effects of prediction and quantization of P and Q are expected to be non-identical. In this case, normal filtering (Bs=2) is applied on the boundary. If intra prediction modes of P and Q are the same, P and Q are most likely to be in the same region, and in this case, filtering direction (block boundary direction) and prediction direction are further compared. If directions of filtering and prediction are the same, ( e.g., as in Figure 2 (a)), no filtering (Bs=0) is done to avoid unintended smoothing across the block boundary between P and Q. On the other hand, if those two directions are different as shown in Figure 2 (b), the weakest filtering (Bs=1) is applied to smooth out weak quantization error appearing on the boundary.

(a) Same direction         (b) Different direction

Figure 2  Example of filtering direction and prediction direction

In summary, on macroblock boundaries, the proposed method can have various filtering (Bs=0, 1, 2, 3) in accordance to intra coding conditions. The strongest filtering (Bs=4) is not applied in this proposal at this time. However, optimization on the assigned Bs values and filtering direction can be further made.

#### Bitstream organization

No need to modify the bitstream syntax.  (i.e. only require the modification of decoding process)

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Microsoft | Jizheng Xu jzxu@microsoft.com |
| NEC | Keiichi Chono chono@ct.jp.nec.com |
| SKT/SKKU | Byeungwoo Jeon bjeon@skku.edu Jungyoup Yang binbak1@ece.skku.ac.kr |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |

Ericsson will cross-check MediaTek's proposal.

Microsoft will cross-check SKT/SKKU's proposal.

NEC will cross-check Microsoft's proposal.

SKT/SKKU will cross-check NEC's proposal.

The goal is to evaluate the performance (BD-rate and improvement of subjective picture quality) of each individual tool listed in this Subset and to explore further coding efficiency improvements as well as combinations of the listed tools, which is optional.

## Subset 2: Wiener-based in-loop filters

### Functionality Addressed

- Improvement of coding efficiency

### Expected Gains

- Improved coding efficiency

- Reduced complexity

- Better understanding of the advantages and disadvantages of individual loop filter aspects

  - Input signals used for the loop filter process

  - Filter shapes (Diamond, Circular)

  - Local adaptation (CU based, Pixel-based)

  - Encoder control strategies

  - Coefficient prediction techniques

- Give some idea on the potential for future harmonization of various in-loop filtering schemes

### Evaluation Criteria

- Coding efficiency: BD-rate

- Complexity in terms of operations and memory access:

  - Relative encoder and decoder runtime

  - Number of encoding passes that are required to estimate the filter coefficients or offsets and perform filtering

  - A description of operations in each of the encoding passes

Note: In case some inconsistency of the interpretation on the evaluation criteria is found among proposals during the editing of CE8 summary report, Coordinators will make efforts to keep the evaluation criteria consistent among proposals and its result may be reported in the CE8 summary report.

### New ALF design description (JCTVC-C113, Qualcomm)

#### General Concept

Qualcomm proposed a new ALF algorithm which reduces complexity of current ALF in TMuC 0.9. In the first step, the filtered/not-filtered map is decided using filters from the previous frames. This map decision step may use more than one filter. Based on the map, new filter coefficients for the current frame are derived, current frame is filtered using derived filter coefficients, and a new filtering map is chosen. Finally, filtered output based on the new filtering map is written back to DPB. Note that the step of designing new filter can be ignored, and filters from the previous frames can be used to filter current frame.

#### Bitstream organization

No need to modify the bitstream syntax in TMuC 0.9.

### Coding Unit Synchronous Picture Quadtree-based Adaptive Loop Filter (JCTVC-C143, MediaTek)

#### General Concept

MediaTeks proposed “coding unit synchronous picture quadtree-based adaptive loop filter” (CS-PQALF). CS-PQALF uses multi-level quadtree partitions to allow local adaptivity for Wiener loop filtering. The partitions boundaries are aligned with the boundaries of the largest coding units (LCUs). Each partition is a basic filter unit (FU) and can be enhanced by different Wiener filters to reduce mean square error between the reconstructed picture and the original picture. For reducing the implementation complexity of CS-PQALF, special attention is paid to reduce the number of encoding passes from 16, for the Qualcomm adaptive loop filter (QC_ALF) in TMuC, to 2, for our CS-PQALF, while maintaining the good coding efficiency performance of the QC_ALF. This reduction of encoding passes can dramatically reduce the power consumption and access time involved in external memory accesses. Simulation results are compared with QC_ALF. The proposed CS-PQALF increases 0.1% and 0.3% BD-rates for high efficiency random access (HE-RA) configuration and high efficiency low delay (HE-LD) configuration, respectively. The encoder execution times are increased by 1% for both HE-RA and HE-LD. The decoder execution times are increased by 3% and 1% for HE-RA and HE-LD, respectively. Note that the software execution time cannot really reflect the reduction of hardware external memory access overhead.

#### Bitstream organization

### DF-combined Adaptive Loop Filter (JCTVC-C173 SHARP)

#### General Concept

DF-combined adaptive loop filter, which was called Parallel adaptive loop filter in C173, uses pre-DF (De-blocking filter) signal and post-DF signal as inputs, where both inputs’ weights are optimized with Wiener-based filter technique. The whole system can be seen as one in-loop filter where De-blocking filter and Wiener-based filter are combined.  This combination provides coding efficiency improvement as well as the functionality to process the two filters in parallel.

sout=iwpre,i⋅spre,i+jwpost,j⋅spost,j+c

Where, sout is output, spre,i and spost are pre-DF pixels and post-DF pixel, respectively. And wpre,i and wpost,j are coefficients for Wiener filter. And c is DC offset.

#### Bitstream organization

The following side information is send in the slice header:

- Flag to indicate the number of input signals (one input or two input)

- Filter coefficients of Wiener in-loop filter

- On/off control information and filter size information

### Adaptive loop filter using prediction and residual (3-Input-ALF) (JCTVC-C214, Panasonic)

#### General Concept

The adaptive loop filter using the prediction and residual (3-Input-ALF) , see references JCTVC-A114 and JCTVC-C214, aims at reducing quantization noise. Based on the prediction signals, the quantized prediction errore', and the reconstructed filter signals'', the following linear filter operations are performed:

s'''=i=1Mwi⋅si''+i=1NwM+i⋅si+i=1OwM+N+i⋅ei'+c

All filter coefficientswi, withi=1,…,M+N+O as well as the offset c are estimated once per frame by minimizing the mean squared error between the original input signalsand the filtered signals''':

D=Es−s'''2→min

The estimated filter coefficients are coded and transmitted to the decoder. The filter sizes M,N,O as well as the quantization step sizes used in the filter coefficients coding step are adaptive and determined by minimization of the Lagrangian costs of overall data rate R and mean squared errorD.:

R⋅λ+D→min

The reduction of the quantization noise leads to a BD bit rate reduction. By reducing the BD bit rate, the 3-Input-ALF contributes to increased subjective quality.

#### Bitstream organization

Additional information transmitted within the bitstream: 
Coefficient information to filter the prediction signal and the quantized prediction error signal.

### 2-pass Quadtree-based Adaptive Loop Filter (JCTVC-C082, Toshiba)

#### General Concept

A loop filter (QALF: Quadtree-based Adaptive Loop Filter) may be applied to the decoded picture before storing to DPB. QALF is intended to reduce coding noise of the decoded picture, and it is decided slice by slice for luminance and chrominance whether the loop filter is applied to a block. A 2-pass encoding scheme is incorporated in this proposal and further improvement of coding efficiency and complexity might be investigated referring the similar schemes.

The luminance filter is a point-symmetrical 2-dimential FIR filter with non-zero coefficients in a concentric circle. Coefficients of the filter are designed slice by slice at encoder and signaled to decoder. The coefficients are adaptively predicted spatially or temporally (including direct prediction). In addition, it is decided block by block at encoder whether the filtering is performed and the decision is signaled to decoder. The chrominance filter is a 5x5-tap point symmetrical rectangular 2-dimentional FIR filter. Coefficients of the filter are designed slice by slice at encoder and signaled to decoder. As for the prediction of filter coefficients, spatial and temporal adaptive prediction in JCTVC-B045 is performed.

#### Bitstream organization

Based on JCTVC-A205_draft001, an additional syntax element to select the prediction mode of filter coefficients is added in slice header. The element signals either spatial, temporal, or direct prediction is used. For the shape of Wiener filter, since the circle shape is used instead of square as described in JCTVC-A205_draft001, appropriate modification is applied.

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Panasonic | Matthias Narroschke Matthias.Narroschke@panasonic.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| SHARP | Tomohiro Ikai ikai.tomohiro@sharp.co.jp |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |

[Tentative cross-check assignment]

MediaTek will cross-check Toshiba’s proposal.

Panasonic will cross-check MediaTek’s proposal.

Qualcomm will cross-check Panasonic’s proposal.

SHARP will cross-check Qualcomm’s proposal.

Toshiba will cross-check SHARP’s proposal and Panasonic’s proposal.

Intel will cross-check Toshiba’s proposal.

## Subset 3: Image clipping and offset

### Functionality Addressed

- Improvement of coding efficiency

### Expected Gains

- BD-rate reduction

### Evaluation Criteria

- BD-rate

- Runtime

Number of encoding passes to calculate offsets

### Controlled Clipping (JCTVC-C146, MediaTek/SHARP)

#### General Concept

MediaTek and Sharp proposed “controlled clipping”. Controlled clipping describes a process that clips predicted or reconstructed pixel values to minimum and maximum values that are signaled in the bit-stream. The clipping process is applied at four stages, namely post-prediction, post-reconstruction, post-deblocking, and post-adaptive loop filter (post-ALF). Results show that the proposed controlled clipping achieves average 0.6% and 0.4% BD-rate reductions for high efficiency random access (HE-RA) and high efficiency low delay (HE-LD) configurations, respectively. The encoding times are increased by 2% and 0% for HE-RA and HE-LD, respectively, and the decoder times are increased by 6% and 2%, respectively.

#### Bitstream organization

### Quadtree-based Adaptive Offset (JCTVC-C147, MediaTek)

#### General Concept

MediaTek proposed “quadtree-based adaptive offset” (QAO). QAO is consisted of two parts, quadtree partitioning and offset compensation. The former splits a picture into multi-level quadtree partitions, and each partition is compensated by one offset compensation method. The latter reduces errors between reconstructed pixels and original pixels of a current picture by using four kinds of offset compensation methods, including uniform band offset, non-uniform band offset, cross pattern edge offset, and diagonal cross pattern edge offset. Simulation results show that in comparison with TE10 anchor which enables adaptive loop filter (ALF) and many high efficiency tools of TMuC0.7 as required in JCTVC-B300, this proposal can achieve 1.2% and 2.0% bitrate reductions for high efficiency random access (HE-RA) configuration and high efficiency low delay (HE-LD) configuration, respectively. The encoding time is increased by 2% and 1% for HE-RA and HE-LD, respectively, and the decoder time is increased by 9% and 9%, respectively.

#### Bitstream organization

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| SHARP | Andrew Segall asegall@sharplabs.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Samsung | Il-Koo Kim ilkoo.kim@samsung.com |

Qualcomm will cross-check MediaTek’s proposal on Controlled Clipping.

Samsung will cross-check MediaTek’s proposal on Quadtree-based Adaptive Offset.

# Time-line and Responsibilities

T1:	2010-Oct-29:	Final CE description and upload

T2a: 2010-Dec-12:	Start cross-checking, proponents to provide software and results to cross-checker(s) for Subset 1

T2b: 2011-Jan-6: 	Start cross-checking, proponents to provide software and results to cross-checker(s) for Subsets 2 and 3

T3:	2011-Jan-7:		Report proponents’ results to Coordinators

T4:	2011-Jan-13:	Report cross-checkers’ results to Coordinators

T5:	2011-Jan-14:	Input document upload

# Previous Contributions

JCTVC-C142 [MediaTek] TE10 subtest 1: Improved deblocking filter

JCTVC-C273 [Microsoft] TE10 Subset 1: Report of content-adaptive de-blocking

JCTVC-C091 [NEC] TE10: Conditional joint deblocking-debanding filter

JCTVC-C130 [SK Telecom and Sungkyunkwan University] TE10 subtest 1: Results of intra deblocking filter testing by SKKU/SKT

JCTVC-C113 [Qualcomm] Adaptive loop filter with low encoder complexity

JCTVC-C143 [MediaTek] TE10 subtest 2: Coding unit synchronous picture quadtree-based adaptive loop filter (QALF)

JCTVC-C173 [Sharp] TE10 subtest 2: Parallel adaptive loop filter

JCTVC-C214 [Panasonic] TE12.5: Results for adaptive loop filter using prediction and residual

JCTVC-C082 [Toshiba] TE10 Subtest 2: Reduction of number of encoding passes for quadtree-based adaptive loop filter (QALF)

JCTVC-C146 [MediaTek/Sharp] TE10 Subtest3: Controlled Clipping

JCTVC-C147 [MediaTek] TE10 Subtest3: Quadtree-based Adaptive Offset

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.

[2]	“Description of HEVC Test Model 1”, JCTVC-C402, Guangzhou, October 2010.

[3]	“HEVC Text Specification Working Draft 0”, JCTVC-C403, Guangzhou, October 2010.