| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C509 WG11 Number: m18600 |
| --- | --- |

| Title: | Core Experiment 9: Motion Vector Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

Some input documents were proposed to the Guangzhou meeting related to motion vector coding. This includes three categories: prediction unit based Merge and Skip tools, the adaption of the set of motion vector predictors at the slice level, and the temporal motion vector memory compression.

This core experiment addresses these topics, and evaluates the performance of the different motion vector coding methods proposed and some possible combinations.

# Participants List

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Canon | P. Onno (patrice.onno@crf.canon.fr) |  | x |
| ETRI & Kyunghee University | S. Jeong (jsy@etri.re.kr) H. Kim (hykim5@etri.re.kr) K. Kim (kimky@khu.ac.kr) G. H. Park (ghpark@khu.ac.kr) |  | x |
| HHI | B. Bross (benjamin.bross@hhi.fraunhofer.de) | x | x |
| Intel | Y.-J. Chiu (yi-jen.chiu@intel.com) L. Xu (lidong.xu@intel.com) W. Zhang (wenhao.zhang@intel.com) |  | x |
| JVC Kenwood | S. Fukushima (fukushima.shigeru@jk-holdings.com) M. Nishitani (nishitani.masayoshi@jk-holdings.com) |  |  |
| LG Electronics | Y. Jeon (yongjoon.jeon@lge.com) |  | x |
| Mediatek | C.-W. Hsu (cw.hsu@mediatek.com) Y.-W. Huang (yuwen.huang@mediatek.com) |  | x |
| Microsoft | J.-Z. Xu (jzxu@microsoft.com) |  | x |
| Mitsubishi | S.I. Sekiguchi (Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp) K. Sugimoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) |  | x |
| Motorola | K. Panusopone (krit@motorola.com) Y. Yu (yyu@motorola.com) |  | x |
| NTT DOCOMO | F. Bossen (bossen@docomolabs-usa.com) Y. Suzuki (suzukiyos@rd.nttdocomo.co.jp) A. Fujibayashi (fujibayashi@nttdocomo.co.jp) | x | x |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) G. Clare (gordon.clare@orange-ftgroup.com) | x | x |
| Qualcomm | W.-J. Chien (wchien@qualcomm.com) | ? | x |
| Samsung | I.K. Kim (ilkoo.kim@samsung.com) |  |  |
| Sharp | A. Segall (asegall@sharplabs.com) Yeping Su (ysu@sharplabs.com) | x | x |
| Sony | K. Sato (Kazushi.Sato@jp.sony.com) T. Suzuki (teruhikos@jp.sony.com) | x | x |
| Sungkyunkwan University | B. Jeon (bjeon@skku.edu) J. Yang (binbak1@ece.skku.ac.kr) | ? | x |

P: proponent, C: cross-checker

# Experiments

## PU Merge and Skip tools

The contribution JCTVC-C109 reports the results of testing the block merging mode at the PU level jointly with a modification of the Skip mode. This subsection of the CE is intended to evaluate the impact of these adaptations on the coding efficiency.

### Modified skip/direct

There are three modifications of the skip/direct modes tested in these experiments. The modifications reduce the set of AMVP candidates used in skip and direct modes. And the disabling of direct mode is also tested in them. The direct mode is not signaled when it is disabled.

The following table shows, the motion vectors that are used for the different modifications.

| skip | direct | (0,0) * | left | above | median | corner | collocated | macro in HM1.0 modified by HHI and DOCOMO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | SAMSUNG_MRG_SKIP_DIRECT 0 DOCOMO_ DISABLE _DIRECT 0 |
| AMVP based | AMVP based | 1 | 1 | 1 | 1 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 0 DOCOMO_ DISABLE _DIRECT 0 |
| Mod. AMVP 1 | Mod. AMVP 1 | 1 | 0 | 0 | 0 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 1 HHI_AMVP_NO_CORNER_FOR_SKIP 0 HHI_AMVP_NO_COL_FOR_SKIP 0 HHI_AMVP_NO_ZERO_FOR_SKIP 0 DOCOMO_ DISABLE _DIRECT 0 |
| Mod. AMVP 2 | Mod. AMVP 2 | 0 | 0 | 0 | 0 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 1 HHI_AMVP_NO_CORNER_FOR_SKIP 0 HHI_AMVP_NO_COL_FOR_SKIP 0 HHI_AMVP_NO_ZERO_FOR_SKIP 1 DOCOMO_ DISABLE _DIRECT 0 |
| Mod. AMVP 3 | Mod. AMVP 3 | 0 | 0 | 0 | 0 | 0 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 1 HHI_AMVP_NO_CORNER_FOR_SKIP 1 HHI_AMVP_NO_COL_FOR_SKIP 0 HHI_AMVP_NO_ZERO_FOR_SKIP 1 DOCOMO_ DISABLE _DIRECT 0 |
| AMVP based | 0 | 1 | 1 | 1 | 1 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 0 DOCOMO_ DISABLE _DIRECT 1 |
| Mod. AMVP 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 1 HHI_AMVP_NO_CORNER_FOR_SKIP 0 HHI_AMVP_NO_COL_FOR_SKIP 0 HHI_AMVP_NO_ZERO_FOR_SKIP 0 DOCOMO_ DISABLE _DIRECT 1 |
| Mod. AMVP 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 1 HHI_AMVP_NO_CORNER_FOR_SKIP 0 HHI_AMVP_NO_COL_FOR_SKIP 0 HHI_AMVP_NO_ZERO_FOR_SKIP 1 DOCOMO_ DISABLE _DIRECT 1 |
| Mod. AMVP 3 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 1 HHI_AMVP_NO_CORNER_FOR_SKIP 1 HHI_AMVP_NO_COL_FOR_SKIP 0 HHI_AMVP_NO_ZERO_FOR_SKIP 1 DOCOMO_ DISABLE _DIRECT 1 |

*	If the zero vector is not used anymore, the signaling is modified. If the motion vector candidate lists for skip/direct are both empty, the direct mode is not signaled, this reduces the number of bits used for the prediction mode signaling. If only one of the two lists is empty, the skip_flag and inter_pred_idc syntax elements are not signaled because they can be inferred from the two lists.

### Modified merge

The block merging mode at the PU/(CU) level was modified as well. The default PU/(CU) based configuration uses the motion vectors left and above of the current block. The modified versions use more than the two motion vectors. The motion vectors used are listed in the following table.

| merge | left | above | collocated | corner** | macro in HM1.0 modified by HHI |
| --- | --- | --- | --- | --- | --- |
| PU/(CU) based | 1 | 1 | 0 | 0 | #define HHI_MRG 1 #define HHI_MRG_PU 1(0) #define HHI_MRG_CU 0(1) #define HHI_MRG_COLLOCATED 0 #define HHI_MRG_CORNER 0 |
| Mod. PU/(CU) based 1 | 1 | 1 | 0 | 1 | #define HHI_MRG 1 #define HHI_MRG_PU 1(0) #define HHI_MRG_CU 0(1) #define HHI_MRG_COLLOCATED 0 #define HHI_MRG_CORNER 1 |
| Mod. PU/(CU) based 2 | 1 | 1 | 1 | 1 | #define HHI_MRG 1 #define HHI_MRG_PU 1(0) #define HHI_MRG_CU 0(1) #define HHI_MRG_COLLOCATED 1 #define HHI_MRG_CORNER 1 |
| Mod. PU/(CU) based 3 | 1 | 1 | 1 | 1 | #define HHI_MRG 1 #define HHI_MRG_PU 1(0) #define HHI_MRG_CU 0(1) #define HHI_MRG_CBF_2Nx2N 0 #define HHI_MRG_COLLOCATED 1 #define HHI_MRG_CORNER 1 |

**	In this case, the corner MV is not above right or below left or above left as in AMVP but above right and below left if both are available.

### Modfied PU merge signaling

An additional syntax element mrg_cbf was added to the PU merging syntax for 2Nx2N inter blocks. A mrg_cbf syntax element equal to 1 signalizes that the current block is merged (merge flag equal to 1) and that no residual data is coded for the current block ( cbf equal to 0 for luma and chroma). There is no need to code the merge flag or the cbf flags because the merge flag can be inferred to be 1 and the cbf flags can be inferred to be 0. A mrg_cbf syntax element equal to 0 signalizes that the merge and cbf flags have to be coded after mrg_cbf. In the HM 1.0 software modified by HHI, mrg_cbf is enabled by default using the following macro:

#define HHI_MRG_CBF_2Nx2N	1

### Experiments

At this stage, the following table is intended to be an indication of the kind of experiments that can be performed.

|  | Skip | Direct | Merge | Software / Configuration | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1.a Reference | AMVP based | AMVP based | CU based*** | HM1.0 | - | - |
| 3.1.b (JCTVC-C109) | Modified AMVP 1 | Modified AMVP 1 | PU based | HM1.0 modified by HHI (available Nov. 5th) | HHI | Motorola |
| 3.1.c | Modified AMVP 2 | Modified AMVP 2 | PU based | HM1.0 modified by HHI (available Nov. 5th) | HHI | Qualcomm |
| 3.1.d | Modified AMVP 3 | Modified AMVP 3 | Modified PU based 1 | HM1.0 modified by HHI (available Nov. 5th) | HHI | Orange Labs |
| 3.1.e | 0 | 0 | Modified PU based 2 | HM1.0 modified by HHI (available Nov. 5th) | HHI | Orange Labs |
| 3.1.f | 0 | 0 | Modified PU based 3 | HM1.0 modified by HHI (available Nov. 5th) | HHI | NTT DOCOMO |
| 3.1.g | AMVP based | AMVP based | PU based | HM1.0 modified by HHI (available Nov. 5th) | MediaTek | Intel |
| 3.1.h | Modified AMVP 1 | Modified AMVP 1 | CU based | HM1.0 modified by HHI (available Nov. 15th) | JVC Kenwood | Mitsubishi |
| 3.1.i | Modified AMVP 2 | Modified AMVP 2 | CU based | HM1.0 modified by HHI (available Nov. 15th) | HHI | NTT DOCOMO |
| 3.1.j | Modified AMVP 3 | Modified AMVP 3 | Modified CU based 1 | HM1.0 modified by HHI (available Nov. 15th) | HHI | SKKU |
| 3.1.k | 0 | 0 | Modified CU based 2 | HM1.0 modified by HHI (available Nov. 15th) | HHI | NTT DOCOMO |
| 3.1.l | 0 | 0 | Modified CU based 3 | HM1.0 modified by HHI (available Nov. 15th) | HHI | NTT DOCOMO |
| 3.1.m | AMVP based | 0 | CU based*** | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO | HHI |
| 3.1.n | Modified AMVP 1 | 0 | PU based | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO | Orange Labs |
| 3.1.o | AMVP based | 0 | PU based | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO | Samsung |
| 3.1.p | Modified AMVP 1 | 0 | CU based*** | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO | ETRI & Kyunghee University |
| 3.1.q | Modified AMVP 2 | 0 | CU based*** | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | HHI | NTT DOCOMO |
| 3.1.r | Modified AMVP 2 | 0 | PU based | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | HHI | NTT DOCOMO |

Note: test 3.1.hi to 3.1.r may be reduced after some preliminary investigation.

***	In these cases, the HHI_MRG_CU macro is not defined and the HHI_MRG_CBF_2Nx2N macro is not defined for CU merging.

## Adaptation and modification of the set of predictors

The contributions JCTVC-B081 and JCTVC-C166 by Sony report that dynamic code number assignment for each of the motion predictor candidate is useful to adapt the set of prediction to the characteristic of the sequence. A similar trend is reported in JCTVC-C291 (Orange Labs), where coding efficiency varying on a time-basis has been shown. Another variation of the adaptation was also reported in JCTVC-B094 (NTT DOCOMO). This subsection of the CE is intended to evaluate the impact of this adaptation on the coding efficiency.

|  | Adaptation | Software / Configuration | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| 3.2.a = 3.1.a Reference | Without adaptation | HM1.0 | - | - |
| 3.2.b | With Adaptation based on JCTVC-C291 |  | Orange Labs | Mediatek LG Electronics |
| 3.2.c | Adaptation based on JCTVC-C166 |  | Sony | Canon |
| 3.2.d | Modification of the set based on JCTVC-B094 |  | NTT DOCOMO | Sharp |

## Temporal motion vector memory compression

The contribution JCTVC-C257 proposes a method to reduce the complexity of motion vector competition.  The focus is reducing the size of the motion vector buffer needed for storing temporal collocated vectors.  The size of the motion vector buffer is significant, considering the granularity of motion representation and that there are two vectors per motion block for B_SLICE.  To reduce the size of that buffer and the associated memory access bandwidth, it is proposes to use a block-based median filter to store motion vectors on a larger granularity.

In this experiment, the coding efficiency impact of the method will be tested. It will also be compared with a method using a simple decimation operation.

|  | Adaptation | Software / Configuration | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| 3.3.a = 3.1.a Reference | Without memory compression | HM1.0 | - | - |
| 3.3.b | With memory compression based on JCTVC-C257 |  | Sharp | NTT DOCOMO |

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-C500. Intra only will not be tested.

Latest version of the common software, v0.9, available on November 1st  will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BDRate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

- T1: 2010-November 1: 	Final CE description and description of proposals

HM1.0 available

- T2: 2010-November 5: 	Modified HM1.0 released by HHI (PU Merge + modified skip)

- T3: 2011-December 17:	 Completion of tool integration, test configuration announcement

Start of cross-check

- T4: 2011-January 7:	Cross-check complete

- T5: 2011-January 11:	Reporting of all results to CE9 coordinators complete

- T6: 2011-January 14:  	Input documents uploaded, CE9 summary report uploaded

# References