| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C509 WG11 Number: m18600 |
| --- | --- |

| Title: | Description of Core Experiment 9: Motion Vector Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

Some input documents were proposed to the Guangzhou meeting related to motion vector coding. This includes three categories: prediction unit based Merge and Skip tools, the adaption of the set of motion vector predictors at the slice level, and the temporal motion vector memory compression.

This tool experiment addresses these topics, and evaluates the performance of the different motion vector coding methods proposed and some possible combinations.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| HHI | B. Bross (benjamin.bross@hhi.fraunhofer.de) |
| Intel | Y.-J. Chiu (yi-jen.chiu@intel.com) L. Xu (lidong.xu@intel.com) W. Zhang (wenhao.zhang@intel.com) |
| Mediatek | C.-W. Hsu (cw.hsu@mediatek.com) Y.-W. Huang (yuwen.huang@mediatek.com) |
| Microsoft | J.-Z. Xu (jzxu@microsoft.com) |
| Motorola | K. Panusopone (krit@motorola.com) |
| NTT DOCOMO | F. Bossen (bossen@docomolabs-usa.com) Y. Suzuki (suzukiyos@rd.nttdocomo.co.jp) |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) G. Clare (gordon.clare@orange-ftgroup.com) |
| Qualcomm | W.-J. Chen (xxx) |
| Sharp | A. Segall (asegall@sharplabs.com) |
| Sony | K. Sato (Kazushi.Sato@jp.sony.com) T. Suzuki (xxx) |
| Sungkyunkwan University | B. Jeon (bjeon@skku.edu) J. Yang (binbak1@ece.skku.ac.kr) |

# Experiments

## PU Merge and Skip tools

The contribution JCTVC-C109 reports the results of testing the block merging mode at the PU level jointly with a modification of the Skip mode. This subsection of the CE is intended to evaluate the impact of these adaptations on the coding efficiency.

At this stage, the following table is intended to be an indication of the kind of experiments that can be performed.

|  | Skip+Direct | Merge |
| --- | --- | --- |
| 3.1.a Reference | AMVP based | CU based |
| 3.1.b | AMVP based | PU based (TMuC 0.8-hhi-bugfix) |
| 3.1.c (JCTVC-C109) | Modified Skip (TMuC 0.8-hhi-bugfix) | PU based (TMuC 0.8-hhi-bugfix) |
| 3.1.d | PU based | CU based |

## Adaptation of the set of predictors

The contributions JCTVC-B081 and JCTVC-C166 by Sony report that dynamic code number assignment for each of the motion predictor candidate is useful to adapt the set of prediction to the characteristic of the sequence. A similar trend is reported in JCTVC-C291 (Orange Labs), where coding efficiency varying on a time-basis has been shown. Another variation of the adaptation was also reported in JCTVC-B094 (NTT DOCOMO).This subsection of the CE is intended to evaluate the impact of this adaptation on the coding efficiency.

|  | Adaptation |
| --- | --- |
| 3.2.a = 3.1.a Reference | Without adaptation |
| 3.2.b | With Adaptation based on JCTVC-C291 |
| .3.2.c | Modification of the set based on JCTVC-B094 |

## Temporal motion vector memory compression

The contribution JCTVC-C257 proposes a method to reduce the complexity of motion vector competition.  The focus is reducing the size of the motion vector buffer needed for storing temporal collocated vectors.  The size of the motion vector buffer is significant, considering the granularity of motion representation and that there are two vectors per motion block for B_SLICE.  To reduce the size of that buffer and the associated memory access bandwidth, it is proposes to use a block-based median filter to store motion vectors on a larger granularity.

In this experiment, the coding efficiency impact of the method will be tested. It will also be compared with a method using a simple decimation operation.

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-C500.

Latest version of the TMuC, available on November 1st  will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BDRate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

- 2010-August 13:	Final TE description, and description of proposals

- 2010-September 10:	Testing complete

- 2010-September 24: 	Cross-check complete

- 2010-October 01:	Input Document upload

# References