| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C510 WG11 Number: m18606 |
| --- | --- |

| Title: | Core Experiment 10: Number of Intra Prediction Directions |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto (Mitsubishi Electric Corporation) | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Introduction

An increased number of directions for intra prediction was introduced in the TMuC and will be integrated into the HEVC Test Model[1]. The purpose of this CE is to characterize the performance and complexity of using different numbers of intra prediction directions. The characterization will be performed by investigating the coding efficiency improvement in terms of BD-BR [2] performance, and by determining the complexity increase for both encoding and decoding using variety of combinations of intra prediction directions for each block size.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| INRIA | Laurent Guillo, laurent.guillo@inria.fr |
| Institute for Infocomm Research | Yih Han Tan, yhtan@i2r.a-star.edu.sg |
| NEC | Keiich Chono, chono@ct.jp.nec.com Hirofumi Aoki, hao@bu.jp.nec.com Kenta Senzaki, k-senzaki@bp.jp.nec.com |
| KAIST | Jaeil Kim, jaeil1203@kaist.ac.kr Munchurl Kim, mkim@ee.kaist.ac.kr |
| MediaTek | Xun Guo, xun.guo@mediatek.com Mei Guo, mei.guo@mediatek.com |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Nokia | Jani Lainema, jani.lainema@nokia.com |
| Samsung | Jianle Chen, jianle.chen@samsung.com |
| Santa Clara University | Guichun Li, g1li@scu.edu |
| Toshiba | Akiyuki Tanizawa, akiyuki.tanizawa@toshiba.co.jp |

# Test points and Cross-checkers

The test points are identified in the table below with respect to the number of intra prediction directions up to the one defined for the HEVC Test Model default configurations[3]. The test shall use TMuC0.9, which is expected to include the new intra mode decision method for unified intra prediction proposed in C207, according to the results from the Guangzhou meeting.

Table 1. Performance Tests for different combinations of number of intra prediction directions

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |
| 4x4 | 17 | 17 | 9 | 9 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 9 |
| 8x8 | 34 | 17 | 9 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 |
| 16x16 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 9 | 34 | 34 | 34 | 17 |
| 32x32 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 9 | 34 | 17 |
| 64x64 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 |

Test #12: To evaluate the upper limit of the performance of the intra prediction method of the test model,

the reference without fast mode decisions, which means that all intra prediction directions are evaluated using RD cost, is also tested.

Table2. Default configurations in TComRom.cpp file

| Default configurations | #if UNIFIED_DIRECTIONAL_INTRA const UChar g_aucIntraModeNumAng[7] = { 3, // 2x2 17, // 4x4 34, // 8x8 34, // 16x16 34, // 32x32 5, // 64x64 5 // 128x128 }; const UChar g_aucIntraModeBitsAng[7] = { 2, // 2x2 5, // 4x4 6, // 8x8 6, // 16x16 6, // 32x32 3, // 64x64 3 // 128x128 }; |
| --- | --- |

Table3. value for g_aucIntraModeBitsAng

| Number of Directions | 3 | 5 | 9 | 17 | 34 |
| --- | --- | --- | --- | --- | --- |
| Number of bits | 2 | 3 | 4 | 5 | 6 |

Cross-checkers for each test are shown in Table4.

Table4. Cross-checkers

| Cross-checker | Test |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 |
| INRIA | x | x | x | x | x | x | x | x | x | x | x |  |
| I2R |  | x |  |  |  |  |  |  |  |  |  | x |
| NEC | x | x | x | x | x | x | x | x | x | x | x | x |
| KAIST | x | x | x | x | x | x | x | x | x | x | x |  |
| MediaTek | x | x | x | x | x | x | x | x | x | x | x | x |
| Mitsubishi | x | x | x | x | x | x | x | x | x | x | x | x |
| Nokia | x | x | x | x | x | x | x | x | x | x | x | x |
| Samsung | x | x | x | x | x | x | x | x | x | x | x | x |
| Santa Clara University | x | x | x | x | x | x | x | x | x | x | x | x |
| Toshiba | x | x | x | x | x | x | x | x | x | x | x | x |

# Core Experiment Tasks

The following tasks shall be performed in this core experiment:

- Determine the performance and complexity of using different numbers of intra prediction directions as compared to the HEVC Test Model.

- Identify the best combination of the number of intra prediction directions for each partition size, from the viewpoint of performance/complexity balance.

# Experimental Conditions

## Software

TMuC0.9 shall be used for the CE tests.

## Test Sequences, Bit Rates and Coding Conditions

Recommended test conditions are the intra-only configuration and test sequences as defined in C500[3], using the reference configuration files also provided in [3]. The participants shall consider both High Compression Efficiency and Low Complexity Intra coding test conditions.

## Evaluation of CE Results

Results of the CE will be evaluated on the Basis of BD-Rate and BD-PSNR measurements. For the complexity measurement, the anchor generation and the tests shall be executed on the same platform including CPU spec, and the encoding and decoding times will be reported for each configuration.

# Timelines

| November 1, 2010 | Deadline for sending email to coordinator expressing interest in participating in core experiment, and final CE description available |
| --- | --- |
| November 1, 2010 | Start verifications |
| January 7, 2011 | Cross-verification completed, report results to coordinator |
| January 14, 2011 | verification results uploading |

# References

- Thomas Wiegand, “HEVC text specification Draft 1”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C403, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.