| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C512 WG11 Number: m18605 |
| --- | --- |

| Title: | Core Experiment 12 : Adaptive Motion Vector Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien 5775 Morehouse Dr. San Diego, CA 92130, USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

- Introduction

The core experiment is established to explore the performance and value of adaptive motion vector resolution (AMVRES) and to analyze the interaction of AMVRES with GPB in HEVC Test Model (HM). In AMVRES, motion vector resolution is signaled adaptively. The adaptation can be applied at various levels, such as slice level, CU level, or PU level. This allows more efficient motion vector coding as well as higher accurate motion prediction. In current HEVC design, GPB also provides a way to generate higher accurate motion prediction by using bi-directional motion prediction. Experiments will be performed with GPB enabled (and GPB disabled as supplemental information) using a common platform, and the coding performance, complexity and other characteristics of each technique will be reported.

# Participants List

| Participant | Person, Email address | P/C |
| --- | --- | --- |
| Qualcomm | Wei-Jung Chien, wchien@qualcomm.com | P |
| MediaTek | Yu-Wen Huang, yuwen.huang@mediatek.com Xun Guo, xun.guo@mediatek.com | C |
| Sungkyunkwan Univ. Sejong Univ. SK Telecom | Byeungwoo Jeon, bjeon@skku.edu Joo-Hee Moon, jhmoon@sejong.ac.kr Ju-ock Lee, ljoblue@sju.ac.kr Jeongyeon Lim, jeongyeon@sktelecom.com | P |
| Sharp | Andrew Segall, asegall@sharplabs.com | C |
| Samsung | Elena Alshina, elena.alshina@gmail.com | P |
| Nokia | Kemal Ugur, Kemal.Ugur@nokia.com | P |

P/C = Proponent / Crosschecking CE partner

# Tools under Test

- JCTVC-C265 [W.-J. Chien, P. Chen, M. Karczewicz (Qualcomm)] TE 12: Evaluation of Adaptive Motion Vector Resolution (AMVRES) [1]
In this technique, the motion vector accuracy is adaptively chosen between 1/4 and 1/8 pel for each motion vector in a prediction unit (PU). The motion vector resolution is signaled with an AMVRES flag if uni-directional motion prediction is used in PU. For bi-directional prediction, a unary binarization is used to code AMVRES flags.

- JCTVC-A124 [Ken McCann, Woo-Jin Han and Il-Koo Kim] “Samsung’s Response to the Call for Proposals on Video Compression Technology,” [2]
In this technique, the motion vector accuracy is adaptively chosen between 1/4 and 1/12 pel for each motion vector in a prediction unit (PU). Additional motion information beyond 1/4 pel accuracy is represented by simple refinement information instead of requiring a more accurate motion vector. More specifically, high accuracy motion is represented by MV with 1/4 pel accuracy + refinement. The benefit of this representation is as follows. Firstly, the magnitude of the motion vector is not increased because the accuracy of motion vector is not changed, only the refinement. Secondly, since motion vector prediction is done without refinement information, there is no increase in the difficulty of motion vector prediction. Finally, the overhead is minimized since the refinement information is sent only when it is useful. For example, for each motion vector component, the first bit of refinement indicates whether the refinement is needed or not. If the bit is zero, it means that there is no more refinement information and next bit is not needed. If the bit is not zero, following bit indicates -1/12 or +1/12 refinement.

- JCTVC-A113 [J. Lim, J. Song (SK telecom), H. Park, C.-W. Seo, D.-Y. Kim, J.O. Lee, M.-J. Kim, S.-W. Hong, M.-H. Jang, H. K. Kim, Y.-L. Lee, J.-K. Han (Sejong Univ.), B. Jeon (Sungkyunkwan Univ.), J.-H. Moon (Sejong Univ.)] “Video coding technology proposal by SK telecom, Sejong Univ. and Sungkyunkwan Univ” [3] 
In this technique, the motion vector accuracy is adaptively chosen between 1/4 and 1/8 pel. The motion vector resolution is signaled as a flag in coding unit (CU) or a prediction unit (PU). The flag in a CU-level signals a choice among 1/4-pel, 1/8-pel, or PU-level signaling. The selective flag for a PU-level signals a choice between 1/4-pel and 1/8-pel.

- JCTVC-C215 [K. Ugur, J. Lainema (Nokia)] Adaptive MV resolution with directional filters
This contribution presents utilizing adaptive MV resolution using 1D directional filters. More specifically:

1. The motion vectors are restricted so that they are allowed to point only to positions that are located where they could be interpolated using 1D directional filters (see Figure 1).

2. The motion vector prediction is always done in quarter pixel. Encoder signals MVD and “one eight pixel refinement bit” to the decoder. If the refinement bit is 0, then MV with quarter-pixel accuracy is used. Otherwise decoder maps the quarter pixel position to one of the one eight pixel positions.

# Core Experiment Tasks

The following tasks shall be performed in this core experiment.

- Implement proposed tools into TMuC v0.9

- Test AMVRES coding performance and computation complexity for high efficiency and low complexity configurations.

- Report the results

# Test Sequences and Test Points

Testing in the CE shall be performed according to the coding conditions and test sequences specified in [JCTVC-C500].

# Time-line and Responsibilities

- Fri. Oct. 29th:	Final CE-description

- Mon. Jan 3rd:  Completion of tool integration, start of testing

- Wed. Jan 5th: 	Cross-check

- Fri. Jan 14th:	Input Documents upload

# Software and Configuration

Use the TMuC version 0.9 available on Nov. 1st, and the common official configurations recommended by JCTVC at the conclusion of this meeting.

Restrictions / modifications for this CE:

- Default low delay and random access configurations. Intra only configuration will not be tested.

- For higher accuracy motion, use DCT-IF or DIF 6 Taps for low complexity configuration and DCT-IF 12 Taps for high efficiency configuration

- Additional test of GPB = 0 for low delay configuration can be provided as supplementary information

- Additional test of DCT-IF 6 taps for low complexity configuration can be provided as supplementary information

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

Objective rate-distortion measurements using PSNR, BD-PSNR, BD-Rate [2]

## Complexity Considerations

For assessment of the complexity impact, the following numbers should be provided:

- Algorithm description including tool configuration.

- Encoding time (relative to the tool switched off)

- Decoding time (relative to the tool switched off)

# Crosscheck List

| Proponent | Crosscheck |
| --- | --- |
| Nokia | MediaTek, Sungkyunkwan Univ. Sejong Univ. SK Telecom |
| Qualcomm | Sharp, Samsung |
| Samsung | MediaTek, Qualcomm |
| Sungkyunkwan Univ. Sejong Univ. SK Telecom | Sharp, Nokia |

# References

- W.-J. Chien, P. Chen, M. Karczewicz, “TE 12: Evaluation of Adaptive Motion Vector Resolution (AMVRES),” JCTVC-C265, Guangzhou, China, Oct 2010.

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- J. Lim, J. Song, H. Park, C.-W. Seo, D.-Y. Kim, J.O. Lee, M.-J. Kim, S.-W. Hong, M.-H. Jang, H. K. Kim, Y.-L. Lee, J.-K. Han, B. Jeon, J.-H. Moon, “Video coding technology proposal by SK telecom, Sejong Univ. and Sungkyunkwan Univ,” JCTVC-A113, Dresden, Germany, April 2010

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.