| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D003 |
| --- | --- |

| Title: | AHG Report: Software development and HM software technical evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen David Flynn Karsten Sühring | Tel: Email: | +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | AHG chairs |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG on Software development and HM software technical evaluation that have taken place between the 3rd and 4th JCT-VC meeting. Activities included integration of tools into a common code base, removal of tools not included in HM, bug fixes, etc. A stripped version of HM software is now available.

# Introduction

Mandates given to the AhG are as follows:

- Coordinate development of the HM software and its distribution to JCTVC members

- Produce documentation of software usage for distribution with the software

- Deliver TMuC 0.9 software version and the reference configuration encodings according to JCTVC-C500 for use in common conditions experiments.

- Edit JCTVC-C500 to reflect configurations required to run the software in "HM mode".

- Deliver HM 1.0 and corresponding reference configuration encodings according to JCTVC-C500. (HM 1.0 will be a stripped version of TMuC 0.9 wherein tools not included in HM are removed from the codebase.)

- Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text

A brief summary of activities related to each mandate is given below.

- Development of the software was coordinated with the parties needing to integrate changes. Several tracks were typically pursued in parallel to speed up development. The distribution of the software was made through the SVN servers set up at HHI and BBC, as announced on the jct-vc email reflector.

- A draft documentation of software usage was produced and distributed on the reflector. A refined version thereof is submitted as input contribution JCTVC-D404.

- Version 0.9 of the software was delivered according to schedule and reference configuration encodings were provided according to the common test conditions through an ftp site at the BBC (ftp://ftp.kw.bbc.co.uk/hevc/tmuc-0.9-anchors/).

- No edits were made to JCTVC-C500. Instead precise configuration options were provided as part of release 0.9 of the software.

- Version 1.0 of the software was delivered ahead of the 4th meeting. Tools not included in the HM design were removed from the code base, resulting in the shrinking of the code base in about half.

- While the first draft of the HEVC HM 1.0 text was not made available until shortly before the 4th meeting, a number of potential mismatches were identified, as further reported below.

# History of revisions

Multiple versions of the TMuC software were produced and announced on the jct-vc email reflector. The following sections give a brief summary of the changes made for each version. A detailed history of changes made to the software can be viewed at http://hevc.kw.bbc.co.uk/trac/timeline

Released versions of the software are available on the SVN server set up at the following URL: https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/version_number where version_number corresponds to one of the versions described below (e.g., 0.9 or HM-1.0). Intermediate code submissions can be found on a variety of branches available at https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/branches/branch_name where branch_name corresponds to a branch (e.g., 0.8-qualcomm).

## Version 0.9

All proposals accepted at 3rd meeting were integrated into version 0.9. Additionally tool configurations were changed to reflect the agreed HM settings.

One issue was raised regarding the combination of LCEC and RQT. It appears that the RQT design for LCEC is specific to the default tree depths specified for low-complexity common conditions. This issue should be further discussed and resolved. The software currently prints a warning message when tree depths other than the defaults are used in conjunction with LCEC. While encoder and decoder function (bit streams are properly decodable), a noticeable degradation in coding performance may be observed.

## Version 1.0

Version 1.0 significantly differs from the previous version as a significant amount of code was removed. This code was related to tools not included in the HM design. Additionally a number of issues were addressed:

- #119 related to an implementation issue in the deblocking filter (disabled for now, alignment with text needs to be confirmed)

- #118 related to an issue with ALF wherein badly formed bit streams were generated for very small values of QP

- #117 related to an issue in the motion vector candidate list generation (disabled for now, alignment with text needs to be confirmed)

- #114 related to an encoder/decoder mismatch issue when setting GPB to 0

- #115 related to an issue in the motion estimation for B0 pictures in RA configurations

- #110 related to an issue in the RD cost computation when using the directional interpolation filter

- #116 related to duplicate testing in the motion estimation

- #100 related to the encoding of the reference frame index (disabled for now, alignment with text needs to be confirmed)

- #107 related to a crash in the encoder when simply displaying a help message

- #111 related to redundant code in the motion estimation

# Potential mismatches with text

As noted in the release notes for version 1.0, three items were identified as areas of potential mismatch between text and software:

- #119 on the deblocking filter; document D395 provides additional details.

- #117 on AMVP candidate list generation; coding results including a fix were provided to the ahg chairs and indicate that coding performance is not affected either way (average BD-rate differences of 0.0% were recorded).

- #100 on coding of reference picture indices.

High-level syntax also needs to be implemented.

Additionally the following should also be considered:

- A potential mismatch in the ordering of MV candidates in AMVP which was raised on the email reflector.

- Document D027 that lists a number of issues potentially related to mismatches between text and software.

# Recommendations

- Continue to develop reference software based on the HM 1.0 version

- Continue to identify bugs and discrepancies with text, and address them

- Continue to improve code quality

- Review reference configurations for common conditions and identify possible appropriate changes