| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D004 WG11 Number: m19161 |
| --- | --- |

| Title: | JCT-VC AHG report: Slice support development and characterization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki Japan | Tel: Email: | +46761153563 rickard sjoberg@ericsson com |
| Source: | Ericsson, Qualcomm, Fujitsu |  |  |

_____________________________

# Abstract

This report summarizes the slice support development and characterization ad-hoc activities between the 3rd and the 4th JCT-VC meeting and the input documents to this meeting related to this ad-hoc group.

# Mandate

| AHG Title and Email Reflector | Chairs |
| --- | --- |
| Slice support development and characterization (jct-vc@lists.rwth-aachen.de) - Implement basic slice support (independently-decodable sequences of largest coding units in raster-scan order) into the HM software. - Identify issues relating to the draft text description of slice functionality - Coordinate the integration of slice support with the software development and HM software technical evaluation ad hoc group - Study technical proposals relating to slice structured coding - Study the coding efficiency and loss resilience impact of slice-structured coding - Identify and discuss additional issues relating to slice-structured coding | R. Sjöberg (chair), Y. Chen, K. Kazui (vice chairs) |

# Status

The main focus of the ad-hoc group has been mandate 1, to implement slice support into the software. The following timeline shows the progress:

| November 5 | Ad-hoc group kick-off message sent to the jct-vc reflector. Request for source code starting point were made. |
| --- | --- |
| November 16 | A branch for the slice source code was created: "0.9-ahg-slices" |
| November 26 | The slice implementation mailing list was announced. The first version of the source code was committed: First version of the slice implementation. The implementation support basic slices defined as a number of consecutive LCUs. The slice coding is controlled by two config parameters that can set the encoder to encode a certain number of LCUs in each slice or to enforce a certain slice size in bytes. #if AD_HOC_SLICES is used around the code changes relative to TMuC 0.9. |
| December 6 | Source code update: In TComPattern::initAdiPattern and TComPattern::initAdiPatternChroma: Made check for bAboveLeftFlag independent of bAboveFlag. This bugfix has no inpact for raster-scan slices but if non-raster-scan slices are introduced this fix is needed. |
| December 16 | Source code update: Three changes: 1) A TComDataCU.cpp cleanup suggested by Kimihiko Kazui 2) A TComPattern.cpp cleanup suggested by Kiran Misra 3) A bufgix for SHARP_SLICE_TEST_OUTOFORDER_DECOMPRESS=1 The encoding/decoding behavior is unchanged |
| January 13 | Source code update: Updated slice code to use one TComSlice object per slice instead of one TComSlice object for all slices. Code provided by Kimihiko Kazui. The encoding/decoding behavior is unchanged. |

The slice source code currently in the repository has been producing bit-exact output since November 26.

Figure 1	 - Performance of 1500 byte slices compared to TMuC 0.9

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.8 | 3.0 | 3.0 | 4.6 | 4.2 | 4.2 |
| Class B | 4.6 | 4.0 | 4.2 | 5.5 | 5.1 | 5.3 |
| Class C | 5.1 | 4.4 | 4.4 | 5.7 | 5.6 | 5.4 |
| Class D | 3.5 | 3.3 | 3.4 | 4.0 | 3.3 | 3.5 |
| Class E | 7.0 | 6.7 | 5.8 | 7.4 | 9.9 | 9.3 |
| All | 4.8 | 4.3 | 4.2 | 5.4 | 5.5 | 5.5 |
| Enc Time[%] | 115% |  |  | 119% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.7 | 5.6 | 4.8 | 3.8 | 3.9 | 3.5 |
| Class B | 4.2 | 6.3 | 5.9 | 4.3 | 3.8 | 4.0 |
| Class C | 3.8 | 5.0 | 4.8 | 3.9 | 3.2 | 3.1 |
| Class D | 2.4 | 3.2 | 3.2 | 2.4 | 1.0 | 1.6 |
| Class E |  |  |  |  |  |  |
| All | 3.5 | 5.0 | 4.8 | 3.6 | 2.9 | 3.0 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.2 | 5.9 | 3.9 | 3.1 | 1.6 | 0.6 |
| Class C | 2.4 | 3.6 | 3.8 | 2.5 | 0.7 | 0.6 |
| Class D | 1.4 | 1.3 | 2.1 | 1.5 | 0.0 | 0.1 |
| Class E | 0.9 | 0.7 | -0.1 | 0.8 | 3.2 | 1.8 |
| All | 2.1 | 3.2 | 2.7 | 2.1 | 1.3 | 0.7 |
| Enc Time[%] | 100% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

The complexity numbers above come from cluster simulations using different machines and are therefore not reliable.

The decoder complexity was measured on a local machine and it was found that the running times of decoding single slice bitstreams is increased with the slice supporting software. The December 16 code was found to be 0.2-0.3 percent slower while the January 13 code was 1-5 percent slower.

# List of input documents related to this AHG

JCTVC-D127 [Chih-Wei Hsu, Chia-Yang Tsai, Yu-Wen Huang, Ching-Yeh Chen, Chih-Ming Fu, Shawmin Lei] Leaf Coding Unit Aligned Slices

JCTVC-D128 [Chia-Yang Tsai, Chih-Wei Hsu, Yu-Wen Huang, Ching-Yeh Chen, Chih-Ming Fu, Shawmin Lei] Slice Boundary Processing and Picture Layer RBSP

JCTVC-D227 [A. Fuldseth (Cisco)] Replacing slices with tiles for high level parallelism

JCTVC-D302 [Y. Lin, C. Lai, J. Zheng, L. Liu] Intra coding improvements for slice boundary blocks

JCTVC-D312 [Q. Shen, Q. Xie, H. Yu] Fine granularity slice partition

JCTVC-D378 [M. Horowitz, S. Xu (eBrisk Video)] Generalized slices

JCTVC-D383 [F. Bossen (DOCOMO USA Labs)] Simplification of end-of-slice coding for arithmetic coding

JCTVC-D396 [Y. Chen, P. Chen, M. Karczewicz] On Slice Granularity

At least the following input documents reported to have used the ad-hoc group source code:

D004, D127, D128, D227, D302, D378

We therefore believe the source code to have been sufficiently tested.

# Recommendations

It is recommended to integrate the slice source code from TMuC version 0.9 into the HM software and to review the relevant input documents. .