| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D025 WG11 Number: m18764 |
| --- | --- |

| Title: | Evaluation results on IBDI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The IBDI deserves further study as it has significant impact on the implementation cost in terms of area and memory bandwidth. The document reports the IBDI coding efficiency with respect to the number of IBDI extension bits, and IBDI performance when the reference frame is stored in 8-bit instead of 12-bit. Evaluation results reveal that 2-bit IBDI extension is a good trade-off as it already captures 80 – 90% of potential IBDI gain. Tests also show that the IBDI gain almost vanishes when the reference frame storage  is reduced from 12-bit to 8-bit.

# Introduction

In the first test model of HEVC (HM1) the IBDI (internal bit depth increase) was adopted into the high efficiency configurations. The IBDI has significant impact on the implementation cost in terms of area and memory bandwidth. If 4-bit extension is used for the IBDI, it is estimated that the area size and memory bandwidth will grow roughly by 50% for the 8-bit input video. Therefore, the coding efficiency and complexity trade-off should be carefully studied for the IBDI.

The document reports the IBDI coding efficiency with respect to the number of IBDI extension bits, and IBDI performance when the reference frame is stored in 8-bit instead of 12-bit.

# Test Conditions

The TMuC-0.9 software is used for the evaluation, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

To disable the transform precision expansion (TPE) in the high efficiency configurations when the number of IBDI extension bits varies, in the simulation the following constants in CommonDef.h have been modified as follows:

| IBDI bits = 4 | IBDI bits = 3 | IBDI bits = 2 | IBDI bits = 1 | IBDI bits = 0 |
| --- | --- | --- | --- | --- |
| g_iShift8x8 = 4; g_iShift16x16 = 4; g_iShift32x32 = 4; g_iShift64x64 = 4; | g_iShift8x8 = 3; g_iShift16x16 = 3; g_iShift32x32 = 3; g_iShift64x64 = 3; | g_iShift8x8 = 2; g_iShift16x16 = 2; g_iShift32x32 = 2; g_iShift64x64 = 2; | g_iShift8x8 = 1; g_iShift16x16 = 1; g_iShift32x32 = 1; g_iShift64x64 = 1; | g_iShift8x8 = 0; g_iShift16x16 = 0; g_iShift32x32 = 0; g_iShift64x64 = 0; |

# Testing Results

## IBDI coding efficiency w.r.t IBDI extension bits

The IBDI coding efficiency is tested with IBDI extension bits 0, 1, 2, 3 for the high efficiency configurations. Table 1 shows the BD-rate increase compared to the anchor (which has 4 bits IBDI extension) when the IBDI extension bits are reduced. As shown in Table 1, 2-bit IBDI extension seems to be a good trade-off, because it already achieves 80 – 90% of the potential IBDI coding gain.  The table also reveals that IBDI is most effective in class E sequences.

|  | Intra (IBDI bits) |  |  |  | Random access (IBDI bits) |  |  |  | Low-delay (IBDI bits) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| Class A | 0.7 | 0.8 | 0.2 | 0.0 | 2.1 | 1.4 | 0.3 | 0.0 |  |  |  |  |
| Class B | 1.3 | 1.0 | 0.2 | 0.0 | 3.5 | 2.1 | 0.4 | 0.1 | 5.4 | 2.8 | 0.8 | 0.1 |
| Class C | 0.5 | 0.5 | 0.1 | 0.0 | 2.0 | 1.0 | 0.2 | 0.0 | 3.5 | 1.5 | 0.3 | 0.1 |
| Class D | 0.3 | 0.4 | 0.0 | 0.0 | 1.3 | 0.7 | 0.1 | 0.0 | 2.2 | 1.0 | 0.2 | 0.0 |
| Class E | 2.1 | 1.4 | 0.2 | 0.0 |  |  |  |  | 13.4 | 7.2 | 2.3 | 0.5 |
| All | 1.0 | 0.8 | 0.1 | 0.0 | 2.3 | 1.3 | 0.3 | 0.0 | 5.6 | 2.9 | 0.8 | 0.2 |

Table 1 BD-rate increase of Y component (IBDI extension bits = 0, 1, 2, 3 vs. anchor IBDI extension bits = 4)

For detailed results please refer to

TMuC0.9-anchor_vs.IBDI0.xls

TMuC0.9-anchor_vs.IBDI1.xls

TMuC0.9-anchor_vs.IBDI2.xls

TMuC0.9-anchor_vs.IBDI3.xls

## IBDI performance when reference frame is stored in 8-bit

This test is designed to evaluate the IBDI performance when the reference frame is stored in 8-bit instead of 12-bit. The 8-bit reference frame storage is simulated with the following two methods

- Rounding:    pixel = ((pixel + 8) >> 4) <<4

- Truncation:   pixel = (pixel >> 4) <<4

Where pixel is a sample of reference frame.

The BD-rate increase is summarized in Table 2 when the reference frame is stored in 8-bit instead of 12-bit for IBDI. By comparing table 1 (IBDI bits = 0 column) with Table 2, it can be seen that the IBDI provides only slight gain 1.1% in the random access configuration, and loses around 1.8% for the low-delay configuration when rounding method is used.  The IBDI performance further degrades when the truncation method is used, it loses 0.2% in the random access configuration and 6.6% in the low-delay configuration, respectively. Huge loss is seen in both cases for class E sequences.

|  | Rounding |  |  | Truncation |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.3 | 1.8 | 1.9 | 3.2 | 7.2 | 10.3 |
| Class B | 1.6 | 2.3 | 2.7 | 3.4 | 10.6 | 14.5 |
| Class C | 1.0 | 1.1 | 1.1 | 2.0 | 4.1 | 4.4 |
| Class D | 0.6 | 0.6 | 0.4 | 1.7 | 4.3 | 3.9 |
| Class E |  |  |  |  |  |  |
| All | 1.2 | 1.5 | 1.6 | 2.5 | 6.7 | 8.4 |
|  | Low-delay |  |  | Low-delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 4.1 | 3.2 | 4.2 | 7.6 | 10.4 | 13.5 |
| Class C | 2.5 | 3.2 | 3.1 | 4.2 | 5.0 | 5.0 |
| Class D | 1.6 | 8.7 | 9.4 | 3.1 | 6.0 | 5.9 |
| Class E | 27.1 | 6.5 | 15.1 | 42.8 | 44.3 | 50.1 |
| All | 7.4 | 5.2 | 7.3 | 12.2 | 14.3 | 16.3 |

Table 2 BD-rate increase when the reference frame is stored in 8-bit instead of 12-bit for IBDI

For detailed results please refer to

TMuC0.9-anchor_vs.IBDI-RFC12to8_rounding.xls

TMuC0.9-anchor_vs.IBDI-RFC12to8_truncation.xls

# Conclusions

IBDI deserves further study as it has significant impact on the implementation cost. Evaluation results reveal that 2-bit IBDI extension is a good trade-off as it already captures 80 – 90% of potential IBDI gain. The results also show that the IBDI is sensitive to the storage precision of the reference frame, the IBDI gain almost vanishes when the reference frame is stored in 8-bit instead of 12-bit.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.