| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D026 |
| --- | --- |

| Title: | CE6.a.5: Santa Clara University and Hisilicon Report on Block Based Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li (Santa Clara University), Lingzhi Liu (Hisilicon Technologies Co. Ltd.) , Nam Ling (Santa Clara University), Jianhua Zheng (Hisilicon Technologies Co. Ltd.), Philipp Zhang (Hisilicon Technologies Co. Ltd.), | Tel: Email: | +1-408-554-4794 g1li@scu.edu lzliu@huawei.com nling@scu.edu zhengjianhua@huawei.com pzhang@huawei.com |
| Source: | Santa Clara University & Hisilicon Technologies Co. Ltd. |  |  |

_____________________________

Abstract

The Intra Plane Mode and Intra Bilinear Mode are implemented in Unified Intra Prediction (UIP) based on the TMuC version 0.9. Simulations were performed on different combinations of Intra Modes and the results were analyzed.

- Introduction

The integration of Intra Plane mode in UIP with Multiple Predictor Sets (MPS) method was introduced in JCTVC-C111. In this CE, both Plane mode and Bilinear mode are integrated as non-directional modes of UIP. The performance of the new UIP with Plane and Bilinear mode (later called MPS Intra in this document), and the MPS Intra with Planar Intra Prediction enabled were evaluated.

- Implementation

The original UIP has 34 intra prediction modes, 1 DC mode and 33 directional modes. In this experiment, Plane mode and Bilinear mode are implemented as additional non-directional modes. The new list of modes is shown in the following table:

| Mode Number | IntraPredType | IntraPredAngleID |
| --- | --- | --- |
| 0 | Intra_Vertical | 0 |
| 1 | Intra_Horizontal | 0 |
| 2 | Intra_DC | - |
| 3 | Intra_Vertical | -8 |
| 4 | Intra_Vertical | -4 |
| 5 | Intra_Vertical | +4 |
| 6 | Intra_Vertical | +8 |
| 7 | Intra_Horizontal | -4 |
| 8 | Intra_Horizontal | +4 |
| 9 | Intra_Horizontal | +8 |
| 10 | Intra_Vertical | -6 |
| 11 | Intra_Vertical | -2 |
| 12 | Intra_Vertical | +2 |
| 13 | Intra_Vertical | +6 |
| 14 | Intra_Horizontal | -6 |
| 15 | Intra_Horizontal | -2 |
| 16 | Intra_Horizontal | +2 |
| 17 | Intra_Horizontal | +6 |
| 18 | Intra_Vertical | -7 |
| 19 | Intra_Vertical | -5 |
| 20 | Intra_Vertical | -3 |
| 21 | Intra_Vertical | -1 |
| 22 | Intra_Vertical | +1 |
| 23 | Intra_Vertical | +3 |
| 24 | Intra_Vertical | +5 |
| 25 | Intra_Vertical | +7 |
| 26 | Intra_Horizontal | -7 |
| 27 | Intra_Horizontal | -5 |
| 28 | Intra_Horizontal | -3 |
| 29 | Intra_Horizontal | -1 |
| 30 | Intra_Horizontal | +1 |
| 31 | Intra_Horizontal | +3 |
| 32 | Intra_Horizontal | +5 |
| 33 | Intra_Horizontal | +7 |
| 34 | Intra_Plane | - |
| 35 | Intra_Bilinear | - |

Table-1 - Unified Intra Prediction modes after integration.

Multiple Predictor Sets (MPS) method is used for PUs with MaxMode == 36 (size of 8x8, 16x16, and 32x32). 36 intra predictors are grouped into 4 sets, 9 in each. Each mode is represented by Group number and Index number. The basic group, group 0, contains the 9 basic prediction modes as in H.264/AVC. The MPS table is shown in Table-2.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 11 | 16 | 17 | 14 | 10 | 12 | 13 | 15 | 9 |
| 2 | 21 | 30 | 34 | 26 | 19 | 23 | 25 | 28 | 32 |
| 3 | 22 | 29 | 35 | 18 | 20 | 24 | 33 | 27 | 31 |

Table-2 - Unified Intra Prediction modes in Multiple Predictor Sets.

- Testing Conditions

In order to check the performance of the MPS Intra, and the effect of planar intra prediction on MPS Intra, two simulations were conducted.

- Simulation with MPS Intra (Plane mode and Bilinear mode integrated);

- Simulation with MPS Intra, with planar intra prediction enabled.

Each simulation was performed in Intra only mode, with both High Efficiency and Low Complexity configurations.

The tests were performed on a Windows 7 Professional 64bit platform, and the software was compiled in 64bit mode.

- Results

  - BD-rate performance

- Simulation 1’s results are shown in Table-3:

| TEST#1 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.2 | -0.5 | -0.3 | -0.4 |
| Class B | -0.2 | -0.2 | -0.3 | -0.5 | -0.4 | -0.4 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.2 |
| Class E | -0.4 | -0.4 | -0.5 | -0.8 | -0.8 | -0.6 |
| All | -0.2 | -0.2 | -0.2 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

Table 3 - UIP with MPS Intra compared with Anchor.

- Simulation 2’s results are shown in Table-4:

| TEST#2 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.1 | 0.7 | 0.6 | 0.4 |
| Class B | -0.2 | -0.3 | -0.3 | 0.7 | 0.5 | 0.3 |
| Class C | 0.0 | -0.1 | -0.1 | 1.1 | 0.8 | 0.7 |
| Class D | 0.0 | 0.0 | 0.0 | 1.0 | 0.7 | 0.6 |
| Class E | -0.4 | -0.3 | -0.5 | 0.7 | 0.1 | 0.4 |
| All | -0.1 | -0.2 | -0.2 | 0.9 | 0.6 | 0.5 |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

Table 4 - UIP with MPS Intra and Planar Intra Prediction enabled compared with Anchor.

The simulation of modified UIP with MPS Intra shows that coding gain is relatively more significant on sequences with higher resolution, such as 720P, 1080P, and 2K. The gain is higher in low complexity case.

According to Table 4, enabling intra planar prediction along with the modified UIP introduces no extra gain, but performance loss.

  - Statistics of UIP predictor usage

| Rank | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mode | 2 | 0 | 1 | 16 | 34 | 11 | 8 | 15 |
| Usage | 13.07% | 10.47% | 8.90% | 6.24% | 4.77% | 4.27% | 4.13% | 3.38% |
| Rank | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Mode | 7 | 5 | 4 | 12 | 30 | 14 | 21 | 10 |
| Usage | 3.37% | 3.07% | 3.06% | 3.02% | 2.86% | 2.84% | 2.65% | 2.49% |
| Rank | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Mode | 3 | 13 | 29 | 6 | 31 | 22 | 9 | 33 |
| Usage | 1.99% | 1.91% | 1.89% | 1.67% | 1.66% | 1.62% | 1.60% | 1.10% |
| Rank | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| Mode | 17 | 23 | 28 | 25 | 20 | 32 | 27 | 26 |
| Usage | 1.07% | 0.94% | 0.87% | 0.76% | 0.75% | 0.68% | 0.65% | 0.60% |
| Rank | 33 | 34 | 35 | 36 |  |  |  |  |
| Mode | 19 | 24 | 18 | 35 |  |  |  |  |
| Usage | 0.56% | 0.55% | 0.53% | 0.03% |  |  |  |  |

Table 5 - UIP Mode Usage on 1080P sequences.

The percentage of each mode used in the coding process is recorded. The statistics of the mode usage on all 1080P sequences in the test is shown in Table 5. The table is sorted by the usage of each mode. It shows that plane mode has very high usage compared to other directional modes. It is the 5th most selected intra mode. According to our testing results, the ranking is similar for 2K and 720P sequences.

The usage of bilinear mode is extremely low, which means that it does not have advantage over other intra modes in UIP. From the data of mode usage, it is clear that most of the gain was contributed by the Plane mode.

  - Subjective Visual Quality

Figure-1 Subjective Visual Quality comparison – Cactus (QP 32).

Figure-2 Subjective Visual Quality comparison – BQTerrace (QP 32).

Figure 1 and Figure 2 are the comparison of visual qualities of decoded sequences. Visual quality is better on the decoded sequences by modified UIP with extra modes, especially in areas with luminance gradients, the pictures look much smoother. Along with the mode usage statistics, it is clear that the visual quality improvement is also contributed by the plane mode.

- Conclusion

MPS Intra performs better than TMuC v0.9 Anchor, on both BD-rate and subjective visual quality. The major contribution of the gain is from the Intra Plane mode. Integrating Plane mode with MPS in UIP can improve the performance.

The Bilinear mode is rarely used. It is not necessary to have this mode for intra prediction.

Enabling Planar Intra Prediction introduces coding loss, it is not necessary either.

- Reference

- Guichun Li, Lingzhi Liu, Nam Ling, Jianhua Zheng, and Philipp Zhang, “Integration of Plane Mode in Unified Intra Prediction”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C111, Guangzhou, China, Oct. 2010.

- Lingzhi Liu, “Multiple Predictor Sets for Intra Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-A022E, April 2010.

- JCT-VC , “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC), of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B100 July, 2010.

- Jung-Hye Min, Sunil Lee, Il-Koo Kim, Woo-Jin Han, Jani Lainema, Kemal Ugur, “Unification of the Directional Intra Prediction Methods in TMuC”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B100, July 2010.

- JCT-VC , “Test Model under Consideration”, Joint Collaborative Team on Video Coding (JCT-VC), of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B205 July, 2010.

- Patent rights declaration(s)

Santa Clara University and Hisilicon Technologies Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).