| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D027 WG11 Number: m18766 |
| --- | --- |

| Title: | List of items requiring clarification/action in HM1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Osamoto Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-4969 Osamoto@ti.com +1-214-480-3816 zhou@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the number of inconsistencies and/or illogical procedures found in TMuC 0.9 software and HM1. These items should be clarified and/or studied.

# Introduction

This document reports the number of inconsistencies and/or illogical procedures found in TMuC 0.9 software and HM1. These items should be clarified and/or studied.

# Items to be Clarified / Studied

- CABAC Binarization of refidx

- Chroma boundary strength calculation for Deblocking filter

- Weak deblocking filter operation for p1 and q1

- Chroma intra prediction for chroma intra prediction mode of 4

- Reference frame management and reference list generation

- Inconsistent order of CABAC sub-block coding

## CABAC Binalization of refidx

ref_idx_l0 / ref_idx_l1 uses the truncated unary which the maximum value is “number of refIdx” - 1. In the current TMuC 0.9 implementation (TDecSbac::parseRefFrmIdx()), if the number of refIdx is 2, it uses (non-truncated) unary. So, 0b is used for refIdx = 0, and 10b is used for refIdx = 1. It is a waste of bits.

Suggested options to change:

- Don’t send the second bin if the number of reference frames is 2.

### Experiment Setup

To avoid sending second bin when number of reference frames is 2, add the following

In encoder, TEncSbac::codeRefFrmIdx:

if (( iRefFrame > 0 ) && (pcCU->getSlice()->getNumRefIdx( eRefList )>2)) { xWriteUnaryMaxSymbol( iRefFrame - 1, &m_cCURefPicSCModel.get( 0, 0, 4 ), 1, pcCU->getSlice()->getNumRefIdx( eRefList )-2 ); } return;

In decoder, TDecSbac::parseRefFrmIdx

if ( uiSymbol && (pcCU->getSlice()->getNumRefIdx( eRefList )>2)) { xReadUnaryMaxSymbol( uiSymbol, &m_cCURefPicSCModel.get( 0, 0, 4 ), 1, pcCU->getSlice()->getNumRefIdx( eRefList )-2 ); uiSymbol++; }

### Results

Average Bin Reduction (compared with TMuC-0.9 anchor)

|  | Reduction in refIdx bins | Reduction in overall bins |
| --- | --- | --- |
| Random Access: | 30.07%; | 1.22% |
| Low delay | 21.27% | 1.6% |

Coding Efficiency (compared with TMuC-0.9 anchor)

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr |
| Class A | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.2 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | -0.1 |

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr |
| Class A |  |  |  |
| Class B | 0.0 | 0.1 | -0.1 |
| Class C | 0.0 | 0.0 | -0.2 |
| Class D | 0.0 | -0.6 | 0.5 |
| Class E | -0.2 | -0.1 | 0.1 |
| All | 0.0 | -0.1 | 0.1 |

For detailed result please refer to TMuC-0.9-refIdxTU.xls.

## Chroma boundary strength calculation for Deblocking filter

TMuC 0.9 has the following implementation for the derivation of chroma filter boundary strength:

If CU size is 8, it uses not-prepared boundary strengths for right CU to determine if a 8-pixel chroma segment should be filtered. This effectively ignores the right CU’s intra flag, and filter flag is determined only by the current and left or top neighbour’s intra flag. For example, if CU A1 and B1 in the following figure are inter-coded, the chroma boundary between A1 and B1 as well as A2 and B2 are NOT filtered even if CU A2 and/or B2 is intra-coded.

In TComLoopFilter::xEdgeFilterChroma()

for (Int iIdxInside = 0; iIdxInside<PartIdxIncr; iIdxInside++) { uiBsAbsIdx = xCalcBsIdx( pcCU, uiAbsZorderIdx, iDir, iEdge, iIdx+iIdxInside); if (ucBs < m_aapucBS[iDir][0][uiBsAbsIdx]) { ucBs = m_aapucBS[iDir][0][uiBsAbsIdx]; } }

For example, if this is for horizontal boundary and uiAbsZorderIdx is 0, this code checks m_aapucBS[iDir][0][uiBsAbsIdx] with uiBsAbsIdx of 0, 1, 4 and 5 for horizontal filtering.

0   1   4   5

2   3   6   7

8   9  12  13

10  11  14  15

But m_aapucBS for 4 and 5 are not calculated yet (because they belong to next CU), and always have 0 (initial value). If block 0 and 1 are not on an intra boundary, the horizontal boundary of 0, 1, 4 and 5 are not filtered even if block 4 and 5 are on an intra boundary.

Suggested options to change:

- Modify the procedure so that the chroma filter is applied in unit of 4-chroma pixels if the CU size is 8 (e.g. A1/B1 boundary in the above example depends on intra-flag of A1/B1, and A2/B2 boundary depends on A2/B2) (this can reduce the complexity of software / hardware because the boundary strength calculation can be localized to CU),

- Calculate m_aapucBS for all CU’s in the current LCU before applying the (chroma) filter, or

- Leave it as is.

## Weak deblocking filter operation for p1 and q1

Current definition uses the division by 2 for the update of p1 and q1 in the weak filter.

p1i = Clip(p1i + D/2)

q1i = Clip(q1i - D/2)

Suggested options to change:

- Use “>> 1” instead of “/2” for simpler hardware implementation.

## Chroma intra prediction for chroma intra prediction mode of 4

Chroma intra prediction uses the luma prediction mode of PU partition #0 when the chroma mode is set to 4, even when PU is partitioned into NxN. For example, let's consider a CU uses the PU partition of NxN and its luma intra prediction is 8, 2, 16 and 31 for each PU partition. If chroma prediction mode is 4 for this CU, all of chroma TU partitions use the intra prediction mode of 8. It is more logical if chroma blocks in PU partition #1 uses mode 2, #2 uses mode 16, ... This won’t increase the encoder / decoder complexity.

if( uiChromaPredMode == 4 ) { UInt uiIntraIdx = pcCU->getIntraSizeIdx( 0 ); uiChromaPredMode = pcCU->angIntraEnabledPredPart( 0 ) ? pcCU->getLumaIntraDir( 0 ) : g_aucIntraModeOrder[ uiIntraIdx ][ pcCU->getLumaIntraDir( 0 ) ]; }

Suggested options to change:

- Match the actual chroma prediction mode to the corresponding Luma prediction mode (quality study needed)

## Reference image management and reference list generation

TMuC0.9 implements different algorithms from H.264 standard, while the TMuC document describes it is the same as H.264. The precise and detail description should be added.

DPB management in TMuC 0.9:

- Size of the DPB is dynamically updated based on the POC of the first P or B-slice in the sequence and num_ref_idx_l0_active_minus1 of each slice.

- A slot for a reconstructed image is reserved before decoding (H.264 adds a reconstructed image to DPB after reconstruction). If there is no available slot in DPB, the image that has the minimum POC is removed from the DPB.

- Reconstructed images are sent to output based on their POC (not by “bumping” process). In this process, it is assumed that POC is incremented by one in the display order.

Reference list generation in TMuC 0.9:

- Only one long-term reference is added to each reference list, which is selected based on POC. (H.264 allows multiple long-term references in a list.)

- The long-term reference is the first or second element of reference list (if any), depending on its POC. (Long-term references follow short-term references in a list in case of H.264.)

## Inconsistent order of CABAC sub-block coding

In the CABAC coding process, the TU partition is divided into 4x4 sub-blocks for abs_greater_one, coef_abs_level and sign syntax elements. The scan order of the 4x4 sub-blocks are defined as follows:

Only TU partition size of 8 uses the “transposed zig-zag scan order”.

if ( uiSubLog2 > 1 ) { uiSubPosX = g_auiFrameScanX[ uiSubLog2 - 1 ][ uiSubBlk ] * 4; uiSubPosY = g_auiFrameScanY[ uiSubLog2 - 1 ][ uiSubBlk ] * 4; } else { uiSubPosX = ( uiSubBlk < 2 ) ? 0 : 1; uiSubPosY = ( uiSubBlk % 2 == 0 ) ? 0 : 1; uiSubPosX *= 4; uiSubPosY *= 4; }

Suggested options to change:

- If this is NOT based on some experimental results that show its efficacy, use the ordinary zig-zag scan order for all TU partition size (removing italic part above) in order to avoid unnecessary complexity and confusion.

# References

- JVT-VC, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405r1, 3rd JCTVC Meeting, Guangzhou, CN, October 2010

- JCT-VC, “Test Model under Consideration”, JCTVC-B205_draft007, 2nd JCT-VC Meeting, Geneva, CH, July 2010

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.

# Patent rights declarations

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Additional statement: to the best of our personal awareness TI does not have patent rights to the content of the document.