| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D031 WG11 Number: m18774 |
| --- | --- |

| Title: | CE7: Cross-check for Samsung’s Proposal on Jointly Optimal Intra Prediction and Adaptive Primary Transform by BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-Check |  |  |
| Author(s) or Contact(s): | Ying Weng Thomas Davies BBC Research & Development 56 Wood Lane London, W12 7SB, United Kingdom | Tel: Email: Tel: Email: | +44-3030409640 ying.weng@bbc.co.uk +44-3030409615 Thomas.davies@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results of “Jointly optimal intra prediction and adaptive primary transform” proposed by Samsung [1]. Both theoretical verification and experimental verification have been implemented: (1) The theoretical cross-check results are: One equation has been found with errors, and two equations have been found wrong. Consequently, the rectifications of these three equations have been done; (2) The experimental cross-check results agree with the overall performance as reported by Samsung based on the source codes provided.

# Theoretical verification

In order to cross-check the algorithm description [1], all the mathematical expressions have been derived. There are several minor errors in the analysis presented in [1], specifically:

(1) In the second-last line, page 5, [1]:

RCol,ik=E[xijxkj+x0(j+idxdy)x0(j+kdxdy)−xijx0(j+idxdy)−xkjx0(j+kdxdy)]

should read:

RCol,ik=E[xijxkj+x0(j+idxdy)x0(j+kdxdy)−xijx0(j+kdxdy)−xkjx0(j+idxdy)]

(2) In the 5th line, page 7, [1]:

Rrow,mn=ρ|m−n|+ρ|(m−n)dxdy|−ρi+|m−n+idxdy|−ρi+|m−n−idxdy|

should read:

Rrow,mn=2ρ|m−n|−ρi+|m−n+idxdy|−ρi+|m−n−idxdy|

(3) Consequently from (2), the 8th line, page 7, [1]:

Rrow,mn=c.limρ→1Rrow,mn1−ρ=c.(−1).[|m−n|+|(m−n)dxdy|−(i+|m−n+idxdy|)−(i+|m−n−idxdy|)c.[2i+|m−n+idxdy|+|m−n−idxdy|−|m−n|]

should read:

Rrow,mn=c.limρ→1Rrow,mn1−ρ=c.(−1).[2|m−n|−(i+|m−n+idxdy|)−(i+|m−n−idxdy|)]c.[2i+|m−n+idxdy|+|m−n−idxdy|−2|m−n|]

However the resulting matrix is still Toeplitz and the conclusions of the analysis still seem to be correct.

# Experimental verification

Experimental verification has been conducted under the test conditions required by [2]. The cross-checking has been done for intra and random access high efficiency and low complexity configurations.

Three tests were conducted:

- Apply the adaptive DST/DCT at block sizes 4x4 and 8x8 only;

- Apply the adaptive DST/DCT at all block sizes

- As A), but use the built-in DCT from TMuC rather than a matrix-multiply version

## Simulation environment

The source code has been compiled and run on a Linux Debian 64bit cluster, with gcc version 4.4.5. The cluster nodes use uniform quad-core Intel X3450 2.66GHz processors, 8GB RAM, running a process per core on average.

# Summary results

## 4x4 and 8x8 only

The experimental cross-check results agree at least up to minor differences. The decoding times appear to be slightly worse compared to the reference. Table 1 provides the summary results for this test.

Table 1: Adaptive DCT/DST for 4x4 and 8x8 blocks only

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | -2.4 | -2.4 | -3.0 | -1.5 | -1.1 |
| Class B | -0.8 | -1.1 | -1.1 | -1.5 | -1.1 | -1.0 |
| Class C | -1.1 | -1.1 | -1.1 | -1.7 | -1.2 | -1.2 |
| Class D | -1.2 | -1.1 | -1.1 | -1.7 | -1.1 | -1.1 |
| Class E | -1.4 | -1.7 | -1.6 | -2.5 | -0.6 | -1.0 |
| All | -1.2 | -1.3 | -1.3 | -1.9 | -1.1 | -1.1 |
| Enc Time[%] | 109% |  |  | 118% |  |  |
| Dec Time[%] | 111% |  |  | 122% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.7 | -0.6 | -1.0 | -0.3 | 0.0 |
| Class B | -0.5 | -0.5 | -0.4 | -0.5 | -0.4 | -0.4 |
| Class C | -0.6 | -0.6 | -0.6 | -0.6 | -0.3 | -0.3 |
| Class D | -0.5 | -0.5 | -0.3 | -0.5 | -0.3 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.5 | -0.5 | -0.6 | -0.3 | -0.3 |
| Enc Time[%] | 106% |  |  | 103% |  |  |
| Dec Time[%] | 108% |  |  | 105% |  |  |

## All block sizes

The results for adaptive DST/DCT agree at least up to minor differences with those provided by Samsung. For Intra coding, the encoding and decoding times are significantly longer than for the anchors, especially in Intra Low Complexity configuration.

Table 1: Adaptive DCT/DST for all block sizes

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.4 | -2.8 | -3.0 | -3.5 | -2.6 | -2.6 |
| Class B | -1.0 | -1.7 | -1.8 | -1.7 | -1.9 | -1.9 |
| Class C | -1.2 | -1.4 | -1.4 | -1.8 | -1.6 | -1.6 |
| Class D | -1.2 | -1.2 | -1.2 | -1.7 | -1.3 | -1.3 |
| Class E | -1.5 | -2.9 | -2.8 | -2.7 | -2.8 | -2.9 |
| All | -1.3 | -1.9 | -1.9 | -2.1 | -2.0 | -2.0 |
| Enc Time[%] | 116% |  |  | 147% |  |  |
| Dec Time[%] | 123% |  |  | 146% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.8 | -0.8 | -1.2 | -0.5 | -0.5 |
| Class B | -0.6 | -1.0 | -0.9 | -0.7 | -0.6 | -0.7 |
| Class C | -0.7 | -0.7 | -0.6 | -0.6 | -0.5 | -0.4 |
| Class D | -0.6 | -0.4 | 0.0 | -0.5 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.7 | -0.6 | -0.7 | -0.5 | -0.5 |
| Enc Time[%] | 103% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 109% |  |  |

## 4x4 and 8x8 using original TMuC DCT

This implementation uses the built-in DCT available in TMuC rather than the matrix multiplication version used in the other simulations. The results agree at least up to up to minor differences with those provided by Samsung. The summary results are as in Table 3.

Table 3: 4x4 and 8x8 adaptive DST/DCT using built-in DCT

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | -2.3 | -2.4 | -3.0 | -1.5 | -1.1 |
| Class B | -0.8 | -1.1 | -1.1 | -1.5 | -1.1 | -1.0 |
| Class C | -1.1 | -1.1 | -1.1 | -1.7 | -1.2 | -1.2 |
| Class D | -1.2 | -1.1 | -1.1 | -1.7 | -1.1 | -1.1 |
| Class E | -1.4 | -1.7 | -1.6 | -2.5 | -0.6 | -1.0 |
| All | -1.2 | -1.3 | -1.3 | -1.9 | -1.1 | -1.1 |
| Enc Time[%] | 106% |  |  | 109% |  |  |
| Dec Time[%] | 106% |  |  | 112% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.5 | -0.4 | -1.1 | -0.3 | 0.1 |
| Class B | -0.5 | -0.3 | -0.2 | -0.5 | -0.4 | -0.4 |
| Class C | -0.6 | -0.5 | -0.5 | -0.6 | -0.3 | -0.3 |
| Class D | -0.5 | -0.5 | -0.4 | -0.5 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.4 | -0.4 | -0.6 | -0.4 | -0.3 |
| Enc Time[%] | 104% |  |  | 98% |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |

# Conclusions and recommendations

The results for all configurations are consistent with those provided by Samsung in terms of bit rate and measured PSNR.

The tool gives good gain and should continue to be under consideration for HEVC. Measured complexity in terms of encoding time appears to be similar to the anchors for the 4x4 and 8x8 only configurations. However, the form of the DST used does not have a straightforward factorization and would require matrix multiplication. Further experimentation could be conducted to determine whether other DST forms would be equally effective; and also how well the approach compares or combines with other mode-adaptive approaches, such as Adaptive Coefficient Scanning.

# References

[1] A. Saxena and F. C. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” JCTVC-C108, Guangzhou, China, October 2010.

[2]	R. Cohen, C. Yeo, and R. Joshi, “CE7: Alternative Transform,” JCTVC-C507, Guangzhou, China, October 2010.

British Broadcasting Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).