| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D032 |
| --- | --- |

| Title: | CE 7: Cross-Check for Toshiba’s proposal on 1-D Directional Unified Transform by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics, Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution summarizes Samsung’s cross-check for Toshiba’s proposal on 1-Dimensional Directional Unified Transform in Core Experiment 7 on Alternate Transforms. The results from Samsung’s simulations closely match with the results provided by Toshiba.

# Introduction

Toshiba has proposed 1-Dimensional Directional Unified Transform (1D-DUT) as a spatial transform scheme for coding intra-prediction residues [2]. Specifically, after intra prediction, the conventional DCT or a KLT, derived using training, and termed as 1D-DUT by Toshiba is applied. The DCT or KLT transforms are applied only at block sizes 4*4 and 8*8, while for higher block sizes, DCT is used.

Toshiba has a DCT implementation similar to proposed in [2] that has minor differences with the Integer based approximate DCT being used in TMuC 0.9. The KLT and DCT proposed by Toshiba use the quantization tables associated with MDDT [3]. For the regular DC mode in intra prediction, TMuC 0.9’s default integer-based approximate DCT, quantization and scaling matrices are used.

Additionally 9 different initial scan orders for block sizes 4*4 and 8*8 have been used for the transform coefficient scanning. This may require additional storage and hardware implementation for the various scan orders.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [1] were followed for conducting the simulations. Tests were conducted on Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity configurations.

## R-D Results

Table 1 shows the simulation results for Toshiba’s proposal as cross-checked by Samsung. Detailed results are available in the attached Excel sheet. The test results matched closely with those provided by Toshiba.

Table 1: Simulation Results for Toshiba’s proposal on 1-d DUT in Core Experiment, as cross-checked by Samsung

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.70 | -2.97 | -2.86 | -2.97 | -1.35 | -0.94 |
| Class B | -1.36 | -1.76 | -1.69 | -1.66 | -1.58 | -1.51 |
| Class C | -2.19 | -1.96 | -1.94 | -2.37 | -1.87 | -1.91 |
| Class D | -2.18 | -1.93 | -1.92 | -2.15 | -1.86 | -1.84 |
| Class E | -2.84 | -2.74 | -2.45 | -3.12 | -0.91 | -1.50 |
| All | -2.12 | -2.14 | -2.05 | -2.31 | -1.57 | -1.60 |
| Enc Time[%] | 109% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 111% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.16 | -0.51 | -0.20 | -1.03 | -0.18 | 0.04 |
| Class B | -0.77 | -0.53 | -0.55 | -0.63 | -0.48 | -0.51 |
| Class C | -0.99 | -0.62 | -0.61 | -0.88 | -0.68 | -0.59 |
| Class D | -0.95 | -0.72 | -0.33 | -0.66 | -0.37 | -0.19 |
| Class E |  |  |  |  |  |  |
| All | -0.93 | -0.60 | -0.46 | -0.76 | -0.47 | -0.37 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed KLT for size 4*4 and 8*8 appears to be derived using a training set. The transform matrices are similar to Toshiba’s previous proposal [2].

## Complexity

The proposed KLT is implemented using a full matrix multiplication. The DCT used by Toshiba is exact DCT and has minor differences with the integer-based approximation of DCT that is being used in TMuC 0.9, and has a hardware-friendly implementation. In terms of bit-shift, quantization and scaling matrices, Toshiba’s DCT and KLT default to MDDT settings. So two different sets of quantization tables are kept at block sizes 4*4 and 8*8: the first corresponds to MDDT, and the other corresponds to integer-based approximate DCT being used as default in TMuC 0.9.

Additionally 9 different scan orders have been used at each size 4*4 and 8*8 for the transform coefficient scanning. These scan orders are based on the intra-prediction mode and a look-up table is used to map the 34 unified intra prediction modes to these set of 9 scans.

# Conclusions

We have cross-checked Toshiba’s proposal on 1-d Directional Unified Transform [2], as part of Core Experiment 7, studied their algorithms and verified the simulation results.

# References

1. R. Cohen, C. Yeo and R. Joshi, “Description of Core Experiment 7: Alternate Transforms,” ITU-T JCTVC-C507, Guangzhou, China, October 2010.

2. A. Tanizawa, J. Yamaguchi, T. Shiodera, T.Chujoh and T.Yamakage, “ One dimensional directional unified transform,” ITU-T JCTVC-C080, Guangzhou, China, October 2010.

3. Y. Ye and M. Karczewicz, “Improved Intra coding,” ITU-T Q.6/SG-16 VCEG, VCEG-AG11, Shenzhen, China, October 2007.