| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D033 |
| --- | --- |

| Title: | CE7: Mode-dependent DCT/DST for intra prediction in video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics, Co., Ltd. |  |  |

_____________________________

# Abstract

It was previously shown that following intra prediction, the optimal transform is a Discrete Sine Transform (DST) with performance close to KLT, along the direction of prediction. In this contribution, we use the conventional Discrete Cosine Transform (DCT), or DST (separably along the vertical or horizontal directions) based on the intra prediction direction. The proposed DCT/DST transform scheme does not require any additional signaling information or Rate-Distortion search during encoding, and works in a single-pass. No training was required to derive the transform and the scheme requires the storage of only one DST matrix in addition to the conventional DCT at each block size. The conventional quantization tables for TMuC 0.9 are retained and no changes have been made to the scanning order. Experimental results are provided with TMuC 0.9 as anchor for the test conditions as stipulated in Core Experiment 7, and average BD Rate gains of 1.3 %, 2.1 %, 0.6 % and 0.7 % are respectively obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency and Random Access Low Complexity settings.

# Introduction

At the JCT-VC meeting in Guangzhou, China in October 2010, Core Experiment 7 [6] was established to evaluate the performance of alternative transforms other than the conventional DCT. In our proposal [5] at Guangzhou, we proposed using either the conventional DCT or a DST for the intra prediction residue based on the prediction mode. It was shown by Han et al. in [2], that following intra prediction, the optimal transform for vertical (respectively horizontal) modes along the vertical (respectively horizontal) direction of prediction is the following DST:

[TS]i,j=22N+1sin(2i−1)jπ2N+1

with performance close to KLT (In the above DST equation, the block size is N*N, and i,j are the row and column indices of the 2-d DST matrix). In [5], we showed that the above DST is the optimal transform for a class of oblique modes in unified intra prediction. We also showed how to choose between the transforms: the conventional DCT and the DST per direction based on the intra prediction mode in unified intra prediction [4]. An independent study, where DST was shown to be the optimal transform for intra-prediction residues was also performed in [7, 8].

In this contribution, we present our experimental results for the stipulated test conditions in [6].

# Mapping from Unified Intra Directions to DCT/DST

We use the following mapping for choosing the transforms: DCT or DST along the vertical or horizontal direction, based on the mode in unified intra prediction:

Table 1: Mapping from intra prediction modes in to DCT/DST along the vertical and horizontal directions

|  | Mode | Vertical Transform | Horizontal Transform |
| --- | --- | --- | --- |
| 0 | VER | DST | DCT |
| 1 | HOR | DCT | DST |
| 2 | DC | DCT | DCT |
| 3 | VER-8 | DST | DST |
| 4 | VER-4 | DST | DST |
| 5 | VER+4 | DST | DCT |
| 6 | VER+8 | DST | DCT |
| 7 | HOR-4 | DST | DST |
| 8 | HOR+4 | DCT | DST |
| 9 | HOR+8 | DCT | DST |
| 10 | VER-6 | DST | DST |
| 11 | VER-2 | DST | DCT |
| 12 | VER+2 | DST | DCT |
| 13 | VER+6 | DST | DCT |
| 14 | HOR-6 | DST | DST |
| 15 | HOR-2 | DCT | DST |
| 16 | HOR+2 | DCT | DST |
| 17 | HOR+6 | DCT | DST |
| 18 | VER-7 | DST | DST |
| 19 | VER-5 | DST | DST |
| 20 | VER-3 | DST | DST |
| 21 | VER-1 | DST | DCT |
| 22 | VER+1 | DST | DCT |
| 23 | VER+3 | DST | DCT |
| 24 | VER+5 | DST | DCT |
| 25 | VER+7 | DST | DCT |
| 26 | HOR-7 | DST | DST |
| 27 | HOR-5 | DST | DST |
| 28 | HOR-3 | DST | DST |
| 29 | HOR-1 | DCT | DST |
| 30 | HOR+1 | DCT | DST |
| 31 | HOR+3 | DCT | DST |
| 32 | HOR+5 | DCT | DST |
| 33 | HOR+7 | DCT | DST |

# Simulation Results

The test conditions in [6] were used for the experiments. The anchor was TMuC 0.9 with the conventional DCT, and the performance of the proposed mode-dependent DCT/DST transform scheme was evaluated for the following 4 settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment.

## R-D Results

We present here the results for the following 3 set of experiments that we performed:

1. Proposed mode-dependent DCT/DST scheme was applied at all block sizes 4*4, 8*8, 16*16 and 32*32. The DCT and DST in mode-dependent DCT/DST scheme were performed using matrix multiplications. However, no new kernel for horizontal and vertical DCT was introduced and the same coefficient values for DCT kernels in TMuC were retained.

2. Proposed mode-dependent DCT/DST scheme was applied only at block sizes 4*4 and 8*8. For higher block sizes 16*16 and 32*32, the conventional DCT in TMuC was always used.  The DCT and DST in mode-dependent DCT/DST scheme were performed using matrix multiplications. Here also, no new kernels for horizontal and vertical DCT was introduced and the same coefficient values for DCT kernels as in TMuC were retained.

3. Proposed mode-dependent DCT/DST scheme was again applied only at block sizes 4*4 and 8*8. For higher block sizes 16*16 and 32*32, the conventional DCT was always used. Only the DST in mode-dependent DCT/DST scheme was performed using matrix multiplications. For the DCT, the conventional TMuC implementation was retained in the mode-dependent DCT/DST as well.

In all the set of experiments, TMuC 0.9 was anchor. Table 2 provides the average R-D results for all the classes A to E for the first set of experiments. Here in the Intra HE and LC settings, BD Rate gains of 1.3 % and 2.1 % for Luma are obtained, while for Random Access High Efficiency and Low Complexity settings, BD Rate gains of 0.6 % and 0.7 % for Luma are obtained respectively. The attached excel file JCTVC-D033_DST_ALL_SIZES.xls   for the contribution contains full results for all the video sequences.

Table 2: DCT/DST applied at block sizes 4*4, 8*8, 16*16 and 32*32

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.4 | -2.8 | -3.0 | -3.5 | -2.6 | -2.6 |
| Class B | -1.0 | -1.7 | -1.8 | -1.7 | -1.9 | -1.9 |
| Class C | -1.2 | -1.4 | -1.4 | -1.8 | -1.6 | -1.6 |
| Class D | -1.2 | -1.2 | -1.2 | -1.7 | -1.3 | -1.3 |
| Class E | -1.5 | -2.9 | -2.8 | -2.7 | -2.8 | -2.9 |
| All | -1.3 | -1.9 | -1.9 | -2.1 | -2.0 | -2.0 |
| Enc Time[%] | 113% |  |  | 121% |  |  |
| Dec Time[%] | 117% |  |  | 127% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.8 | -0.8 | -1.2 | -0.5 | -0.5 |
| Class B | -0.6 | -1.0 | -0.9 | -0.7 | -0.6 | -0.7 |
| Class C | -0.7 | -0.7 | -0.6 | -0.6 | -0.5 | -0.4 |
| Class D | -0.6 | -0.4 | 0.0 | -0.5 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.7 | -0.6 | -0.7 | -0.5 | -0.5 |
| Enc Time[%] | 105% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 109% |  |  |

Table 3 provides the average R-D results for all the classes A to E when proposed DCT/DST scheme is applied at only block sizes 4*4 and 8*8. For higher block sizes: 16*16 and 32*32, the conventional DCT is always used. Here in the Intra High Efficiency and Low Complexity settings, BD Rate gains of 1.2 % and 1.9 % for Luma are obtained, while for Random Access High Efficiency and Low Complexity settings, BD Rate gains of 0.6 % and 0.6 % for Luma are obtained respectively. The attached excel file JCTVC-D033_DST_SIZE_4_8.xls for the contribution contains full results for all the video sequences.

Table 3: DCT/DST applied at block sizes 4*4 and 8*8. For block sizes 16*16 and 32*32, conventional DCT is always used. DCT operations are carried out via matrix Implementation.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | -2.4 | -2.4 | -3.0 | -1.5 | -1.1 |
| Class B | -0.8 | -1.1 | -1.1 | -1.5 | -1.1 | -1.0 |
| Class C | -1.1 | -1.1 | -1.1 | -1.7 | -1.2 | -1.2 |
| Class D | -1.2 | -1.1 | -1.1 | -1.7 | -1.1 | -1.1 |
| Class E | -1.4 | -1.7 | -1.6 | -2.5 | -0.6 | -1.0 |
| All | -1.2 | -1.3 | -1.3 | -1.9 | -1.1 | -1.1 |
| Enc Time[%] | 103% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 107% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.7 | -0.6 | -1.0 | -0.3 | 0.0 |
| Class B | -0.5 | -0.5 | -0.4 | -0.5 | -0.4 | -0.4 |
| Class C | -0.6 | -0.6 | -0.6 | -0.6 | -0.3 | -0.3 |
| Class D | -0.5 | -0.5 | -0.3 | -0.5 | -0.3 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.5 | -0.5 | -0.6 | -0.3 | -0.3 |
| Enc Time[%] | 105% |  |  | 98% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

Table 4 provides the average R-D results for all the classes A to E when proposed DCT/DST scheme is applied at only block sizes 4*4 and 8*8.  For higher block sizes: 16*16 and 32*32, the conventional DCT is always used. Even for sizes 4*4 and 8*8, the DCT part of mode-dependent DCT/DST is carried out using TMuC implementation. Only the DST at sizes 4*4 and 8*8 is carried out via matrix multiplications.

Here in the Intra High Efficiency and Low Complexity settings, BD Rate gains of 1.2 % and 1.9 % for Luma are obtained, while for Random Access High Efficiency and Low Complexity settings, BD Rate gains of 0.6 % and 0.6 % for Luma are obtained respectively. The attached excel file JCTVC-D033_DST_SIZE_4_8_DCT_TMuC.xls for the contribution contains full results for all the video sequences.

Table 4: DCT/DST applied at block sizes 4*4 and 8*8. For block sizes 16*16 and 32*32, conventional DCT is always used. DCT operations are carried out via TMuC Implementation.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | -2.3 | -2.4 | -3.0 | -1.5 | -1.1 |
| Class B | -0.8 | -1.1 | -1.1 | -1.5 | -1.1 | -1.0 |
| Class C | -1.1 | -1.1 | -1.1 | -1.7 | -1.2 | -1.2 |
| Class D | -1.2 | -1.1 | -1.1 | -1.7 | -1.1 | -1.1 |
| Class E | -1.4 | -1.7 | -1.6 | -2.5 | -0.6 | -1.0 |
| All | -1.2 | -1.3 | -1.3 | -1.9 | -1.1 | -1.1 |
| Enc Time[%] | 104% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.5 | -0.4 | -1.1 | -0.3 | 0.1 |
| Class B | -0.5 | -0.3 | -0.2 | -0.5 | -0.4 | -0.4 |
| Class C | -0.6 | -0.5 | -0.5 | -0.6 | -0.3 | -0.3 |
| Class D | -0.5 | -0.5 | -0.4 | -0.5 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.4 | -0.4 | -0.6 | -0.4 | -0.3 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |

### Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed DST was derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in TMuC 0.9.  Note that we neither use the quantization and inverse quantization matrices associated with MDDT [1], nor have introduced any additional quantization matrices for the proposed DST matrix. We only require the storage of one additional matrix corresponding to DST at a block size.

Bit Precision: In our implementation, we have used the following number of bits for DST precision.

1. 4*4 Forward and Backward DST: 6-bit shift.

2. 8*8 Forward and Backward DST: 7-bit shift.

3. 16*16 and 32*32 Forward and Backward DST: 10-bit shift

Retaining Low Complexity, Hardware friendly DCT: Note that in the TMuC 0.9 software, a reduced complexity approximation to the DCT is used for a hardware-friendly implementation, to minimize the number of multiplications and additions. We have retained this reduced-complexity approximate-DCT in our proposal and simulations. If the actual full complexity DCT is used in simulations, the performance of our proposal will also improve, but this will also require the storage for the full-complexity DCT matrices.  Additionally, these full-complexity DCT matrices will require the storage of another set of quantization matrices in the standard, since for current Intra and Inter-Coding in TMuC 0.9, only the reduced-complexity DCT matrices and corresponding quantization matrices are used.

Scanning: In this proposal we have not used any additional mode-dependent scanning methods based on intra-prediction mode following the transform coefficients and have retained the default scanning method in TMuC 0.9. The use of any mode-dependent scanning method, in conjunction with the proposed DCT/DST scheme will also enhance the overall system performance.

Encoding and decoding times as compared to TMuC 0.9 anchors are shown along-with R-D results in Section 3.1 in Simulation Results. Note that the slightly high encoding/decoding complexity relative to TMuC 0.9 anchors for the proposed mode-dependent DCT/DST when it is applied at all block sizes is due to full un-optimized DST matrix multiplication at size 32*32. This can be reduced by using fast implementations of DST.

When the DCT/DST is applied only for block sizes 4*4 and 8*8, the encoding and decoding complexity is almost the same as TMuC 0.9 anchors.

For the DST, implementation is performed via full matrix multiplication.

# Conclusions

In this contribution, we have presented the results for our earlier proposal in Core Experiment 7 for using mode-dependent DCT or DST transform [5] for intra-prediction residues instead of the conventional DCT. The proposed mode-dependent DCT/DST scheme requires the storage of only 1 additional DST matrix at a block-size, is based on the intra prediction mode, and does not require any explicit signaling. There is no requirement of introducing additional quantization matrices in our proposal. Simulation results show significant gains in compression performance as compared to TMuC 0.9 anchors. We recommend adopting this proposal into HM.

# References

1. Y. Ye and M. Karczewicz, “Improved Intra coding,” ITU-T Q.6/SG-16 VCEG, VCEG-AG11, Shenzhen, China, October 2007.

2. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

3. Test Model under Consideration, ITU-T JCTVC-B205_draft002, Geneva, Switzerland, July 2010

4. Min et al., “Unification of the Directional Intra Prediction Methods in TMuC,” ITU-T JCTVC-B100_revision02, Geneva, Switzerland, July 2010

5.  A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

6. R. Cohen, C. Yeo and R. Joshi, “Description of Core Experiment 7: Alternate Transforms,” ITU-T JCTVC-C507, Guangzhou, China, October 2010.

7. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

8. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Results for mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-C037, Guangzhou, China, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).