| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D034r1 WG11 Number: m18781 |
| --- | --- |

| Title: | Specification of cropped Super Hi-Vision test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Kazuhisa Iguchi Shinichi Sakaida Yoshiaki Shishikui NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp iguchi.k-eq@nhk.or.jp sakaida.s-gq@nhk.or.jp shishikui.y-hw@nhk.or.jp |
| Source: | NHK ( Japan Broadcasting corporation ) |  |  |

_____________________________

# Abstract

The test material ad hoc group has discussed new test materials for JCT-VC activity. It was pointed out that the variety of current test materials were not sufficient. This document contributes the specification of cropped Super Hi-Vision (SHV) sequences as new test materials for beyond HDTV sequences. NHK has already contributed SHV format test sequences, which are in 7680 x 4680/60 fps/10 bpp format, to JCT-VC [1]. Moreover, JCTVC-C055 reported that these cropped sequences, which were in the 2560 x 1600/60 fps/8 bpp format, were available for evaluating new video coding methods and their performance by TMuC 0.7 [2]. In the Guangzhou meeting, a viewing session was held and the sequences are well known. This document describes the cropped SHV sequences information. We propose these cropped sequences as new test materials, which are a part of Class A sequences. We are prepared to contribute these sequences to JCT-VC.

# Introduction

HEVC is required to support a wide variety of picture sizes ranging from at least QVGA to 8K x 4K [3]. The current highest resolution of formal test sequences is limited to 2560 x 1600 and these sequences consist of the number of frames of 5 sec. The current 2560 x 1600 sequences are only in 30-fps format and the contents are captured with a fixed camera with low chroma power. The contributed SHV sequences include scenes captured with a moving camera, colorful scenes and night shots, scenes containing large motion, and 60-fps video sequences. The sequences can provide variation of Class A sequences. Thus, the sequences are also valuable for JCT-VC activities. This contribution describes the specifications of cropping SHV sequences into 2560 x 1600.

# Specifications of cropped test sequences

i) Cropping area of sequences

The sequences are cropped into 2560 x 1600 considering the content of these sequences. The cropped area information is shown in Table 1.

Table 1 Information of cropped regions

| Sequence | Cropped region (Left, Top) – (Right, Bottom) |
| --- | --- |
| Steam locomotive train | ( 3456, 1984)-( 6015, 3583) |
| Nebuta festival | ( 3904, 1408)-( 6463, 3007) |

Figure 1-(a) Steam locomotive train (7680 x 4320)

Figure 1-(b) Steam locomotive train (2560 x 1600)

Figure 2-(a) Nebuta festival (7680 x 4320)

Figure 2-(b) Nebuta festival (2560 x 1600)

ii) Bit-depth conversion

The bit-depth of the original SHV sequences is 10 bpp. The cropped SHV sequences were converted to 8-bpp signals. The signals are according to the specifications of ITU-R BT. 709 [4]. The bit-depth conversion process is as follows.

// 10- to 8-bit conversion process

value = round ( value >> 2 );

if ( value > 254 ) value = 254;

else if ( value < 1 ) value = 1;

iii) Information of frames

The cropped sequences consist of 300 frames (5 sec) from the 0th to 299th frame and the note of copy right and release notes at the 300th frame.

iv) Conditions of use

The copyright statement is attached to the end of the test sequences. The sequences must be treated under the same condition of JCT-VC A023. Upon duplication of the data, all the frames should be copied. However, the final frame is not subject to signal processing.

# Coding results by HM 0.9

Table 2 shows the coding results by HM 0.9 under Class A anchor conditions. We show these data as the anchor data for these sequences.

Table 2-(a). Coding results for intra setting

| Sequence | QP | Intra |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | Y (dB) | U (dB) | V (dB) | kbps | Y (dB) | U (dB) | V (dB) |
| SL | 22 | 103845.57 | 43.73 | 46.41 | 46.05 | 114344.12 | 43.26 | 46.54 | 46.19 |
|  | 27 | 49547.71 | 41.42 | 45.58 | 45.32 | 54554.13 | 41.04 | 45.75 | 45.48 |
|  | 32 | 27368.74 | 39.83 | 44.97 | 44.91 | 29505.18 | 39.46 | 45.16 | 45.03 |
|  | 37 | 15551.91 | 38.20 | 44.48 | 44.43 | 16369.64 | 37.79 | 44.57 | 44.47 |
| Nebuta | 22 | 424648.01 | 42.66 | 41.01 | 40.56 | 510001.83 | 41.56 | 40.89 | 40.13 |
|  | 27 | 265107.89 | 39.11 | 38.47 | 37.16 | 283255.11 | 37.16 | 38.33 | 36.99 |
|  | 32 | 159814.91 | 34.89 | 36.64 | 35.46 | 163901.36 | 33.55 | 36.46 | 35.23 |
|  | 37 | 85171.79 | 30.96 | 34.74 | 33.62 | 83420.53 | 30.04 | 34.60 | 33.51 |

Table 2-(b). Coding results for random access setting

| Sequence | QP | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | Y (dB) | U (dB) | V (dB) | kbps | Y (dB) | U (dB) | V (dB) |
| SL | 22 | 24669.90 | 41.49 | 46.22 | 45.89 | 24208.29 | 41.10 | 46.29 | 45.92 |
|  | 27 | 6389.29 | 40.06 | 45.48 | 45.29 | 6716.89 | 39.80 | 45.63 | 45.38 |
|  | 32 | 2646.17 | 38.80 | 44.84 | 44.80 | 2926.44 | 38.48 | 45.04 | 44.94 |
|  | 37 | 1253.32 | 37.11 | 44.22 | 44.37 | 1481.77 | 36.75 | 44.46 | 44.47 |
| Nebuta | 22 | 223111.26 | 39.57 | 38.94 | 37.61 | 244505.62 | 37.41 | 39.04 | 37.63 |
|  | 27 | 98393.10 | 33.73 | 37.72 | 36.35 | 101156.15 | 32.36 | 37.59 | 36.16 |
|  | 32 | 31835.72 | 29.75 | 36.52 | 35.27 | 26131.37 | 28.96 | 36.27 | 35.00 |
|  | 37 | 7059.01 | 27.97 | 35.21 | 34.03 | 6994.17 | 27.72 | 34.86 | 33.78 |

# Patent rights declaration(s)

NHK (Japan Broadcasting corporation) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] S. Sakaida, Y. Shishikui, A. Ichigaya, Y. Matsuo, K. Iguchi and T. Toyoda, "7680 × 4320 format test sequences for JCT-VC," JCTVC-A023, April 2010.

[2] K. Iguchi, A. Ichigaya and Y. Shishikui, "Performance report of TMuC for Super Hi-Vision," JCTVC-C055, October 2010.

[3] "Vision, Applications and Requirements for High-Performance Video Coding (HVC)," ISO/IEC JTC1/SC29/WG11 document N11096, Jan. 2010.

[4] ITU-R, Recommendation BT.709-5: Parameter Values for the HDTV Standards for Production and International Programme Exchange, 2002.