| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D036 |
| --- | --- |

| Title: | Matrix multiplication specification for HEVC transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mangesh Sadafale Texas Instruments Inc., India Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | mangesh@ti.com madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution is a continuation of prior work presented in JCTVC-C226 and presents more details and software optimization results. JCTVC-C226 proposed the use of matrix multiplication to specify HEVC transforms. Matrix multiplication has the advantage that it is friendly to parallel processing with minimal dependency and control logic. In hardware, matrix multiplication results in low-area architecture, while in software it leads to efficient implementation on SIMD processors. Matrix multiplication has better fixed-point behavior than Chen’s DCT/IDCT which allows for elimination of existing norm correction matrices in HM. The memory requirement for storing norm correction matrices in the TMuC decoder goes down from 7.5 KB to 6 bytes. The corresponding memory requirements in the TMuC encoder goes down from 7.5 KB to 12 bytes. Fixed-point version of matrix multiplication DCT/IDCT along with norm correction matrix elimination optimization was implemented in TMuC-0.9. Simulation results indicate matrix multiplication DCT/IDCT achieves similar coding efficiency performance when compared to TMuC-0.9 with Chen’s DCT/IDCT (Average BD-Rate: 0.1 to -0.3%). TMuC-0.9 decoder and encoder with matrix multiplication DCT/IDCT implemented with even-odd decomposition optimization have run times that are comparable to TMuC-0.9 with Chen’s implementation. Decoding time ratios on the same PC are in the range of 99%-100% range. Encoder time ratios on heterogeneous Linux cluster range from 93%-107%.

# Introduction

In HM 1.0 [1], 4x4, 8x8, 16x16, and 32x32 DCT transforms are supported. Large block size transforms (16x16 and 32x32 transforms) improve the coding efficiency, but also increase the implementation complexity significantly.  Hence implementation complexity of large block transforms need to be studied carefully to ensure feasibility of HEVC implementations.

Hardware codecs are expected to play an important role in deployment of HEVC solutions since HEVC is expected to be used for high definition and above video resolution. Hardware acceleration for video is already standard in desktop, mobile, and portable devices. In software, HEVC codecs can be expected to run on processors that support extensive SIMD operations. Already, 8-way SIMD  architectures are becoming commonplace. Hence transforms that are efficient to implement in both hardware and in software on SIMD machines are needed. This contribution proposes matrix multiplication specification of HEVC transforms to meet the needs of efficient implementation in both hardware and in software/SIMD. Matrix multiplication implementation has the advantage that it is friendly to parallel processing with minimal dependency and control logic. In hardware, matrix multiplication results in high throughput/low-area architecture, while in software it leads to efficient implementation on SIMD processors. The matrix multiplication architecture is configurable and can be re-used across various DCT transform block sizes for HEVC. Matrix multiplication has better fixed-point behavior than Chen’s DCT/IDCT which allows for elimination of existing norm correction matrices in HM. The memory requirement for storing norm correction matrices in the TMuC decoder goes down from 7.5 KB to 6 bytes. The corresponding memory requirements in the TMuC encoder goes down from 7.5 KB to 12 bytes.

# Matrix multiplication implementation

TMuC-0.9 code was modified as follows to study rate-distortion performance of matrix multiplication implementation.

## DCT/IDCT by matrix multiplication

Direct matrix multiplication DCT/IDCT was implemented for all block sizes – 4x4 to 32x32. Transforms of size 4x4 and 8x8 use transform matrices identical to those used in H.264. Transforms of size 16x16 and 32x32 use a fixed point approximation of IDCT matrix as the transform matrix. Elements of 16x16 and 32x32 transform matrix are represented by 7-bit signed numbers. Note that separate transform matrices need not be stored for 16x16 and 32x32 block sizes. The largest transform matrix contents can be re-used by proper indexing leading to significant savings in memory for storing transform matrix entries. In addition to direct matrix multiplication DCT/IDCT implementation, an optimized version of DCT/IDCT using even-odd decomposition [2] was implemented to improve the decoder and encoder run-times.

## Quantization modification

In HM 1.0, the quantization and inverse quantization process incorporate norm correction matrices for transforms of sizes 32x32 and 16x16. Hence quantization and inverse quantization require matrices of the following sizes:

- Quantization of 32x32 block: UInt g_aiQuantCoef1024[6][1024];

- Inverse quantization of 32x32 block: UInt g_aiDeQuantCoef1024[6][1024];

- Quantization of 16x16 block: UInt g_aiQuantCoef256[6][256];

- Inverse quantization of 16x16 block: UInt g_aiDeQuantCoef256[6][256];

The above quantization matrices were found to provide about 0.1-0.2% gain in limited tests versus not using the quantization matrices.

Direct matrix multiplication has better quantization behavior. So we replace the above quantization matrices with scalar values, one for each QP mod value as follows:

- Quantization of 16x16/32x32 block:

  - UInt g_aiQuantCoef256_s[6]  = {205,186, 158, 146, 128, 114};

  - UInt g_aiQuantCoef1024_s[6]  = {102, 93, 79, 73 ,64, 57};

- Inverse quantization of 16x16/32x32 block:

  - UInt g_aiDeQuantCoef[6]  = {10, 11, 13, 14, 16, 18};

The memory requirement in the decoder goes down from 7.5 KB to 6 bytes. The memory requirements in the encoder goes down from 7.5KB to 12 bytes.

# Simulation results

Encoder simulations were run on Linux cluster. Decoding of bitstreams was carried out sequentially on a PC with Intel(R) Core(TM)2 CPU 6600 @ 2.4GHz, 3.25 GB of RAM. Encoder and decoder both used two-stages of even-odd decomposition. Table below presents BD-Rate and encoder/decoder run-times in comparison with TMuC-0.9 anchors.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.1 | 0.5 | 0.5 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class E | -0.1 | 0.0 | -0.2 | 0.0 | 0.3 | 0.3 |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 100% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | -0.1 | 0.5 | 0.3 |
| Class B | -0.1 | -0.1 | 0.0 | -0.1 | 0.2 | 0.0 |
| Class C | 0.0 | -0.2 | -0.1 | 0.0 | 0.3 | 0.2 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | -0.2 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | 95% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.3 | -0.1 | 0.2 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 |
| Class D | 0.1 | -0.3 | -0.1 | -0.1 | 0.1 | -0.1 |
| Class E | -0.2 | -0.1 | -0.6 | -0.3 | -0.6 | 0.1 |
| All | 0.0 | 0.0 | -0.3 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 93% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Complexity analysis

Complexity depends not only on number of additions and multiplications but also on bit-width of the operands. Other factors in complexity are: size of the intermediate transpose buffer, throughput, and hardware area.

## Number of additions and multiplications

|  | 1D 32-point transform |  |  | 1D 16-point transform |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TMuC-0.9 | MatMult in SW | MatMult in HW | TMuC-0.9 | MatMult in SW | MatMult in HW |
| MULTs | 116 | 342 | 300 | 44 | 82 | - |
| ADDs | 210 | 404 | 424 | 82 | 116 | - |

Though the number of additions and multiplications is higher for matrix multiplication based IDCT specification, the hardware complexity ends up being lower since the bit-width of each operand is lower for matrix multiplication based IDCT when compared to Chen’s implementation as shown in next subsection. In software, TMuC-0.9 decoder and encoder with matrix multiplication DCT/IDCT implemented with even-odd decomposition optimization have run times that are comparable to TMuC-0.9 with Chen’s implementation. Decoding time ratios on the same PC are in the range of 99%-100% range. Encoder time ratios on heterogeneous Linux cluster range from 93%-107%.

## Bit-width sizes for operands in inverse transform

Bit width sizes were measured in the software by inputting 2^bpp-1 (bpp = 8 for IBDI off,  bpp = 12 for IBDI on) for all samples at input of forward transform and by quantizing with QP = 1. The maximum absolute values at intermediate stages were recorded and the corresponding bit-widths required were calculated. Table below list the bit-widths required.

In software for IBDI Off case, matrix multiplication requires 16-bit multiplications and 32-bit additions where as Chen’s requires 32-bit multiplications and 64-bit additions.

|  | IBDI Off |  |  |  |
| --- | --- | --- | --- | --- |
|  | Chen's |  | Matmult |  |
|  | Input of Inverse transform | Intermediate values after first transform stage | Input of Inverse transform | Intermediate values after first transform stage |
| 16x16 | 21 bits | 24 bits | 15 bits | 15 bits |
| 32x32 | 20 bits | 24 bits | 15 bits | 15 bits |
|  | IBDI On |  |  |  |
|  | Chen's |  | Matmult |  |
|  | Input of Inverse transform | Intermediate values after first transform stage | Input of Inverse transform | Intermediate values after first transform stage |
| 16x16 | 25 bits | 24 bits | 19 bits | 19 bits |
| 32x32 | 24 bits | 24 bits | 19 bits | 19 bits |

## Transpose buffer size

Transpose buffer is an important component in the transform implementation. Transpose buffer stores the output after first transform stage. It needs to provide high speed access to data in it. As a result of which it is expensive and its size needs to be minimized. In hardware the size of the transpose buffer is directly proportional to the bit-width of intermediate values after first transform stage. So in hardware for IBDI off, matrix multiplication provide approximately 37.5% reduction in transpose buffer size. It provides around 20.8% reduction in transpose buffer size for IBDI on. In software for IBDI off, matrix multiplication (16-bit memory) require half the memory size when compared to Chen’s (32-bit memory). For IBDI on in software, both matrix multiplication and Chen’s algorithms need 32-bit memory.

## Throughput

Chen’s IDCT uses several stages of butterfly type of structure to reduce computational complexity as shown in Figure below.

Figure 1: Signal flow graph of Chen’s fast 16-point DCT transform [3].

The butterfly structure introduces serial dependency and leads to multipliers getting cascaded one after the other. Also, after each butterfly stage involving multiplication, a rounding/truncation operation is carried out to prevent bit-width increase of data that results from cascaded multiplication. These factors lead to increased delay in hardware implementation and limit the maximum frequency at which the IDCT block can be run. This problem is more acute for large block sizes since the number of stages in Chen’s IDCT increases with block size. Table below illustrates the issue with an example implementation in hardware.

|  | TMuC-0.9 Chen's IDCT | Matrix multiplication IDCT |
| --- | --- | --- |
| Number of cascaded multipliers | 4 (for 32x32) 3 for (16x16) | 0 for all |
| Max frequency in Low-power 45nm (with NO Pipeline) | 100MHz | 275MHz |

## Hardware area

Example hardware area number ratios for 1D 32x32 transform implementation in RTL are provided below.

# References

- JCTVC, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, 3rd. JCT-VC Meeting, Guangzhou, CN, Oct. 2010.

- Hung, C.-Yu.; Landman, P;. “Compact inverse discrete cosine transform circuit for MPEG video decoding”, IEEE Workshop on Signal Processing Systems: Design and Implementation, (SIPS’ 97), pp.364-373, Nov. 1997.

- JCTVC, “Test Model under Consideration”, JCTVC-A205, 1st JCTVC Meeting: Dresden, DE, 15-23 April, 2010

- M. Sadafale, M. Budagavi, “Low-complexity configurable transform architecture for HEVC,” JCTVC-C226/m18266, ITU-T/ISO/IEC JCT-VC meeting, Guangzhou, China, Oct. 2010.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).