| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D037 |
| --- | --- |

| Title: | DCT+Hadamard large transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Ajit Gupte Texas Instruments Inc., India | Tel: Email: Tel: Email: | madhukar@ti.com a-gupte1@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution is a continuation of prior work presented in JCTVC-C255 and presents more details and software optimization results. JCTVC-C255 proposed a class of transforms for large block sizes which is a combination of DCT+Hadamard transforms for reducing computational complexity. When DCT+Hadamard is applied to only 32x32 Inter blocks, the average BD-Rate degradation is in the range of 0.2-0.4%. When DCT+Hadamard is applied to both Inter and Intra blocks, the average BD-Rate degradation is in the range of 0.4-0.8%. The 1D DCT+Hadamard transform involves application of two size-N/2 DCT transforms instead of one size-N DCT transform resulting in reduction in number of multiplications required. There is 50% reduction in number of multiplications for DCT+Hadamard implemented with matrix multiplication and 24% reduction in number of multiplications for DCT+Hadamard implemented with Chen’s algorithm.

# Introduction

HM uses several sizes of transform up to 32x32. The complexity of transforms typically increases with O(Nlog(N)) for fast algorithms and O(N^3) for implementation with direct matrix multiplication which translates to around 5X to 8X increase in complexity depending on the architecture used for DCT implementation. Hence it is desirable to reduce complexity of large size transforms.

JCTVC-C255 presented results for the following transforms:

- Hadamard32

- DCT16+Hadamard2 (Forward transform: Hadamard followed by DCT, Inverse transform: IDCT followed by Hadamard)

- Hadamard2+DCT16 (Forward transform: DCT followed by Hadamard, Inverse transform: Hadamard followed by IDCT)

- DCT32

The combination of DCT16+Hadamard2 (DCTH) was found to provide the best coding performance in JCTVC-C255.

# DCT+Hadamard transform

The Inverse DCTH transform matrix is defined using the following matrices (equation below shows an example of 8x8 Inverse DCTH matrix):

H2=111−1

H=H20000H20000H20000H2

P=1000000000001000010000000000010000100000000000100001000000000001

D4=IDCT 4×4 matrix

D=D400D4

The 8x8 Inverse DCTH matrix is given by:

T=H P D

The 32x32 Inverse DCTH matrix can be similarly constructed. The coefficients are transmitted in the order of increasing sequency of forward transform matrix (i.e. low sequency to high sequency).

# Results

DCTH 32x32 forward and inverse transforms were integrated into TMuC-0.9. The underlying DCTs in DCTH were implemented using optimization techniques presented in [1]. Encoder simulations were run on Linux cluster. Decoding of bitstreams was carried out sequentially on a PC with Intel(R) Core(TM)2 CPU 6600 @ 2.4GHz, 3.25 GB of RAM.

## DCTH applied to only Inter blocks

Table below presents BD-Rate and encoder/decoder run-times in comparison with TMuC-0.9 anchors. Results are for all frames.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.1 | 0.5 | 0.5 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class E | -0.1 | 0.0 | -0.2 | 0.0 | 0.3 | 0.3 |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 96% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | -0.1 | 0.0 | 0.5 | 0.4 |
| Class B | 0.4 | -0.1 | -0.1 | 0.4 | 0.0 | -0.1 |
| Class C | 0.1 | 0.0 | -0.2 | 0.0 | 0.2 | 0.1 |
| Class D | 0.0 | 0.2 | 0.3 | 0.0 | -0.1 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 93% |  |  | 95% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | -0.3 | -0.4 | 0.5 | 0.0 | -0.3 |
| Class C | 0.2 | -0.1 | 0.0 | 0.1 | 0.0 | 0.1 |
| Class D | 0.2 | -0.1 | -0.2 | -0.1 | 0.1 | -0.1 |
| Class E | 0.4 | -0.1 | -1.2 | -0.1 | -0.3 | -0.1 |
| All | 0.4 | -0.2 | -0.4 | 0.2 | 0.0 | -0.1 |
| Enc Time[%] | 97% |  |  | 92% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

## DCTH applied to both Inter and Intra blocks

Table below presents BD-Rate and encoder/decoder run-times in comparison with TMuC-0.9 anchors. Results are for 65 frames only.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.6 | 0.3 | 0.1 | 0.6 | 0.8 |
| Class B | 1.7 | 2.3 | 2.0 | 3.0 | 1.4 | 1.4 |
| Class C | 0.3 | 0.6 | 0.4 | 0.0 | 0.1 | 0.0 |
| Class D | 0.2 | 0.4 | 0.3 | 0.0 | 0.1 | 0.1 |
| Class E | 0.9 | 1.3 | 1.3 | 0.4 | 0.9 | 0.9 |
| All | 0.8 | 1.1 | 1.0 | 0.9 | 0.7 | 0.7 |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.3 | 0.2 | -0.1 | 0.5 | 0.4 |
| Class B | 1.2 | 1.0 | 0.7 | 1.5 | 0.0 | -0.3 |
| Class C | 0.3 | 0.2 | 0.2 | 0.2 | 0.0 | 0.0 |
| Class D | 0.2 | 0.8 | 0.5 | 0.2 | 0.4 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.6 | 0.6 | 0.5 | 0.6 | 0.2 | 0.0 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 0.1 | 0.2 | 0.8 | 0.0 | 0.0 |
| Class C | 0.2 | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class D | 0.2 | 0.3 | -0.8 | -0.1 | 0.9 | 0.0 |
| Class E | 0.3 | 1.1 | -0.2 | -0.1 | 0.2 | -0.2 |
| All | 0.4 | 0.3 | -0.2 | 0.2 | 0.2 | 0.0 |

# Complexity

The number of additions and multiplications were calculated for 32x32 DCTH transform. Table below provides the number of operations for matrix multiplication implementation and for Chen’s implementation. There is 50% reduction in number of multiplications for DCTH implemented with matrix multiplication architecture and 24% reduction in number of multiplication for DCTH implemented with Chen’s algorithm.

|  | MatMult |  |  | Chen |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DCT | DCTH | % Reduction | DCT | DCTH | % Reduction |
| MULTs | 342 | 172 | 50% | 116 | 88 | 24% |
| ADDs | 404 | 264 | 35% | 210 | 196 | 7% |

# Conclusions

The class of DCT+Hadamard transforms appears to provide an interesting trade-off in terms of complexity reduction and rate-distorition performance. Other complexity reduction techniques such as zonal coding etc. can be built on top it to further reduce complexity. Hence, we would like to recommend further study DCT+Hadamard transforms.

# References

- M. Sadafale and M. Budagavi, “Matrix multiplication specification for HEVC transforms,” JCTVC-D036/m18783, ITU-T/ISO/IEC JCT-VC meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).