| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D039 |
| --- | --- |

| Title: | ALF decode complexity analysis and reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | madhukar@ti.com sze@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution analyzes implementation complexity of Adaptive loop filter (ALF) at decoder. Implementation complexity analysis involves not just analysis of computations but also analysis of memory bandwidth and memory size (area). There are two ways to implement ALF filtering at decoder – frame-based filtering and LCU-based filtering. For both frame-based and LCU-based filtering, line buffers that store previous lines of deblocked image are needed to reduce memory bandwidth requirements. This contribution presents two ALF filter sets that reduce vertical size of filter thereby reducing the line buffer/memory bandwidth requirements. Both filter sets have a maximum vertical size of 5. Filter set 1 reduces memory bandwidth/memory size requirements by 50% and worst case computations by 20%. Filter set 2 has similar computational complexity as existing HM ALF filters but reduces memory bandwidth/memory size requirements by 50%. Existing HM ALF filters provide average BD-Rate savings in range of 3.3%-4.1%. Filter set 1 and set 2 provide average BD-Rate savings in the range of 3.2%-4%.

# Introduction

HM 1.0 ALF uses three diamond shaped filters of sizes 9, 7, 5 as shown in Figure 1. A maximum of up to 16 filters can be signaled per slice. Each filter requires (N*N/4+1) multiplications. For size 9 filter, this translates to 21 multiplications. The number of pixels that need to be read to carry out one filtering operation is also 21.

9-Diamond

7-Diamond

5-Diamond

Figure 1: HM 1.0 ALF filter set.

ALF decode filtering can be carried out in either frame-based or LCU-based fashion. In frame-based ALF filtering, the deblocking filter output is first stored in a frame buffer. The deblocked filter output is then read to carry out ALF filtering. Figure 2 shows an example frame-based ALF filtering with size-5 filter. The red-lines in Figure 2 are pixel lines. For a size-9 filter, the deblock frame buffer needs to be read 21 times which is not feasible since the memory bandwidth requirements go up to 21 frame buffer reads. Clearly a caching mechanism is needed to reduce memory bandwidth requirements.

Figure 2: Frame-based ALF decode.

1

2

3

4

5

6

7

Filtering at (4, x)

Filtering at (5, x+1)

1

2

3

4

5

6

7

Figure 3: Frame-based ALF decode filtering with line buffer cache.

Figure 3 shows an example of frame-based ALF decode filtering with line buffer cache. The figure on left shows filtering of line 4. The figure on right shows filtering of line 5. From the Figure 3, one can see that Lines 3, 4, 5, 6 get used in filtering of both lines 4 and 5. Data on these lines can be cached in line buffers to avoid reading them again from deblock frame output buffer. With the line buffer caching mechanism shown in Figure 3, the deblock frame output buffer needs to be read only once (also written once). However, this is still expensive. Memory bandwidth requirements can be further reduced by using LCU-based filtering instead as shown in Figure 4.

Input

Output

Input

Lines that need to be cached

Output

Figure 4: LCU-based ALF decode filtering with line buffer cache.

In LCU-based filtering, ALF filtering is carried out on an LCU basis as and when the deblock filter output gets generated. The left part of Figure 4 shows ALF filtering of LCU(0,0) and the right part shows ALF filtering of LCU(1,0). The red lines show the deblock filter pixel lines (which is input to ALF filters) and the blue lines are the ALF filtered output. Since the ALF filter is a non-causal filter and uses right and bottom LCU data (which are not yet available), not all pixels in a LCU can be ALF filtered. The ALF filtered pixels are shown in solid blue line and the LCU pixels that cannot be ALF filtered when the LCU is generated are shown in dotted blue lines.

From Figure 4 it should be noted that even for LCU-based filtering, the previous N-1 lines need to be cached in line buffers to avoid additional memory bandwidth as shown in Figure 4.

Assuming a 4Kx2K image with maximum ALF filter size = 9 and IBDI on, the line buffer memory size for luma is: 4K * 8 lines *12bits = 48Kbytes. This translates to around 140 Kgates in hardware. As a comparison, a state of the art AVC/SVC/MVC video decoder chip [1] uses 414 Kgates and 9 Kbytes of memory. Hence, it is crucial to reduce the line buffer requirements for ALF decode filtering for ALF to be a part of HEVC standard that can be cost-effectively implemented.

# ALF filters with reduced vertical size

For a given image size, the vertical size of ALF filters determines the size of line buffer/memory bandwidth requirements. These requirements can be reduced by reducing the vertical size of ALF filter. This contribution presents two ALF filter sets that reduce vertical size of filter. Figure 5 shows both these filter sets. Both filter sets have a maximum vertical size of 5. Since these filters operate on only five lines instead of the original nine, the size of the line buffers goes down from 8 lines to 4 lines. Filter set 1 reduces memory bandwidth/memory size requirements by 50% and worst case computations by 20%. Filter set 2 has similar computational complexity as existing HM ALF filters but reduces memory bandwidth/memory size requirements by 50%.

Filter set 1

Filter set 2

Figure 5: ALF filters with reduced vertical size.

# Simulation results

Proposed ALF filter set 1 (Nx5-Set 1) and filter set 2 (Nx5-Set 2) were integrated into TMuC-0.9. Table below provides the BD-Rate savings of existing HM 1.0 ALF filter set, proposed Nx5-Set 1 and Nx5-Set 2. It can be seen from tables below the proposed ALF filter sets capture most of the ALF coding gains while reducing line buffer size/memory bandwidth by 50%.

# Conclusions

This contribution analyzes the implementation complexity of Adaptive loop filter (ALF) at decoder. Implementation complexity analysis involves not just analysis of computations but also analysis of memory bandwidth and memory size (area). There are two ways to implement ALF filtering at decoder – frame-based filtering and LCU-based filtering. For both frame-based and LCU-based filtering, line buffers that store previous lines of deblocked image are needed to reduce memory bandwidth requirements. This contribution presents two ALF filter sets that reduce vertical size of filter thereby reducing the line buffer/memory bandwidth requirements. Both filter sets have a maximum vertical size of 5. Filter set 1 reduces memory bandwidth/memory size requirements by 50% and worst case computations by 20%. Filter set 2 has similar computational complexity as existing HM ALF filters but reduces memory bandwidth/memory size requirements by 50%. Existing HM ALF filters provide average BD-Rate savings in range of 3.3%-4.1%. Filter set 1 and set 2 provide average BD-Rate savings in the range of 3.2%-4%. Both the proposed ALF filter sets capture most of the ALF coding gains. Hence we recommend that the proposed ALF filters be adopted in HM pending successful cross-verification.

# References

- T-D. Chuang et. al., “A 59.5mW Scalable/Multi-View Video Decoder Chip for Quad/3D Full HDTV and Video Streaming Applications,” IEEE ISSCC 2010.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).