| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D040_r2 WG11 Number: m18787 |
| --- | --- |

| Title: | PSNR computation on R′G′B′ color system |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dzung Hoang | Tel: Email: | dzung.hoang@zenverge.com |
| Source: | Zenverge, Inc. |  |  |

_____________________________

# Abstract

The R′G′B′ color system is commonly used in the specification of digital video capture and display devices. Digital video transmission and compression systems, on the other hand, commonly use the Y′CBCR component color system. Digital video compression systems code the residuals of the Y′, CB, and CR components separately. As a result, objective distortion in video compression is commonly measured using the Y′, CB, and CR components, with MSE the most commonly used measure. In the evaluation of a digital video compression system, it can be difficult to juggle three different distortion measurements, especially when trying to balance the coding of the Y′, CB, and CR components. In this document, we propose to compute a single distortion measure based upon a combined MSE of R′, G′, and B′ components. To illustrate the proposed R′G′B′ distortion measure, we present an analysis of the simulation results from JCTVC-D035.

- Introduction

The RGB color system is commonly used in the specification of digital video capture and display devices. It is well-known that due to properties of the human visual system (HVS), a wide range of colors can be captured and reproduced using combinations of the three primary colors red, green, and blue (RGB). Because the HVS operates nonlinearly, the gamma-corrected R′G′B′ color triplet is used for coding to make the perceptual error more uniform when the color triplet is quantized to a fixed number of bits. The precise definition of the primary colors and the gamma correction are specified in video standards such as ITU-R Rec. 709 and SMPTE RP 145.

Digital video transmission and compression systems, on the other hand, commonly use the Y′CBCR component color system. Standards such as Rec. ITU-R BT.601, Rec. ITU-R BT.709, and SMPTE 240M specify the conversion process between R′G′B′ and Y′CBCR. Rec. ITU-R BT.601 is applied to standard definition video, while the others are applied to high definition video.

Digital video compression systems code the residuals of the Y′, CB, and CR components separately. As a result, objective distortion in video compression is commonly measured using the Y′, CB, and CR components, with mean square error (MSE) being the most commonly used measure. Computed MSE distortion is reported using the PSNR, which is a logarithmic scaling of MSE. In the evaluation of a digital video compression system, it can be difficult to juggle three different distortion measurements, especially when trying to balance the coding of the Y′, CB, and CR components. Many practitioners and researchers focus on just the Y′ component (luma), unless their work is directly related to the CB and CR components (chroma). For coding techniques that affect both luma and chroma, adhoc criteria are often applied to judge the results.

In this document, we propose to convert reconstructed frames from Y′CBCR to R′G′B′ and then computing a single MSE distortion on the R′G′B′ components. This gives a single distortion value that can be used in conjunction with the existing Y′CBCR MSE distortion values to evaluate a coding technique. Since R′G′B′ is used in the video capture and display devices, a distortion based upon R′G′B′ components seems almost obvious.

In the rest of this document, we briefly review the computation of PSNR, describe the computational details of a tool to compute R′G′B′ PSNR, and present experimental results from contribution JCTVC-D035 to illustrate the proposed R′G′B′ PSNR measure.

# Computation of PSNR

The definition of PSNR for 8-bit unsigned data is as follows:

PSNR=10log2552MSE	()

where MSE is the well-known mean square error. As applied to a frame of video, PSNR is computed from the MSE of the pixels that comprise the frame. When applied to a video sequence, the PSNR should also be computed from the MSE of the pixels that comprise the video sequence. However, the current HM software computes the sequence-level PSNR as the average of the frame-level PSNR, which we refer to as APSNR to differentiate it from the PSNR defined in Eq. 1.

APSNRseq=iPSNRframe(i)N	()

While APSNR is simpler to compute from frame-level PSNR, its use should be discontinued in today’s age of 64-bit computing and high efficiency video coding.

# Computing R′G′B′ PSNR

In this section, we describe the computations performed by a tool to compute R′G′B′ PSNR.

## Converting from Y′CBCR to R′G′B′

In order to convert an Y′CBCR frame to R′G′B′, we need to know how the Y′CBCR frame was generated in the first place. Standards such as Rec. ITU-R BT.601, Rec. ITU-R BT.709, and SMPTE 240M specify the conversion process between a R′G′B′ triplet and a corresponding Y′CBCR triplet.

The PSNR tool can convert from Y′CBCR to R′G′B′ using one of four selectable color space conversion matrices shown in Table 1. These matrices convert Y′CBCR samples in the range [0, 255] to R′G′B′ samples in the nominal range [0.0, 1.0]. Because of the lossy compression of Y′CBCR, the converted R′G′B′ samples can lie outside the nominal range. The PSNR tool can optionally clip the R′G′B′ samples to the range [0, 1].

Table 1: Color space conversion matrices supported by PSNR tool

| Name | Matrix |
| --- | --- |
| CS_REC_709_Y219 | R'G'B'=0.004566210.00.007030360.00456621−0.00083627−0.002089840.004566210.008283930.0⋅Y'CBCR−16128128 |
| CS_REC_601_Y219 | R'G'B'=0.004566210.00.006258930.00456621−0.00153396−0.003188110.004566210.007910710.0⋅Y'CBCR−16128128 |
| CS_REC_709_Y255 | R'G'B'=0.003921570.00.0061515650.00392157−0.00073173625−0.001828610.003921570.007248438750.0⋅Y'CBCR−16128128 |
| CS_REC_601_Y255 | R'G'B'=0.003921570.00.005476563750.00392157−0.001342215−0.002789596250.003921570.006921871250.0⋅Y'CBCR−16128128 |

The matrices CS_REC_601_Y219 and CS_REC_709_Y219 assume that Y′ values fall within the nominal range [16, 235]. The matrices CS_REC_601_Y255 and CS_REC_709_Y255 are scaled to accommodate Y′ values in the range [0, 255].

Before Y′CBCR to R′G′B′ conversion can take place, the 4:2:0 Y′CBCR frames must be upsampled to 4:4:4. The 4:2:0 Y′CBCR frames are first upsampled to 4:2:2 and then from 4:2:2 to 4:4:4. The upsampling from 4:2:0 to 4:2:2 uses the following vertical polyphase filter coefficients: [3, -16, 67, 227, -32, 7]/256 and [7, -32, 227, 67, -16, 3]/256. The upsampling from 4:2:2 to 4:4:4 uses the following horizontal polyphase filter coefficients: [0, 0, 256, 0, 0]/256 and [21, -52, 159, 159, -52, 21]/256. These coefficients are borrowed from the MSSG reference software for MPEG-2 video.

Once the original and reconstructed frames are converted to R′G′B′, the MSE is computed for each R′G′B′ component as well as a combined MSE using all components.

# Test Case

We present a subset of the results from JCTVC-D035 as a test case for R′G′B′ PSNR. Table 2 collects the BD-rate results comparing IBDI against no-IBDI for the low-delay high-efficiency (LD-HE) configuration. Shown are BD-rate values computed for Y′, CB, CR, R′, G′, B′, and R′G′B′. For these results, PSNR was computed using Eq 1. The color space matrix CS_REC_709_Y219 was used and clipping of R′G′B′ was enabled.

Table 2: BD-rate increase comparing IBDI to no-IBDI for LD-HE configuration

|  |  | BD-Rate Increase |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | Cb | Cr | R′ | G′ | B′ | R′G′B′ |
| Class B | Kimono | -2.2 | -4.3 | -4.8 | -3.3 | -2.4 | -3.8 | -3.3 |
| 1080p | ParkScene | -2.6 | -7.0 | -10.1 | -5.2 | -2.9 | -6.2 | -5.0 |
|  | Cactus | -4.0 | -5.3 | -6.7 | -5.5 | -4.1 | -4.9 | -5.0 |
|  | BasketballDrive | -3.1 | -9.1 | -7.2 | -5.6 | -3.6 | -6.9 | -5.7 |
|  | BQTerrace | -5.1 | -25.0 | -31.0 | -11.3 | -5.5 | -14.2 | -11.1 |
|  | Average | -3.4 | -10.2 | -11.9 | -6.2 | -3.7 | -7.2 | -6.0 |
| Class C | BasketballDrill | -2.8 | -6.4 | -5.3 | -4.5 | -3.2 | -5.3 | -4.6 |
| WVGA | BQMall | -3.6 | -9.1 | -9.8 | -6.3 | -4.0 | -6.7 | -6.0 |
|  | PartyScene | -1.8 | -9.7 | -10.7 | -5.0 | -2.3 | -6.2 | -4.8 |
|  | RaceHorses | -1.0 | -2.9 | -2.7 | -1.9 | -1.1 | -2.2 | -1.9 |
|  | Average | -2.3 | -7.0 | -7.2 | -4.4 | -2.6 | -5.1 | -4.3 |
| Class D | BasketballPass | -1.0 | -4.7 | -1.9 | -1.6 | -1.2 | -3.5 | -2.2 |
| WQVGA | BQSquare | -2.2 | -29.3 | -28.9 | -8.6 | -2.8 | -10.0 | -7.5 |
|  | BlowingBubbles | -1.9 | -10.0 | -11.4 | -5.7 | -2.5 | -6.6 | -5.3 |
|  | RaceHorses | -1.1 | -2.8 | -2.5 | -1.8 | -1.2 | -2.1 | -1.9 |
|  | Average | -1.5 | -11.7 | -11.2 | -4.4 | -1.9 | -5.6 | -4.2 |
| Class E | Vidyo1 | -10.1 | -10.7 | -10.3 | -10.2 | -10.0 | -10.1 | -10.1 |
| WQVGA | Vidyo3 | -10.4 | 1.6 | -5.3 | -9.0 | -10.0 | -5.3 | -8.0 |
|  | Vidyo4 | -10.4 | -11.1 | -9.9 | -10.3 | -10.3 | -10.7 | -10.5 |
|  | Average | -10.3 | -6.7 | -8.5 | -9.8 | -10.1 | -8.7 | -9.5 |
| All | Average | -3.9 | -9.1 | -9.9 | -6.0 | -4.2 | -6.5 | -5.8 |

Some sequences, such as BQTerrace and BQSquare, show a big difference among the BD-rate increases for Y′, CB, and CR. In all cases, the R′G′B′ BD-rate represents a middle ground of the results for Y′, CB, and CR. The ParkScene and Cactus sequences exhibit very different BD-rate for Y′, CB, and CR, but have the same R′G′B′ BD-rate. It is easy to make biased conclusions regarding ParkScene and Cactus, given the disparity between their Y′ BD-rate.

# Recommendations

Zenverge recommends adoption of R′G′B′ PSNR as an additional distortion measure for evaluating coding tools. We also recommend modifying the HM to compute PSNR instead of APSNR. We volunteer to add the code to compute R′G′B′ PSNR to HM if requested by the committee.

The PSNR tool computes the combined R′G′B′ PSNR by giving equal weighting to the R′, G′, and B′ components. It is also possible and may be desirable to weight the components to match the color sensitivity of the HVS. We seek input from experts regarding the appropriate perceptually-based weights to use.

It would be useful to collect meta-data regarding the appropriate color conversion matrix to use for each test sequence.

# References

[1]	Stephen Wolf and Margaret Pinson, “Reference Algorithm for Computing Peak Signal to Noise Ratio (PSNR) of a Video Sequence with a Constant Delay,” Available URL: http://www.its.bldrdoc.gov/pub/n3/video/itut_com9_c6.pdf

[2]  Charles Poynton, Digital Video and HDTV Algorithms and Interfaces, Morgan Kaufmann Publishers, 2003.

# Patent rights declaration(s)

The contributor has no patent rights on the content of this contribution.