| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D041 WG11 Number: m18788 |
| --- | --- |

| Title: | Finer scaling of quantization parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dzung Hoang | Tel: Email: | dzung.hoang@zenverge.com |
| Source: | Zenverge, Inc. |  |  |

_____________________________

# Abstract

The current Test Model under Consideration (TMuC) employs a quantization parameter (QP) scaling that is borrowed from the AVC standard. In AVC, the quantization step size increases by approximately 12.25% with each increment of QP, so that the quantization step size doubles when QP is incremented by 6. For the purpose of rate control, this 12.25% increment is too coarse for certain applications, such as low-delay coding. This contribution proposes a specification that allows the granularity of the quantization parameter (QP) to be varied at the slice and picture levels. Backward-compatibility with the AVC approach is maintained at one of the granularity settings. In addition to varying the granularity, this contribution also proposes a variable QP offset that can be specified at the slice and picture levels.

- Problem Statement

The current Test Model under Consideration (TMuC) employs a quantization parameter (QP) scaling that is borrowed from the AVC standard. In AVC, the quantization step size increases by approximately 12.25% with each increment of QP, so that the quantization step size doubles when QP is incremented by 6. For the purpose of rate control, this 12.25% increment may be too coarse for certain applications, such as low-delay coding where bit rate variations need to be tightly controlled due to a small encoder buffer. For perceptual quantization, a smaller increment is desirable so that the quantization of each MB can be better modulated to equalize the perceived distortion of each MB, based upon the local spatial characteristics.

In AVC, the QP can vary from 0 to 51, inclusive. A full transition of QP from 0 to 51 translates to a 362-fold increase in the quantization step size. However, in practice only a small range of QP values are used when coding a scene at a given bit rate. At scene changes, QP can change due to a change in scene complexity, but then QP typically varies only slightly within a scene.

There have been prior efforts to address the coarse quantization step size increment of AVC. In JCTVC-A114, the quantization step size was proposed to double every 16 increments of QP.

# Proposal Description

In AVC, the quantization step size applied to each transform coefficient is determined approximately as follows:

Δi,j≈2QP6×mi,j	()

where mi,j is a frequency-dependent scaling factor associated with the integer block transform. In AVC, the quantization step size is computed using the normAdjust(m,i,j) and normAdjust8x8(m,i,j) functions. These functions combine the QP scaling with the transform scaling.

We propose to add two parameters to the quantization step size computation as follows:

Δi,j≈2QP+αβ×mi,j	()

where  is an additive offset and  determines the granularity of QP increment, or equivalently, the doubling interval. For higher values of ,  is useful to compensate for the lower dynamic range of quantization step size. For example, an encoder can change  upon detecting a scene change to reflect a change in coding complexity. In Table 1, we compute the quantization step size increment and maximum dynamic range for values of  between 6 and 18.

Table 1: Characteristics of quantization step size increment given 

|  | increment | dynamic range per max QP |  |  |
| --- | --- | --- | --- | --- |
|  |  | 51 | 63 | 71 |
| 6 | 12.25% | 362.04 | 1448.15 | 3649.12 |
| 7 | 10.41% | 156.03 | 512.00 | 1130.59 |
| 8 | 9.05% | 83.00 | 234.75 | 469.51 |
| 9 | 8.01% | 50.80 | 128.00 | 237.02 |
| 10 | 7.18% | 34.30 | 78.79 | 137.19 |
| 11 | 6.50% | 24.87 | 52.98 | 87.70 |
| 12 | 5.95% | 19.03 | 38.05 | 60.41 |
| 13 | 5.48% | 15.17 | 28.76 | 44.06 |
| 14 | 5.08% | 12.49 | 22.63 | 33.62 |
| 15 | 4.73% | 10.56 | 18.38 | 26.60 |
| 16 | 4.43% | 9.11 | 15.32 | 21.67 |
| 17 | 4.16% | 8.00 | 13.05 | 18.08 |
| 18 | 3.93% | 7.13 | 11.31 | 15.40 |

Observing that the quantization step size increment decreases more slowly as  increases, we conclude that we do not need to adopt many values for . We propose restricting the selection of  to the following set: {8, 16}. This choice allows for a single scaling matrix design for =16. The scaling matrix for =8 is simply a sampling of the matrix for =16.

One concern with increasing  is the reduction in the dynamic range of quantization step size given the same range of QP values. To offset this, we propose to increase the QP range. Based upon prior experience with AVC, we note that most encodings use a QP interval of size between 20 to 30. This translates to a dynamic range of 10 and 40, respectively. The current maximum QP of AVC is sufficient for =8. For =16, we propose either maximum QP of 63 or 71.

The  parameter can be used to adjust the mapping between QP and quantization step size to allow higher or lower bit rate operation. We propose  be selectable in the range [0, max(QP)].

# Proposed Syntax

The QP scaling should be optionally signaled at the sequence, picture, and slice levels. At the each syntax level, we propose to add a one-bit flag to signal whether QP scaling information is enabled for that level. If enabled, the QP granularity and offsets are signaled.

Table 2: Proposed Additions to seq_parameter_set_rbsp()

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| qp_scaling_flag | 0 | u(1) |
| if( qp_scaling_flag ) { |  |  |
| qp_granularity | 0 | u(1) |
| qp_luma_offset | 0 | ue(v) |
| qp_cr_offset | 0 | ue(v) |
| qp_cb_offset | 0 | ue(v) |
| } |  |  |
| … |  |  |

Table 3: Proposed Additions to pic_parameter_set_rbsp()

| pic_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| qp_scaling_flag | 0 | u(1) |
| if( qp_scaling_flag ) { |  |  |
| qp_granularity | 0 | u(1) |
| qp_luma_offset | 0 | ue(v) |
| qp_cr_offset | 0 | ue(v) |
| qp_cb_offset | 0 | ue(v) |
| } |  |  |
| … |  |  |

Table 4: Proposed Additions to slice_header()

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| qp_scaling_flag | 0 | u(1) |
| if( qp_scaling_flag ) { |  |  |
| qp_granularity | 0 | u(1) |
| qp_luma_offset | 0 | ue(v) |
| qp_cr_offset | 0 | ue(v) |
| qp_cb_offset | 0 | ue(v) |
| } |  |  |
| … |  |  |

The semantics of the qp_granularity field is as follows.

Table 5: Mapping of qp_granularity to 

| qp_granularity |  |
| --- | --- |
| 0 | 8 |
| 1 | 16 |

# Proposed Decoder Operation

Adoption of this proposal would require change to the inverse quantization process. Below is an example of the required changes. The exact changes would depend upon the block transform specification.

eij = (dij * QPScale(qp_beta, QP % qp_beta) * LevelScaleNxN [ i ][ j ]) / 64, with i, j = 0..N-1

where  QPScale(b, x) = round(2^(x/b)*64). The scaling factor of 64 can be changed to adjust the arithmetic precision. Because the two selectable  values are related by a factor of 2, a single look-up table can be designed.

# Experimental Results

We gather some statistics on coding rate by sweeping the QP from 22 to 42 in increments of 1 for one sequence. We chose to code the RaceHorses_832x480_30 sequence under the random-access-high-efficiency test condition using TMuC version 0.7. The results are summarized in the table below. It can be seen that the bit rate decreases on average by approximately 17% for each increment of QP. In our opinion, this is a rather coarse change which is not ideal for rate control.

Table 6: Coding Statistics with QP Sweep

| QP | Rate | Delta Rate | PSNR-Y | PSNR-U | PSNR-V |
| --- | --- | --- | --- | --- | --- |
| 22 | 5006.89 | 0.00% | 39.15 | 41.20 | 42.69 |
| 23 | 4177.68 | -19.85% | 38.44 | 40.70 | 42.23 |
| 24 | 3516.22 | -18.81% | 37.78 | 40.25 | 41.80 |
| 25 | 2961.36 | -18.74% | 37.14 | 39.78 | 41.36 |
| 26 | 2506.22 | -18.16% | 36.53 | 39.32 | 40.94 |
| 27 | 2123.41 | -18.03% | 35.92 | 38.87 | 40.52 |
| 28 | 1809.84 | -17.33% | 35.33 | 38.42 | 40.11 |
| 29 | 1554.19 | -16.45% | 34.76 | 38.06 | 39.78 |
| 30 | 1339.94 | -15.99% | 34.20 | 37.68 | 39.41 |
| 31 | 1152.45 | -16.27% | 33.63 | 37.27 | 39.02 |
| 32 | 992.93 | -16.07% | 33.08 | 36.82 | 38.58 |
| 33 | 859.44 | -15.53% | 32.53 | 36.47 | 38.21 |
| 34 | 744.96 | -15.37% | 32.00 | 36.11 | 37.86 |
| 35 | 645.09 | -15.48% | 31.47 | 35.75 | 37.47 |
| 36 | 558.66 | -15.47% | 30.97 | 35.38 | 37.05 |
| 37 | 480.47 | -16.27% | 30.44 | 35.05 | 36.69 |
| 38 | 413.90 | -16.08% | 29.94 | 34.72 | 36.29 |
| 39 | 353.24 | -17.17% | 29.42 | 34.37 | 35.91 |
| 40 | 299.29 | -18.02% | 28.92 | 34.01 | 35.47 |
| 41 | 253.86 | -17.90% | 28.43 | 33.64 | 35.08 |
| 42 | 214.42 | -18.39% | 27.96 | 33.24 | 34.64 |

# Patent rights declaration(s)

Zenverge, Inc., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).