| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D042_r3 WG11 Number: m18789 |
| --- | --- |

| Title: | CE7: Alternative Transforms – Summary Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com |
| Source: | Mitsubishi Electric Research Laboratories, Institute for Infocomm Research, Qualcomm |  |  |

_____________________________

# Abstract

This document provides a summary and table of documents related to Core Experiment 7 (CE7) on Alternative Transforms.

# Introduction

The purpose of Core Experiment 7 (CE7) [1] is to investigate the performance of transforms other than the conventional integer DCT-like transform currently in HM (TMuC 0.9) [2]. The nine proposals submitted for CE7 along with their corresponding cross-verification documents and any related contributions are summarized in Table 1.

Table 1. CE7-related documents

| Tool # | Proponent (Tool Type) | CE7 Doc. | Cross-verification |  |  | Other related docs. | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Toshiba (1DDUT) | D107 | D264 (Qualcomm) | D032 (Samsung) |  |  |  |
| 2 | I2R (DCT/Derived KLT-DST) | D046 | D353 (Samsung) | D306 (Huawei) | D104 (Toshiba) | D048 D049 |  |
| 3 | Samsung (Fast ROT) | D357 | D030 (MERL) | D409 (Qualcomm) |  | D180 |  |
| 4 | Samsung & Qualcomm (Combined ROT/MDDT) |  |  |  |  |  | Proponent requested this be withdrawn because this is a combined tool. |
| 5 | Samsung (DCT/DST) | D033 | D105 (Toshiba) | D031 (BBC) | D088 (NHK) |  |  |
| 6 | Qualcomm (MDDT Improvements) | D399 | D307 (Huawei) | D354 (Samsung) | D078 (I2R) | D392 |  |
| 7 | Beijing Univ. Tech. (Symmetric transform matrices) |  |  |  |  |  | Proponent withdrew from CE7. |
| 8 | Huawei (Symmetry-based MDDT simplification) | D304 | D047 (I2R) |  |  |  |  |
| 9 | Peking Univ. (Residual reordering) | D284 | D290 (Mediatek) |  |  | D286 |  |

# Summary of proposed alternative transforms

The types of proposed transforms and methods generally fall into one or more of the following categories:

- MDDT Simplification

  - Separable DCT/KLT

  - Separable DCT/DST

  - Mode-mapping (Intra prediction direction mapped to transform mode)

  - Coefficient ordering/scanning

  - Use of symmetry to reduce the number of transforms and number of scanning orders.

- ROT Improvements

- Residual reordering combined with KLT

Descriptions for each of the proposals are as follows:

| Tool # | Proponent | Doc. | Tool Description |
| --- | --- | --- | --- |
| 1 | Toshiba | JCTVC-D107 | 1DDUT is a spatial transform scheme for intra coding. 1DDUT has only two 1-D transform matrices, which are Discrete Cosine Transform (DCT) and the directional transform based on Karhunen-Loeve Transform (KLT). It’s applied for 4x4 block and 8x8 block only. 1DDUT was proposed in the contribution of JCTVC-B042 at the JCT-VC Geneva meeting and JCTVC-C080 at the JCT-VC Guangzhou meeting. |
| 2 | I2R | JCTVC-D046 | To simplify the operation of Mode Dependent Directional Transform (MDDT), a Mode-Dependent Fast Separable KLT for Block-based Intra Coding was previously introduced that requires only two transform matrices: a DCT and a DST (Odd Type-3 DST). The 4x4 DST also has a structure that can be exploited to reduce the operation count of the transform operation. |
| 3 | Samsung | JCTVC-D357 | Rotational Transform (ROT) was proposed to be an adaptive transform for efficient Intra residual encoding. In TE12 the performance of this tool was reported, and high encoding complexity was indicated as the main disadvantage of this tool. This experiment is targeted to investigate a fast version of ROT implemented on top of HM. |
| 4 |  |  |  |
| 5 | Samsung | JCTVC-D033 | In this contribution, we use the conventional Discrete Cosine Transform (DCT), or DST (separably along the vertical or horizontal directions) based on the intra prediction direction. The proposed DCT/DST transform scheme does not require any additional signaling information or Rate-Distortion search during encoding, and works in a single-pass. No training was required to derive the transform and the scheme requires the storage of only one DST matrix in addition to the conventional DCT at each block size. The conventional quantization tables for TMuC 0.9 are retained and no changes have been made to the scanning order. |
| 6 | Qualcomm | JCTVC-D399 | In this contribution, we compare several different methods of mode dependent intra residual coding. They are mode dependent fix scan, adaptive switch scan and adaptive switch KLT+scan. We will show the trade off between coding efficiency and complexity. |
| 7 |  |  |  |
| 8 | Huawei | JCTVC-D304 | A SMDDT (Simplified MDDT) scheme is proposed in this document. It comprises two parts, ST (Simplified Transform) and SSO (Symmetry-based Scanning Order). In the SMDDT-ST, the combination of DCT and DST is used. In the SMDDT-SSO, 2 additional scanning orders as well as the existing zigzag scanning order are used. |
| 9 | Peking Univ. | JCTVC-D284 | The proposed tools apply mode-dependent reordering, transform and coefficient scanning for directional intra prediction and coding in HM. For mode-dependent transform, a single transform matrix is employed for all the directional intra prediction modes defined in HM, and the original DCT method is maintained for DC prediction residue. Moreover, for 4x4 transform, the applied transform is a simplified 4x4 DST featured by multiplierless arithmetic and low dynamic range of internal results. |

# Summary of Experimental Results

Below are tables summarizing the reported experimental results for each tool listed in the CE description. Entries highlighted in yellow indicate that only one contribution (either the proponent or cross-verifier) reported these values, so they have not yet been independently verified. These tables also include any provided LD-HE and LD-LC experimental results, even though those configurations were not required by CE7.

|  | Overall BD-Rate change for the luma (Y) components (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Config | 1 (D107) | 2 (D046) | 3 (D357) | 4 | 5 (D033) | 6 (D399) | 7 | 8 (D304) | 9 (D284) |
| I-HE | -2.1 | -1.3 | -1.9 ¦ -1.1 |  | -1.3 ¦ -1.2 ¦ -1.2 | -2.3 ¦ -2.6 ¦ -3.1 |  | -2.0 | -2.0 |
| I-LC | -2.3 | -2.2 | -2.9 ¦ -1.5 |  | -2.1 ¦ -1.9 ¦ -1.9 |  |  | -2.9 | -2.5 |
| RA-HE | -0.9 | -0.6 | -0.9 ¦ -0.9 |  | -0.6 ¦ -0.6 ¦ -0.6 | -1.0 ¦ -1.1 ¦ -1.3 |  | -0.8 | -0.9 |
| RA-LC | -0.8 | -0.7 | -1.2 ¦ -1.2 |  | -0.7 ¦ -0.6 ¦ -0.6 |  |  | -1.0 | -0.9 |
| LD-HE | -0.3 |  | -0.2 ¦ -0.2 |  |  | -0.3 ¦ -0.4 ¦ -0.5 |  |  |  |
| LD-LC | -0.2 |  | -0.2 ¦ -0.2 |  |  |  |  |  |  |

|  | Encoding time as compared to reference (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Config | 1 (D107) | 2 (D046) | 3 (D357) | 4 | 5 (D033) | 6 (D399) | 7 | 8 (D304) | 9 (D284) |
| I-HE | 106 | 101 | 159 ¦ 119 |  | 113 ¦ 103 ¦ 104 | 112 ¦ 135 ¦ 296 |  | 106 | 104 |
| I-LC | 105 | 103 | 219 ¦ 118 |  | 121 ¦ 97 ¦ 97 |  |  | 106 | 106 |
| RA-HE | 100 | 100 | 104 ¦ 104 |  | 105 ¦ 105 ¦ 100 | 106 ¦ 118 ¦ 133 |  | 100 | 107 |
| RA-LC | 101 | 100 | 103 ¦ 103 |  | 99 ¦ 98 ¦ 98 |  |  | 100 | 99 |
| LD-HE | 100 |  | 103 ¦ 103 |  |  | -- ¦ 118 ¦ 137 |  |  |  |
| LD-LC | 100 |  | 100 ¦ 100 |  |  |  |  |  |  |

|  | Decoding time as compared to reference (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Config | 1 (D107) | 2 (D046) | 3 (D357) | 4 | 5 (D033) | 6 (D399) | 7 | 8 (D304) | 9 (D290) |
| I-HE | 104 | 101 | 98 ¦ 101 |  | 117 ¦ 102 ¦ 102 | 109 ¦ 111 ¦ 112 |  | 101 | 104 |
| I-LC | 107 | 103 | 105 ¦ 103 |  | 127 ¦ 107 ¦ 99 |  |  | 103 | 103 |
| RA-HE | 105 | 100 | 102 ¦ 102 |  | 107 ¦ 104 ¦ 103 | 103 ¦ 101 ¦ 101 |  | 100 | 108 |
| RA-LC | 105 | 100 | 101 ¦ 101 |  | 109 ¦ 105 ¦ 101 |  |  | 101 | 115 |
| LD-HE | 101 |  | 103 ¦ 103 |  |  | -- ¦ 100 ¦ 100 |  |  |  |
| LD-LC | 102 |  | 99 ¦ 99 |  |  |  |  |  |  |

Some proponents provided more than one version of their tool. For those cases, results for each of their implementations are included in the above tables. These cases are:

- Tool 3 (D357): Fast ROT | Fast ROT only for best Intra prediction direction.

- Tool 5 (D033): DCT/DST via matrix mult. all block sizes | same but only for 4x4 & 8x8 | same but only for 4x4 & 8x8 and the DCT part of DCT/DST used the TMuC implementation.

- Tool 6 (D399): Mode dependent fixed scan | Adaptive switch scan | Adaptive switch KLT + scan.

# Summary of other related contributions

Some contributions have been submitted that are not part of the original JCTVC-C507 Core Experiment description, but they are closely related. Below are tables summarizing the reported experimental results for each related tool. Entries highlighted in yellow indicate have not yet been independently verified. These tables also include any provided LD-HE and LD-LC experimental results, even though those configurations were not required by CE7.

|  | Overall BD-Rate change for the luma (Y) components (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tool Config | D048 Orthogonal Integer DST | D049 Orthogonal Integer DST + MDSS | D392 Joint proposal: 1DDUT + new scan | D222 Study of ROT in TMuC 0.9 |  |
| I-HE | -1.3 | -2.0 | -2.1 | -2.6 |  |
| I-LC | -2.0 |  | -2.6 | -3.5 |  |
| RA-HE | -0.6 | -0.9 | -0.9 | -1.2 |  |
| RA-LC | -0.6 |  | -0.8 | -1.3 |  |
| LD-HE |  |  | -0.2 |  |  |
| LD-LC |  |  | -0.2 |  |  |

|  | Encoding time as compared to reference (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tool Config | D048 Orthogonal DST | D049 Orthogonal DST + MDSS | D392 Joint proposal: 1DDUT + new scan | D222 Study of ROT in TMuC 0.9 |  |
| I-HE | 102 | 108 | 106 | 404 |  |
| I-LC | 105 |  | 114 | 557 |  |
| RA-HE | 101 | 102 | 101 | 154 |  |
| RA-LC | 101 |  | 101 | 133 |  |
| LD-HE |  |  | 101 |  |  |
| LD-LC |  |  | 102 |  |  |

|  | Decoding time as compared to reference (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tool Config | D048 Orthogonal DST | D049 Orthogonal DST + MDSS | D392 Joint proposal: 1DDUT + new scan | D222 Study of ROT in TMuC 0.9 |  |
| I-HE | 101 | 102 | 105 | 163 |  |
| I-LC | 104 |  | 116 | 164 |  |
| RA-HE | 100 | 101 | 102 | 113 |  |
| RA-LC | 100 |  | 104 | 165 |  |
| LD-HE |  |  | 101 |  |  |
| LD-LC |  |  | 103 |  |  |

# References

- R. Cohen, C. Yeo, R. Joshi, et al., “CE7: Alternative Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C507, Guangzhou, CN, October 2010.

- TMuC 0.9 software, available at svn://hevc.kw.bbc.co.uk/svn/jctvc-tmuc