| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D043 WG11 Number: m18790 |
| --- | --- |

| Title: | CE8: Conditional joint deblocking-debanding filter |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents performance of conditional joint deblocking-debanding filter JCTVC-C091 [1]. Using conditional joint deblocking-debanding filter, experimental results show average BD-rate changes of:

- 0.07%(Y), -0.01%(U), -0.01%(V) for all intra high efficiency setting, 0.00%(Y), -0.01%(U), 0.00%(V) for random access high efficiency setting, 0.07%(Y), -0.14%(U), 0.10%(V) for low delay high efficiency setting;

- 0.50%(Y), 0.00%(U), 0.00%(V) for all intra low complexity setting, 0.09%(Y), 0.01%(U), 0.03% (V) for random access low complexity setting, 0.08%(Y), 0.08%(U), -0.02%(V) for low delay low complexity setting;

- 0.51%(Y), 0.00%(U), 0.00%(V) for all intra high efficiency setting w/o ALF, 0.12%(Y), 0.10%(U), 0.06%(V) for random access high efficiency setting w/o ALF, 0.06%(Y), 0.01%(U), -0.09%(V) for low delay high efficiency setting w/o ALF.

Subjective quality analysis shows that the conditional joint deblocking-debanding filter suppresses banding noises significantly in Kimono sequence, especially for high efficiency settings with and w/o ALF, as expected from the informal viewing results of the last JCT-VC meeting.

Based on the simulation results and subjective quality analysis, it is proposed that:

- The conditional joint deblocking-debanding filter is considered as a visual quality enhancement tool for IBDI in HM codec;

- Its benefits are further studied in the context of reference picture memory compression technologies based on fixed rounding of IBDI’s extended bit depth values for reducing reference picture storage.

- Introduction

Banding-noise is one of signal-dependent noises which appear in high spatial resolution video coded by lossy compressions. The banding-noise is very noticeable in areas of low detail with subtle changes in color. One approach for dealing with the banding-noise is to encode input video with higher sample bit-depth in order to avoid the lost of the low detail. Another approach is to mask the banding-noise with signal-independent noise, as described in JCTVC-C091 [1].

This contribution briefly reviews the conditional joint deblocking-debanding filter in TMuC0.9 and presents its simulation results conducted according to the coding conditions described in JCTVC-C500 [2] and JCTVC-C508 [3].

# Conditional joint deblocking-debanding filter

The conditional joint deblocking-debanding filter is implemented on the top of the deblocking filter described in JCTVC-B205 [4]. Detailed description is presented in Appendix A as a proposed description of deblocking filter process of HM. As it can be seen from the description of Appendix A, the amount of the TMuC modification is small and the syntax change is unnecessary. The software is also found in attachment, “JCTVC-D045-NEC-code.zip”.

The following shows the steps of the conditional joint deblocking-debanding filter process.

Given a line which crosses either vertical or horizontal edge of an intra-coded block boundary, the conditional joint deblocking-debanding filter process is conducted as follows:

- Determine the index offset indexOffset on the basis of the pixel-position (posX, posY) of q00 and edge direction dir by

indexOffset=((posY+(posX<<4))∧4095)⋯if(dir is vertical)

- Conditionally apply the Luma strong filtering as the deblocking filter does,

- Conditionally inject pseudo-noise loaded from a LUT to the Luma strong filtered images as follows:

pji = Clip0-255(pji + LUT [(indexOffset + (i << 4) – j - 1) & 4095]);	j= 0,…,7

qji = Clip0-255(qji + LUT [(indexOffset + (i << 4) + j   ) & 4095]);	j= 0,…,Size

Figure 1 shows an example of a line crossing vertical edge of an intra-coded block boundary.

Figure 1	Example of a current line that crosses a vertical edge of an intra-coded block boundary.

It should be noted here that the number of pseudo-random values in the LUT is 4096, which is about the same as the width of 4Kx2K video, and each of pseudo-random values takes -1, +1 or, 0. Thus the size of the LUT is about 1KB. The frequencies of +1 and -1 values are the same and each is 12.5%. Thus the maximum MSE increase caused by the conditional joint deblocking-debanding filter is only 0.5 (=0.25 * 1 * 2).

# Simulation results

3.1. Coding conditions

The conditional joint deblocking-debanding filter is implemented on the top of the deblocking filter of TMuC0.9. We conducted its performance evaluation according to the coding conditions described in [2]. The attached Excel sheets (JCTVC-D043-NEC-CE8-results.xls and JCTVC-D043-NEC-CE8-addtional-results.xls) provide the results obtained by using (a) the TMuC0.9 software as Anchor and (b) the TMuC0.9 with the conditional joint deblocking-debanding filter as Proposal.

3.2. Computing platforms

Two 64-bit executables for encoding and decoding were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platforms listed below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

As inferred from the list, CPU automated over-clocking happened for some encoding results. Therefore individual encoding time can be significantly varied.

3.3. Results

Table 1 and Table 2 show the summary of BD-Rates, and relative encode and decode times (%) against Anchor results. They show the following observations:

- Proposal negligibly affects BD-rate of all coding conditions in high efficiency coding with ALF since the small pseudo noise is conditionally added and ALF attenuates the conditionally added small pseudo-noise.

- Proposal slightly affects BD-rate of all intra coding results with low complexity setting or high efficiency coding setting w/o ALF. However, it should be noted that its actual coding loss is only decrease in PSNR value and that the maximum PSNR decrease is 0.20 dB for Vidyo3 whose PSNR is higher than 40dB, that is, Proposal does not degrade visual quality of decoded videos because of its negligible MSE increase.

- Proposal negligibly affects BD-rate of other coding conditions with low complexity setting or high efficiency coding setting w/o ALF because the small noise is injected into only intra-coded images.

- Proposal does not affect encode and decode times significantly (in fact, it is within the measurement error for decoding time) as deduced from its simple algorithm.

Table 1		Summary results of common test conditions.

Table 2 	Summary results of CE8 Subtest1 additional common test conditions. (High efficiency settings without ALF.)

3.4. Subjective quality analysis

As typical subjective quality improvements, the cropped images of Kimono 144th I-frames of Random Access Anchor and Proposal encoded with QP=27 are shown in an attachment “NEC-Example-Kimono144th.pdf”. Banding-noise tends to appear on sky regions of encoded Kimono sequence because the original sky regions contain low detail with subtle changes in color. By comparing the images, the following observations are obtained:

- High efficiency (HE) and Low complexity (LC) Anchor results are heavily degraded by banding-noise;

- LC Proposal result shows moderate banding noise;

- HE Proposal results (HE and HE w/o ALF) show no banding noise.

The second and third observations indicate that the conditional joint deblocking-debanding filter is particularly useful for IBDI encoding.

# Remarks

Although current HM design does not use fixed rounding of IBDI’s extended bit depth values in prediction loop, an IBDI rounding-error conscious memory compression based on in-loop fixed rounding can be one of the good operation points of HEVC codec in terms of the balance between BD-rate and reference picture memory bandwidth, as described in our another contribution JCTVC-D045 [5]. In such a operation point of HEVC codec, it is expected that the conditional joint deblocking-debanding filter will be an essential tool in order to avoid banding-noise caused by in-loop fixed rounding that cannot be easily suppressed by out-loop processing. Therefore it is recommended that its benefits are further studied in the context of reference picture memory compression technologies based on fixed rounding of IBDI’s extended bit depth values.

# Conclusions

This contribution has presented performance of conditional joint deblocking-debanding filter. Based on the simulation results and subjective quality analysis, it is recommended that:

- The conditional joint deblocking-debanding filter is considered as a visual quality enhancement tool for IBDI in HM codec;

- Its benefits are further studied in the context of reference picture memory compression technologies based on fixed rounding of IBDI’s extended bit depth values for reducing reference picture storage.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- K. Chono, K. Senzaki, H. Aoki, J. Tajime, and Y. Senda, "TE10: Conditional joint deblocking-debanding filter," JCTVC-C091 Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- T. Yamakage et al., “Description of CE8: In-loop filtering,” Document: JCTVC-C508, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, 2nd JCT-VC Meeting, Geneva, July 2010. Document: JCTVC- B205, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010.

- K. Chono, K. Senzaki, H. Aoki, J. Tajime, and Y. Senda, “Rounding-error conscious memory compression method for IBDI,” JCT-VC document JCTVC-D045, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Guangzhou, Daegu, Korea, 20-28 January 2011.

Appendix A: Proposed description of deblocking filter process

(The modified portion of JCT-VC B205 is highlighted with red colour and underline.)

The deblocking filter process shall be applied to all prediction unit partition edges and transform unit edges of a picture, except edges at the boundary of the picture, and edges of prediction unit partitions and transform unit partitions smaller than 8 pixels in either vertical or horizontal direction. For the transform units and prediction units with edges smaller than 8 pixels, only the edges lying on the 8x8 grid are filtered.

The deblocking filter process is invoked for the luma and chroma components separately. The filter process is in LCU order. Each LCU is processed in CU order. For each CU, vertical edges are filtered first, starting with the edge on the left-hand side of the prediction unit proceeding through the edges towards the right-hand side of the prediction unit in their geometrical order. Then, horizontal edges are filtered starting with the edge on the top of the CU proceeding through the edges towards the bottom of the CU in their geometrical order.

The variables filterInternalEdgesFlag, filterLeftPuEdgeFlag, filterTopPuEdgeFlag are derived as specified by the following ordered steps.

- The derivation process for neighbouring largest coding units specified in subclause Error: Reference source not found is invoked and the output is assigned to lcuAddrA and lcuAddrB

- The variables filterInternalEdgesFlag, filterLeftPuEdgeFlag and filterTopPuEdgeFlag are derived as specified by the following ordered steps:

- The variable filterInternalEdgesFlag is derived as follows.

- the variable filterInternalEdgesFlag is set equal to 1

- The variable filterLeftPuEdgeFlag is derived as follows.

- If any of the following conditions is true, the variable filterLeftPuEdgeFlag is set equal to 0:

- CurrLcuAddr % PicWidthInLCUs is equal to 0

- the largest coding unit lcuAddA is not available

- Otherwise, the variable filterLeftPuEdgeFlag is set equal to 1

- The variable filterTopPuEdgeFlag is derived as follows.

- If any of the following conditions is true, the variable filterTopPuEdgeFlag is set equal to 0:

- CurrLcuAddr is less than PicWidthInLCUs

- the largest coding unit lcuAddB is not available

- Otherwise, the variable filterTopPuEdgeFlag is set equal to 1

Fig. 5-1 shows the possible edges from the partition boundaries inside the prediction unit which can be interpreted as luma or chroma edges.

Fig. 5-1 Partition boundaries in a prediction unit to be filtered

When interpreting the edges in Fig. 5-1 as luma edges and chroma edges, the partition mode PartMode is considered as well as the value of the variables filterInternalEdgesFlag, filterLeftPUEdgeFlag and filterTopPUEdgeFlag.

The vertical edge interpretation depending on the partition mode is specified as follows:

- The first vertical edge can be filtered when filterLeftPuEdgeFlag is equal to 1 and the width of the prediction unit is equal or larger than 8 pixels.

- If PartMode is PART_nLx2N, the second vertical edge can be filtered when filterInternalEdgesFlag is equal to 1 and the distance between the first and the second vertical edges is equal or larger than 8 pixels.

- Otherwise, if PartMode is PART_Nx2N or PART_NxN, the third vertical edge can be filtered when filterInternalEdgesFlag is equal to 1 and the distance between the first and the third vertical edges is equal or larger than 8 pixels.

- Otherwise, if PartMode is PART_nRx2N, the fourth vertical edge can be filtered when filterInternalEdgesFlag is equal to 1 and the distance between the fourth vertical edges and the next PU or the picture boundary is equal or larger than 8 pixels.

The horizontal edge interpretation depending on the partition mode is specified as follows:

- The first horizontal edge can be filtered when filterTopPuEdgeFlag is equal to 1 and the height of the prediction unit is equal or larger than 8 pixels.

- If PartMode is PART_2NxnU, the second horizontal edge can be filtered when filterInternalEdgesFlag is equal to 1 and the distance between the first and the second horizontal edges is equal or larger than 8 pixels.

- Otherwise, if PartMode is PART_2NxN or PART_NxN, the third horizontal edge can be filtered when filterInternalEdgesFlag is equal to 1 and the distance between the first and the third horizontal edges is equal or larger than 8 pixels.

- Otherwise, if PartMode is PART_2NxnD, the fourth horizontal edge can filtered when filterInternalEdgesFlag is equal to 1 and the distance between the fourth horizontal edges and the next PU or the picture boundary is equal or larger than 8 pixels.

The vertical and horizontal edges interpretation depending on transform unit size is specified as follows:

Fig. 5-3 shows the possible edges from the partition boundaries inside the transform unit which can be interpreted as luma or chroma edges.

Vertical edges

Horizontal edges

MinTransformUnitSize

MinTransformUnitSize

Fig. 5-3 Partition boundaries in a transform unit to be filtered

Only vertical edges in the transform unit with a distance equal to 8 or larger can be filtered. This means that if MinTransformUnitSize=4 only every second vertical edge can be filtered. Only horizontal edges in the transform unit with a distance equal to 8 or larger can be filtered. This means that if MinTransformUnitSize=4 only every second horizontal edge can be filtered.

When a horizontal or vertical edge from either partitions of a prediction unit or from partitions of a transform unit can be filtered according to above processes, another process follows to check if parts of respective edge can be filtered. The deblocking filter operates on k parts, where k ranges 0 to K-1, of edges for luma as well as for chroma, where K is the width or height of the current CU divided by 8 for horisontal edges and vertical edges respectively. The size of each part is 8 pixel. The pixels on respective side of parts of the edges are in the following denoted A and B where B corresponds to a part of the partition at largest coding unit address lcuAddr, prediction unit index puIdx and transform unit index tuIdx. In the following, A corresponds to the left neighbouring partition to B for vertical edges and to the above neighbouring partition to B for horizontal edges.

p30 p20 p10 p00

p31 p21 p11 p01

p32 p22 p12 p02

A p33 p23 p13 p03

P34 p24 p14 p04

P35 p25 p15 p05

P36 p26 p16 p06

P37 p27 p17 p07

q00 q10 q20 q30

q01 q11 q21 q31

q02 q12 q22 q32

q03 q13 q23 q33  B

q04 q14 q24 q34

q05 q15 q25 q35

q06 q16 q26 q36

q07 q17 q27 q37

Notation of an 8 pixel part of a vertical edge for deblocking

Based on the lcuAddrA, puIdxA, and tuIdxA, the width and height of the transform unit which current partition A belongs to, TUWidthA and TUHeightA, are determined. Similarly, based on the lcuAddrB, puIdxB, and tuIdxB, the width and height of the transform unit which current partition B belongs to, TUWidthB and TUHeightB, are determined.

The k:th part of a vertical edge in luma the pixels in partition A and B have following correspondence with reconstructed pixel values in luma:

qji = S'L[ xS + j, yS + i + k*8]

with j=0..max(3, TUWidthB-9) and i=0..7

pji = S'L[ xS - j - 1, yS + i + k*8]

with j=0..7 and i=0..7

where S'L, xS and yS are defined in 5.3.5.

The k:th part of a horizontal edge in luma the pixels in partition A and B have following correspondence with reconstructed pixel values in luma:

qji = S'L[ xS + j + k*8, yS + i ]

with j=0..7 and i=0.. max(3, TUHeightB-9)

pji = S'L[ xS + j + k*8, yS - i - 1 ]

with j=0..7 and i=0..7

The k:th part of a vertical edge in chroma the pixels in partition A and B have following correspondance with reconstructed pixel values in chroma:

qji = S'C[ xSC + j, ySC + i + k*8]

pji = S'C[ xSC - j - 1, ySC + i+ k*8 ]

with j=0..3 and i=0..7

where S'C, xSC and ySC are defined in 5.3.5.

The k:th part of a horizontal edge in chroma the pixels in partition A and B have following correspondance with reconstructed pixel values in chroma:

qji = S'C[ xSC + j+ k*8, ySC + i ]

pji = S'C[ xSC + j+ k*8, ySC - i - 1]

with j=0..7 and i=0..3

### Luma filtering

A part of the luma edge between A and B can be filtered if one of the following conditions is true

- A or B has PredMode==MODE_INTRA

- or A or B has nonzero transform coefficients

- or A and B use different reference frames or different number of motion vectors

-	or one motion vector is used to predict partition A and one motion vector is used to predict the partition B and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma frame samples,

-	or two motion vectors and two different reference pictures are used to predict the partition A and two motion vectors for the same two reference pictures are used to predict the partition B and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two partitions for the same reference picture is greater than or equal to 4 in units of quarter luma frame samples,

-	or two motion vectors for the same reference picture are used to predict the partition A and two motion vectors for the same reference picture are used to predict the partition B and the absolute difference between the horizontal or vertical component of the corresponding motion vectors used in the prediction of the two partitions is greater than or equal to 4 in units of quarter luma frame samples.

- and  |p22 - 2*p12 + p02| + |q22 - 2*q12 + q02| + |p25 - 2*p15 + p05| + |q25 - 2*q15 + q05| < β

The filtering is also performed on the 8 pixel edge if the conditions described above are fulfilled for any pair of the smaller partitions or transform units on the different sides of the edge. For example, if A consists of two 4x4 partitions (A1 and A2) then the above process except the last item is done for both partition pairs A1 - B and A2 – B, and the last item is applied for the pair A-B. If pairs A1 - B or pair A2 - B need to be filtered then the whole 8 pixel edge between A and B is filtered.

A part of the luma edge between A and B is not filtered if one of the following conditions is true.

- A and B belong to the same transform unit partition and the same prediction unit partition

- or the edge is a prediction unit border and both A and B have planar_flag ==1.

If the edge between A and B is filtered, one of two types of filtering (weak or strong filtering) is performed. The choice between the strong and the weak filtering is done separately for each line depending on the following conditions. For each line i = 0,7, the strong filtering is performed if all the following conditions are true, otherwise, weak filtering is performed.

- d < (β>>2)

- and (|p3i - p0i| + |q0i – q3i|) < (β>>3)

- and |p0i – q0i| < ((5*tC + 1)>>1),

where tC and β depends on qp their relations are shown in the table below. The tC is calculated as tC (qp+4) when A or B has PredMode==MODE_INTRA.

If the strong filtering is chosen for a current line, the debanding filtering is subsequently applied to the A image pji of the current line processed by the strong filtering if all the following conditions are true, otherwise, the debanding filtering is not applied to the part A image pji of the line.

- The line crosses a vertical edge

- or the line crosses a horizontal edge

- and A has PredMode==MODE_INTRA

- and TUWidthA is larger than or equal to 16

- and |p0i – q0i| is less than or equal to (1 << increased_bit_depth_luma)

- and |p0i – p3i| is less than or equal to (1 << increased_bit_depth_luma),

where p0i, q0i, and p3i are values prior to the strong filter process.

Similarly, if the strong filtering is chosen for a current line, the debanding filtering is subsequently applied to the B image qji of the current line processed by the strong filtering if all the following conditions are true, otherwise, the debanding filtering is not applied to the part B image qji of the line.

- The line crosses a vertical edge

- or the line crosses a horizontal edge

- and B has PredMode==MODE_INTRA

- and TUWidthB is larger than or equal to 16

- and |p0i – q0i| is less than or equal to (1 << increased_bit_depth_luma)

- and |q0i – q3i| is less than or equal to (1 << increased_bit_depth_luma),

where p0i, q0i, and q3i are values prior to the strong filter process.

Relation between qp, tc and β

| qp | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tc | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 |
| qp | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| tc | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 |
| β | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| qp | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| tc | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 |
| β | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 |  |  |
| qp | 54 | 55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| tc | 14 | 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| β |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Weak filtering

Weak filtering mode filtering is performed based on the above conditions,

 = Clip(-tC,tC, (13*(q0i - p0i) + 4*( q1i - p1i) - 5*( q2i - p2i)+16)>>5))	i = 0,7

p0i = Clip0-255(p0i + )				i = 0,7

q0i = Clip0-255(q0i - ) 				i = 0,7

p1i = Clip0-255(p1i + /2) 				i = 0,7

q1i = Clip0-255(q1i - /2) 				i = 0,7

Strong filtering

Strong filtering mode is performed with the following set of operations.

p0i=Clip0-255((p2i + 2*p1i + 2*p0i +2*q0i + q1i + 4)>>3);		i = 0,7

q0i=Clip0-255((p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4)>>3);		i = 0,7

p1i=Clip0-255((p2i + p1i + p0i + q0i +2)>>2);			i = 0,7

q1i=Clip0-255((p0i + q0i + q1i + q2i +2)>>2);			i = 0,7

p2i=Clip0-255((2*p3i + 3*p2i + p1i + p0i + q0i + 4)>>3);		i = 0,7

q2i=Clip0-255((p0i + q0i + q1i + 3*q2i + 2*q3i + 4)>>3);		i = 0,7

Debanding filtering

Debanding filtering is performed with the following set of operations.

pji=Clip0-255(pji + (npji << increased_bit_depth_luma));		i = 0,7, j= 0,7

qji=Clip0-255(qji + (nqji << increased_bit_depth_luma));		i = 0,7, j= 0,Size

where Size, npji, and nqji are derived as follows:

If the current line crosses a vertical edge, the Size, npji, and nqji are derived as follows:

Size=min(TUWidthB−9,TUWidthB-(TUWidthB>>2)-1) ⋯ if(B has planar_flag == 1)

indexOffset = ((PartBPosY + (PartBPosX << 4)) & 4095) ;

npji = PnLUT [(indexOffset + (i << 4) – j - 1) & 4095];

nqji = PnLUT [(indexOffset + (i << 4) + j  ) & 4095],

otherwise, the Size, npji, and nqji are derived as follows:

Size=min(TUHeightB−9,TUHeightB-(TUHeightB>>2)-1) ⋯ if(B has planar_flag == 1)

indexOffset = ((PartBPosX + (PartBPosY << 4)) & 4095);

npji = PnLUT [(indexOffset + (i << 4) – j - 1) & 4095];

nqji = PnLUT [(indexOffset + (i << 4) + j  ) & 4095],

where PnLUT is the pseudo noise LUT specified in Annex B, and PartBPosX and PartBPosY are the horizontal and vertical sample positions of the top-left corner of B which are determined by lcuAddrB, puIdxB, tuIdxB, and k.

### Chroma filtering

The chroma edge between A and B can be filtered if the following condition is true.

- A or B has PredMode==MODE_INTRA

The filtering is not applied if both following conditions are true.

- The edge is a prediction unit edge

and both A and B have planar_flag==1.

If filtering is performed based on the above condition,

 = Clip(-tC,tC,((((q0i - p0i) << 2) + p1i - q1i + 4)  >> 3))	i = 0,7

p0i = Clip0-255(p0i + )						i = 0,7

q0i = Clip0-255(q0i - )						i = 0,7

tC depend on qp and their relations are shown in the table above. For chroma filtering tC is calculated as tC (qp+4).

### Planar mode filtering

There is a special filtering mode which is applied to the prediction unit edge when both prediction units are encoded in intra planar mode. Luma and chroma edges between A and B are filtered with planar filtering if all of the following conditions are true, otherwise normal filtering described in Luma and Chroma filtering are performed.

- Edge between A and B is a prediction unit edge

- Both A and B have planar_flag==1

- The prediction units for both A and B have the same size.

Let variable blkSize equals to width (or height) of prediction unit, blkSizeHalf= blkSize>>1 and blkSizeQuarter = blkSize>>2 and bitShift = log2(blkSize) - 1. If luma edges are filtered, let S = S'L, xPos = xS, yPos = yS, otherwise  let S = S'C, xPos = xSC, yPos = ySC

The horizontal planar mode filtering is performed with the following set of operations.

for(k=0;k<blkSize;k++) {

a1 = S [xPos - blkSizeQuarter][yPos+k];

a2 = S [xPos + blkSizeQuarter][yPos+k];

for(l=1;l<blkSizeHalf;l++)

S[xPos-blkSizeQuarter+l] [yPos+k] = (a1*(blkSizeHalf-l) + a2*l + blkSizeQuarter) >> bitShift;

}

The vertical planar mode filtering for luma component is performed with the following set of operations.

for(k=0; k<blkSize; k++) {

a1 = S[xPos +k] [yPos-blkSizeQuarter];

a2 = S[xPos +k] [yPos+blkSizeQuarter];

for(l=1; l<blkSizeHalf; l++)

S[xPos +k][ yPos -blkSizeQuarter+l] = (a1*(blkSizeHalf-l) + a2*l + blkSizeQuarter) >> bitShift;

}

Appendix B: Specification of pseudo noise LUT for debanding filtering process

PnLUT [4096] = {

1,-1, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,

0, 0, 1,-1, 0, 0, 0, 1, 0, 0, 0, 1, 0,-1, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 1, 0, 0,-1, 1, 0, 0,

0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 1,-1, 0, 1, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0,-1, 0, 1, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 1,-1, 1, 0, 0,-1, 0, 1,-1, 0, 0, 1,-1,

0, 0, 1,-1, 0, 0, 1,-1, 0, 1,-1, 0, 1,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,

-1, 1,-1, 0, 0, 1,-1, 0, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 1, 0,-1, 0, 0, 0, 1,-1, 1, 0,-1, 1, 0,

0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0,-1, 1, 0,-1, 0, 1,-1, 0, 1, 0, 0,

0, 1,-1, 0, 1,-1, 1,-1, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0,-1, 0, 1,-1, 0, 0, 0, 0, 0, 1,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0,-1, 1,

0, 0, 0, 0, 1,-1, 0, 0, 0,-1, 1, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 1,

0, 0, 0, 0, 0, 1, 0, 0, 1,-1, 1,-1, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,

0, 0, 0, 0, 0, 0, 0, 0,-1, 0,-1, 0, 0, 0, 1, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 1, 0, 0, 0, 0, 1,-1, 0, 0, 0,-1, 0, 0, 0, 0, 0,-1, 0, 1,-1, 0, 0,

0, 0, 1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 1, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0,-1, 0, 0, 0, 0,

0, 0, 0, 1, 0,-1, 1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0,-1, 0, 0,-1, 1, 0,-1, 1,-1, 1, 0, 0, 0, 0, 0,

0, 0, 0, 0, 1, 0, 0, 0, 1,-1, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 1,-1, 0, 0, 1, 0, 0,

0,-1, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0,-1, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0,

-1, 0, 1, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0,

0, 1,-1, 0, 0, 0,-1, 0, 0, 1, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0,-1, 0,

0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0,-1, 1, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 1,-1,

0, 0, 0,-1, 1,-1, 0, 0, 1, 0,-1, 0, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 1, 0, 0, 0, 0, 1,-1, 1, 0,

0, 0,-1, 1, 0, 0,-1, 0, 1, 0,-1, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0,-1, 0, 0, 0, 1, 0,

0, 0,-1, 0, 0, 0,-1, 0, 0, 0, 1,-1, 0, 1, 0, 0, 0, 0, 0, 0, 1, 0,-1, 1, 0,-1, 0, 0, 0, 0, 0,-1,

0, 1, 0, 0,-1, 1, 0, 0,-1, 1,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 1, 0, 0,-1, 1, 0, 0, 0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0,

0, 0,-1, 0, 0, 0, 1, 0, 0, 0,-1, 0, 1,-1, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0,

-1, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0,-1, 0, 0, 1,-1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0,

0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0,

-1, 1, 0,-1, 1,-1, 1,-1, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0,-1, 0, 1,

-1, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0,-1, 0, 0, 1, 0, 0, 0, 1,-1, 0, 0, 0, 0, 1, 0,-1,

1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1,-1, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0,

-1, 0, 0, 0,-1, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 1, 0,

0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 1, 0, 0, 0, 0,-1, 1, 0, 0, 0,-1, 0, 0, 0,

0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 1,-1, 1,-1, 1,

-1, 1,-1, 0, 0, 1,-1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1,-1, 0, 0, 0,-1, 0, 1, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

1,-1, 0, 1,-1, 0, 0, 0, 0,-1, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0,

1, 0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0,-1, 1, 0, 0,-1, 1, 0,-1,

0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1,-1, 0, 0, 0, 0, 0,-1, 1,-1, 1, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0,-1, 0, 1, 0,

0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0,-1, 0,

0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0,-1, 1, 0, 1,-1, 0, 0, 0, 1,-1, 1,-1, 0, 0, 1,-1, 1, 0, 1,-1,

0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

-1, 1, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,

1,-1, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,-1, 0, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0,

0,-1, 0, 1, 0, 0, 0,-1, 1, 0, 0, 0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 0,

0, 0,-1, 0, 0, 0, 1, 0, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 1, 0, 0, 1, 0,

0, 0, 0, 0, 1, 0, 1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0,-1, 0, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 1, 0,-1, 0, 0, 1,-1, 1, 0,-1, 0, 0, 1,-1, 0,

0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 1, 0,-1, 1, 0, 0,-1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 1, 0, 0,-1, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0,

0, 1,-1, 0, 1, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0, 0, 0,-1, 1, 0, 0, 0, 0,-1,

0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0,

0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,-1, 0,-1, 1,-1, 0, 0, 0, 0,-1,

0, 0, 0, 1,-1, 0, 0, 0,-1, 0, 0, 1, 0,-1, 1, 0, 0,-1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 1, 0, 0, 0, 0,

0, 1,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0,-1, 1,-1, 0, 0, 0,-1, 1, 0,-1, 0, 0, 0, 1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0,-1, 1, 0,-1, 1, 0,-1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 1, 0, 0,-1,

1, 0,-1, 0, 0, 0, 0, 0, 1,-1, 0, 1,-1, 0, 0, 1,-1, 0, 0, 0,-1, 1,-1, 1, 0, 0, 1,-1, 1,-1, 0, 0,

0, 0, 0, 0,-1, 0, 1, 0, 0,-1, 1, 0, 1,-1, 0, 1, 0,-1, 0, 0, 0, 1, 0,-1, 1, 0, 1,-1, 0, 0, 0,-1,

0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0,-1, 0, 1, 0, 0, 0, 0, 0,-1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0, 0, 0, 0, 0, 1, 0, 0, 1,

0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 1,-1, 0, 0,-1, 0,

0, 0,-1, 1, 0,-1, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 1,-1, 1, 0,-1, 0, 1,-1, 0, 1,

0,-1, 0, 0, 0, 0, 1,-1, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0,

0,-1, 1, 0, 0, 0, 0, 0, 1, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 1,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1,-1, 0, 0, 0, 0, 0, 1,-1,

0, 0,-1, 0, 1,-1, 0, 0, 1, 0, 0, 0, 0, 0,-1, 0, 0,-1, 1,-1, 0, 0, 0,-1, 1,-1, 0, 0, 1,-1, 0,-1,

1, 0,-1, 1, 0, 0, 0, 0, 0, 1,-1, 1,-1, 1,-1, 1, 0, 0,-1, 1, 0,-1, 1, 0, 0, 0,-1, 1, 0, 0, 0, 0,

0,-1, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 1, 0, 0,-1, 0, 0,-1, 0, 0, 0,-1, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1,

1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 1, 0, 0,

0, 0,-1, 0, 0, 0, 1,-1, 0, 0, 0,-1, 0, 0, 0, 1,-1, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0,-1, 0,

0, 0,-1, 1, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0,

0, 0, 0, 0, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,-1, 1, 0, 0,-1, 0, 0, 0, 0, 0, 1,-1,

0, 0, 0, 1, 0,-1, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0,-1, 0, 0, 0,

0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0,-1, 0, 0,-1, 0, 0, 0,-1, 1, 0,-1, 0, 0, 1,-1, 1, 0, 0,-1, 1,-1, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 1, 0,-1, 1,-1, 0, 1,-1, 1, 0, 0, 0,-1, 0, 0, 0, 0, 0,

1, 0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0,

1,-1, 1, 0, 0, 0, 0, 1, 0, 0,-1, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0,-1, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0,

1, 0,-1, 0, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0,

-1, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 1,-1, 0, 0, 1,-1, 1,-1, 1, 0, 0, 1,-1,

0,-1, 1,-1, 0, 1, 0, 0, 0,-1, 1, 0, 1, 0, 0,-1, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0,

0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 1, 0, 0, 0, 0, 0, 0,

0, 0, 0,-1, 0, 0, 1,-1, 0, 1,-1, 1, 0, 0, 0, 0, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

1,-1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 1, 0, 1,-1, 0,-1, 0, 0, 0, 1,-1,

0, 0, 1, 0, 0, 0,-1, 0, 0, 0, 1, 0,-1, 1, 0, 0, 0, 0, 0,-1, 0, 0, 1, 0, 0, 0, 0, 0, 0,-1, 0, 0,

1, 0,-1, 1,-1, 0, 1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0, 0, 0, 1,-1, 1, 0, 0, 0, 0,

1,-1, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 1, 0,-1, 0, 0, 1,-1, 0, 0, 0,

0, 0, 0, 1,-1, 0, 0, 0,-1, 1, 0,-1, 0, 0,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0,

0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 1, 0, 0,-1, 0, 0, 0, 0,

0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 1, 0,-1, 0, 0, 1,-1, 0, 0, 0, 1,-1, 1, 0, 0, 0, 0, 1,

-1, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 1, 0,-1, 1, 0, 0,-1, 1, 0, 1, 0,

0, 0,-1, 0, 0,-1, 0, 0, 1, 0, 0,-1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 1, 0, 0, 1, 0, 0,-1, 0, 0, 0, 0,-1, 0, 0, 0,

0, 0, 0, 0, 0, 1,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0, 0, 0, 0, 1,-1,

0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 1,-1, 1,-1, 0, 0, 0, 0,

0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 1, 0,-1, 1,-1, 0, 1, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 1, 0,

0, 0, 1,-1, 0, 1,-1, 1,-1, 1, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0,-1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 1, 0,-1, 0, 0, 1,-1, 1,-1, 0, 0, 0, 0,-1, 0, 0, 0, 0,

0, 1,-1, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0,-1,

0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0,-1, 0, 0, 0, 0, 0,-1, 1, 0, 0, 1, 0, 0,-1, 0, 1, 0, 0,

0, 0, 0,-1, 1, 0,-1, 0, 1, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 1, 0, 0,

0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0,-1, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0,

1, 0, 0, 0, 0, 1, 0,-1, 1,-1, 0, 1, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0,-1, 0, 1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0,-1, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0,-1, 1,-1,

0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 1,-1, 1, 0, 0,-1, 1,-1, 1, 0,-1, 0, 0, 0, 0, 0, 0, 0,-1, 0,

1, 0, 0, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0,-1, 0, 0, 0, 0, 0, 1, 0, 0, 0,-1, 1,

0,-1, 0, 0, 0, 0,-1, 0, 0, 0,-1, 0, 0, 1,-1, 1,-1, 0, 0, 0, 0, 0,-1, 0, 1, 0, 0, 0, 0, 0, 0, 0,

0, 0,-1, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0,-1, 0, 1, 0,-1, 1, 0, 0, 0, 0, 0, 0,-1, 0, 1, 0, 0, 0,

0,-1, 0, 0, 0, 0, 0, 0,-1, 0, 0,-1, 0, 0, 1,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,-1, 1,-1, 0, 0, 0,-1, 1, 0,-1, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0,

0, 0,-1, 1, 0, 0, 0, 1, 0,-1, 0, 0, 0, 0, 0, 0, 1,-1, 0, 1, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0,

-1, 1,-1, 0, 1,-1, 0, 0, 0, 0,-1, 0,-1, 1, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1,

-1, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0,-1, 1, 0, 0, 0, 0, 0, 0, 0, 0,-1, 0, 0, 0, 0, 0, 0, 0, 1,-1, 0,

};